3GPP TSG RAN WG1 Meeting #89	     R1-1707256

Hangzhou, P.R. China 15th – 19th May 2017

Agenda Item:	7.1.3.2.3

Source: 	Fujitsu

Title:                     PUCCH resource allocation

Document for:	Discussion/Decision

# Introduction

Until the last RAN1 meeting, the following agreements have been achieved for PUCCH resource allocation.

Agreements:

- A combination of semi-static configuration and (at least for some types of UCI information) dynamic signaling is used to determine the PUCCH resource both for the ‘long and short PUCCH formats’

  - The PUCCH resource includes time, frequency and, when applicable, code domains.

    - FFS details e.g., if the time in the PUCCH resource includes both slot and symbol, or only symbol in a slot

Agreements:

- NR supports PUCCH resource allocation for HARQ-ACK transmission with following manner.

  - A set of PUCCH resources is configured by high layer signaling

    - FFS: other mechanisms

  - A PUCCH resource within the configured set is indicated by DCI.

- PUCCH resource determination rule is defined at least for the case where the dedicated PUCCH resources is unknown to the UE

  - FFS: details of PUCCH resource determination rule including implicit resource mapping and/or explicit signaling

- This does not preclude implicit resource mapping

It has been agreed to support PUCCH resource allocation by using DCI which is one method of explicit signaling. In this contribution, we share some discussions on implicit PUCCH resource mapping.

# Discussion

In LTE, there are multiple PUCCH formats defined such as PUCCH format 1 and PUCCH format 2 defined in release 8 and release 9, PUCCH format 3 defined in release 10, and PUCCH format 4 and PUCCH format 5 defined in release 13. Different PUCCH formats have different payload sizes carrying combinations of one or more of HARQ-ACK, SR and CSI feedback, possibly for multiple component carriers. The methods of allocating PUCCH resources are also different for different PUCCH formats including implicit indication, semi-static configuration, and a combination of semi-static and dynamic signaling. The latter two being methods of explicit signaling. In NR, it has been agreed to support a combination of semi-static and dynamic signaling at least for some types of UCI. Generally this explicit method is beneficial from the flexibility perspective. However, when the UCI payload is small and/or there are a relatively large number of users, the explicit method may result in much more signaling overhead. From the efficiency perspective, an implicit method is more suitable for some types of UCI, and thus it can be supported at least as a complement to the explicit signaling. This also coincides with LTE principles. In LTE, for PUCCH format 1a/1b which carries 1 or 2 bits of ACK/NACK, implicit PUCCH mapping is used; while for PUCCH formats which have more payload bits, PUCCH resources are allocated by explicit signaling.

Proposal 1: NR supports implicit PUCCH resource allocation as a complement to explicit signaling.

In NR, the general framework accommodating explicit and implicit PUCCH resource allocation can be similar with that of LTE. There can be different PUCCH formats depending on e.g. UCI payloads. PUCCH resources of some PUCCH formats can be explicitly configured, and PUCCH resources of other PUCCH formats can be implicitly determined. The resource blocks of different PUCCH formats can be separated by configuration. For some PUCCH formats with small UCI payloads, it would be more efficient to let multiple UEs share a given set of resource blocks. PUCCH resources within this set can be distinguished in an implicit way. In LTE FDD, implicit PUCCH resource allocation is achieved by determining the PUCCH resource according to the lowest CCE index of the PDCCH. In NR, more flexible HARQ timing is to be supported, and thus PDCCHs for different UEs in multiple time instants (e.g. slots) could correspond to the same PUCCH HARQ-ACK feedback instant [1] [2]. A similar situation could also occur if an individual UE receives PDCCHs in multiple time instants. Assuming CCEs are independently numbered within each slot, there will be PUCCH resource collisions if using only the lowest CCE index as shown in Figure 1. Actually such collisions have been encountered in LTE TDD, where PDSCH in multiple DL subframes may need to be acknowledged in the same uplink subframe. In that case, PUCCH resource collisions are avoided by using a subframe-dependent offset on top of the lowest CCE index. Therefore, it is reasonable that similar methods are reused in NR, with necessary modifications.

Figure 1. Illustration of PUCCH resource collision avoidance

It has been agreed that timing between DL data reception and corresponding acknowledgement can be dynamically indicated. As shown in Figure 1, PUCCH feedback in slot n may correspond to multiple PDCCHs received in different slots. We assume that PUCCH HARQ-ACK feedback can be transmitted, at least by some UEs, corresponding to PDCCHs in any slot from n to n-m. Assuming  is the lowest CCE index of PDCCH received in slot n-m, the corresponding PUCCH resource can be determined using , where  denotes the total number of CCEs in slot n-k. Introducing the offset  is approximately equivalent to having CCEs globally numbered and thus avoiding the aforementioned PUCCH collisions. Similar with LTE PDCCH/EPDCCH, further adjustments can be made by indicating  by DCI and signalingby higher layers. For example,  can be used to avoid collisions caused by using the same  in independent control resource sets of different UEs;  can be used to avoid resource overlap with other PUCCH formats. Also the term  can be replaced by a more general form such as a function of the lowest CCE index and DM-RS antenna ports. It should also be possible to assign different PUCCH resource regions to different UEs, which would allow for example, separation of PUCCHs with different formats and/or payload sizes requiring different amounts of resource per PUCCH. Such considerations lead to the following proposal:

Proposal 2: For implicit PUCCH resource allocation, similar methods as in LTE TDD (using the lowest CCE index , offsets , DCI field  and higher layer configured) can be considered with necessary modifications.

# Conclusions

In this contribution, we share our views on PUCCH resource allocation. It has been agreed to support the combination of semi-static and dynamic signaling at least for some types of UCI. Based on the analyses, we give the following proposals.

Proposal 1: NR supports implicit PUCCH resource allocation as a complement to explicit signaling.

Proposal 2: For implicit PUCCH resource allocation, similar methods as in LTE TDD (using the lowest CCE index , offsets , DCI field  and higher layer configured) can be considered with necessary modifications.

# References

- R1-1703296, “On PUCCH resource allocation”, Ericsson, RAN1#88.

- R1-1705392, “Resource allocation for PUCCH”, Samsung, RAN1#88b.