3GPP TSG RAN WG1 Meeting RAN1 #89													R1-1707344

Hangzhou, P.R. China 15th – 19th May 2017

Source: 	Intel Corporation

Title:	SS Block based RRM measurements

Agenda item:	7.1.1.5.1

Document for:	Discussion and Decision

# Introduction

In this contribution, we discuss use of PBCH DM-RS for idle SS block based RRM measurements and the definition of SS Block RSRP. The following agreement regarding mobility were made in RAN1 #88bis.

Agreements: - Clarify previous RAN1 agreements on the RSRP definition for DL RRM measurements for L3 mobility as follows - Define SS block RSRP and CSI-RS RSRP as - SS block RSRP : measured RSRP from SSS - FFS additional use of PBCH-DMRS for measurement - CSI-RS RSRP : measured RSRP from CSI-RS in connected mode

# SS Block RSRP Definition & Reference Signal for RSRP

As discussed in our companion contribution [1], we propose to use the same antenna port for NR SSS and NR PBCH transmission. This implies that both reference signals could be available to the UE for performing SS block RSRP measurements. Furthermore, similar to LTE we can leave the exact measurement methodology to UE implementation. As long as the RSRP accuracy can be met, we do not believe any restriction should be made.

Based on the proposal made and the existing RSRP description from LTE, we propose to adopt the following SS block RSRP description.

Proposal:

- Adopt the following SS Block RSRP description.

------ Start of Proposed Text for SS Block RSRP -----

X.Y.Z SS Block Reference Signal Received Power (RSRP)

| Definition | SS Block Reference signal received power (RSRP), is defined as the linear average over the power contributions (in [W]) of the resource elements that carry NR SSS signals and NR PBCH reference signals for within the considered measurement frequency bandwidth. For RSRP determination the NR SSS signals and NR PBCH reference signals according to TS 38.211 [Z] shall be used. The reference point for the SS Block RSRP shall be the antenna connector of the UE. If receiver diversity is in use by the UE, the reported value shall not be lower than the corresponding SS Block RSRP of any of the individual diversity branches. |
| --- | --- |
| Applicable for | RRC_IDLE intra-frequency, RRC_IDLE inter-frequency, RRC_INACTIVE intra-frequency, RRC_INACTIVE inter-frequency, RRC_CONNECTED intra-frequency, RRC_CONNECTED inter-frequency |

NOTE 1: The number of resource elements among NR SSS signals and NR PBCH reference signals within the considered measurement frequency bandwidth and within the measurement period that are used by the UE to determine SS Block RSRP is left up to the UE implementation with the limitation that corresponding measurement accuracy requirements have to be fulfilled.

NOTE 2: The power per resource element is determined from the energy received during the useful part of the symbol, excluding the CP.

------ End of Proposed Text for SS Block RSRP -----

# Conclusions

In this contribution, we discussed the reference signal for SS block RSRP measurements. Our proposal is summarized as below:

Proposal:

- Adopt the SS Block RSRP description shown in Section 2 of this contribution.

# Reference

- R1-1707339, “Details on NR PBCH design,” 3GPP TSG RAN WG1 #89, Intel Corporation.