3GPP TSG RAN WG1 Meeting #89		                                                      R1-1707450

Hangzhou, China 15th - 19th May 2017

Source:	CATT

Title:	Discussion on resource pool sharing between mode 3 and mode 4

Agenda Item:	6.2.3.1.3

Document for:	Discussion and Decision

Introduction

There are two types of resource assignment for 3GPP LTE V2X communication, where the transmission resources of mode 3 UE are assigned by eNB, while the transmission resources of mode 4 UE are selected by UE itself through sensing. Both of the resource pools of mode 3 and mode 4 are configured by eNB, and also can be adjusted according to some measurements. At present the two resource pools are configured independently with each other.

In RAN #75 meeting [1], the following objective for radio resource pool sharing in PC5 of 3GPP V2X Phase2 work item is as follows:

- Specify solutions for the following PC5 functionalities, which can co-exist in the same resource pools as Rel-14 functionality and use the same scheduling assignment format (which can be decoded by Rel-14 UEs), without causing significant degradation to Rel-14 PC5 operation compared to that of Rel-14 UEs: [RAN1, RAN2, RAN4]

d)  Radio resource pool sharing between UEs using mode 3 and UEs using mode 4;

The advantage of dedicated resource pool lies in the collision avoidance between mode 3 UE and mode 4 UE since their resources come from different resource pools, and also lies in the collision avoidance between any two mode 3 UEs since both of their resources are assigned by eNB. However, dedicated resource pool may lead to low efficiency and inflexibility. For example, mode 4 UE is not allowed to select resources from mode 3 UE’s dedicated resource pool even if mode 4 UE’s dedicated resource pool encounters severe congestion. Although eNB can adjust resource pools according to some measurements, the adjustment is infrequent. Therefore radio resource pool sharing between UEs using mode 3 and UEs using mode 4 is investigated in this contribution for improving resource efficiency and flexibility.

Discussion

  - Analysis of issues on mode 3/4 pool sharing

If mode 3 UE and mode 4 UE share a common resource pool, from the point of resource selection, it has a negative impact on both mode UEs and mode 4 UEs.

According to the present specification [2], the resource reservation field of SCI format 1 for sidelink transmission mode 3 is set as zero. From the point of mode 4 UEs, it means that the resource indicated in the SA will not be kept for the transmission of the next transport block. So the mode 4 UEs will not try to exclude the resource even if the resource is actually occupied and reserved by a mode 3 UE and the RSRP measurement of the resource is higher than the threshold. Comparing to the dedicated resource pool, resource pool sharing can obviously increase the probability of resource collision between two types of UEs. Mode 4 UEs can avoid the collision of resources used for mode 3 UEs if mode 3 UEs set the field of resource reservation to SPS period or the mode type of mode 3 UE is indicated.

For resource selection of mode 3 UE, eNB may not acknowledge all information (particularly occupying resources) from UE which is not scheduled by eNB, i.e. mode 4 UEs. That makes it difficult to eNB to well select resources for mode 3 UEs while avoiding any transmission collision with mode 4 UEs.

From the above analysis, the problem brought by mode 3/4 pool sharing lies in the risk of resource collision between mode 3 UEs and mode 4 UEs. To ensure communication reliability, some enhancements can be further considered.

  - Discussion on the potential enhancements

Enhancements 1: SPS period is indicated in the resource reservation field of SCI format 1 of mode 3 UEs.

Proposal 1: Resource reservation field of SCI format 1 of mode 3 UEs shall contain the SPS period in case of mode 3/4 pool sharing.

Enhancements 2: The mode type is indicated in the SCI format 1.

If the mode 3 UEs can be distinguished from mode 4 UEs, mode 4 UEs can avoid the resources occupied by mode 3 UEs even if the actual SPS periods of mode 3 UEs are not indicated in the SA. According to the present specification, there are three reserved code points in resource reservation field in SCI format 1. If the resource reservation field of SA of mode 3 UE is set as one of the three reserved code points, mode 4 UE will recognize the type of mode 3 UE by decoding the SA, and can adopt special rules to protect mode 3 UE’s resources. For example, the SPS periods of mode 3 UEs can be considered as all of the system allowed SPS periods, and the resource occupied by mode 3 UEs can considered to be reserved for many times.

Proposal 2: Mode type shall be indicated in the SCI format 1 in case of mode 3/4 pool sharing.

Enhancement 3: eNB acknowledges resource occupying information as much as possible.

Since the resources of mode 3 UE are assigned by eNB itself, the main information eNB needs to know is resource selection of mode 4 UEs. So we have the following observation and proposals.

Observation1: eNB needs to know information, particular resource selection information, of mode 4 UEs when mode 3 and 4 UEs are sharing the same resource pool.

Proposal 3: UE reports the sensing results to eNB, if mode 3 and mode 4 UEs share the same resource pool.

The sensing results reported by UE are used for helping eNB to know the resource occupying level of shared resource pool, and then assign non conflict resources to mode 3 UEs. Thus, information such as the resource allocation, resource occupying level, occupying period could be included in the sensing results reported by UE. In addition, resource multiplexing may be considered by eNB for improving resource efficiency, and hence UE’s geographical information would be helpful. Report mode and resource granularity also should be considered and further discussed in sake of uplink overhead.

Proposal 4: The resource allocation, resource occupying level, occupying period and UE’s geographical information shall be included in the sensing results reported by UE if the report is verified.

Proposal 5: The report mode and resource granularity shall be further discussed.

If UEs report sensing results by RRC signal, overhead reduction should be considered to avoid severe impact to cellular users. The reporting overhead depends on report bits of each report, report frequency, and also the number of reporting UEs. For example, eNB can determine the report ratio and choose some of the UE to report.

Proposal 6: Report contents, frequency and number of reporting UEs can be configured by eNB with consideration on overhead.

Enhancement 4: Configure a standby dedicated resource pool for mode 3 UE.

The design concept of mode 3 transmission is avoiding resource collision. However, even with some enhancement, resource collision still may occur in the shared resource pool. For example, whether are the resource occupying level that eNB obtained from UE reporting correct and sufficient? For protecting transmission of mode 3 UE, it is possible configure a standby dedicated resource pool for mode 3 UE. As long as eNB is puzzled by the reporting, it can choose a resource from the standby dedicated resource pool to avoid potential collision.

Proposal 7: To configure a standby dedicated resource pool for mode 3 UE in case of mode 3/4 pool sharing.

Enhancement 5: Find and avoid the existed collision as soon as possible.

In addition, enhanced collision avoidance shall be considered. eNB and UE shall have a try to find the existed collision. There are 3 options to achieve the enhancement as follows:

- Option 1：eNB can broadcast resources used by mode 3 UE. When some mode 4 UEs find a collision, it can reselect other resources.

- Option 2：eNB also can find resource collision from the reporting of mode 4 UE if the resource occupied by mode 4 UE itself is indicated.

- Option 3： A collision report can be triggered if a UE finds a resource collision by sensing. If the collision is between a mode 3 UE and a mode 4 UE, eNB can adjust the resource of mode 3 UE, and if the collision is between two mode 4 UEs, eNB can inform one of them or both of them to avoid collision.

Proposal 8: Enhanced collision avoidance as above options shall be considered.

Conclusion

In this contribution, we have the following observation and proposals:

Proposal 1: Resource reservation field of SCI format 1 of mode 3 UEs shall contain the SPS period in case of mode 3/4 pool sharing.

Proposal 2: Mode type shall be indicated in the SCI format 1 in case of mode 3/4 pool sharing.

Observation1: eNB needs to know information, particular resource selection information, of mode 4 UEs when mode 3 and 4 UEs are sharing the same resource pool.

Proposal 3: UE reports the sensing results to eNB, if mode 3 and mode 4 UEs share the same resource pool.

Proposal 4: The resource allocation, resource occupying level, occupying period and UE’s geographical information shall be included in the sensing results reported by UE if the report is verified.

Proposal 5: The report mode and resource granularity shall be further discussed.

Proposal 6: Report contents, frequency and number of reporting UEs can be configured by eNB with consideration on overhead.

Proposal 7: To configure a standby dedicated resource pool for mode 3 UE in case of mode 3/4 pool sharing.

Proposal 8: Enhanced collision avoidance as above options shall be considered.

References

  - RP-170798, "New WID on 3GPP V2X Phase 2", Huawei, CATT, LG Electronics, HiSilicon, China Unicom, Dubrovnik, Croatia, 3GPP RAN #75.

  - 3GPP TS 36.213. “Introduction of V2X”.