3GPP TSG RAN WG1 Meeting #89		                                                      R1-1707499

Hangzhou, P. R. China, 15th – 19th May 2017

Source:	CATT

Title:	Discussion on NR DCI formats

Agenda Item:	7.1.3.1.4

Document for:	Discussion and Decision

# Introduction

NR would support more flexible DL control signaling mechanisms compared to LTE. Such flexibility relaxes the scheduling and timing constraints present in LTE, thereby providing an easier specification path for more advanced features in later NR releases. On the other hand a greater degree of flexibility inevitably results in increased signaling overhead at both physical and higher layers. Furthermore, array architectures and above 6GHz deployments would require additional signaling for CSI/interference measurement, beam management and, possibly, phase noise compensation. This contribution presents some options for constructing DCI formats. Furthermore, we provide a preliminary overview of the DCI fields that would be required in NR.

# DCI Structure

LTE DCI formats were defined based on the configured DL transmission modes, which led to an increasing number of DCI formats as new features were added.  Table 1 shows the maximum LTE TDD Rel-13 DCI format payload sizes without bit padding. Here we have assumed several features (CA, cross-scheduling, A-SRS) are enabled in order to highlight the maximum possible payload sizes for typical system bandwidths.

Table 1 LTE TDD Rel-13 DCI payload sizes including CRC with CA, CIF, A-SRS all configured

| BW (RBs) | 0 | 1 | 1A | 1B | 1C | 1D | 2 | 2A | 2B | 2C | 2D | 41 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 25 | 48 | 51 | 49 | 52 | 28 | 52 | 66 | 62 | 61 | 64 | 66 | 60 |
| 50 | 50 | 55 | 51 | 54 | 29 | 54 | 70 | 66 | 65 | 68 | 70 | 62 |
| 100 | 52 | 63 | 53 | 56 | 31 | 56 | 78 | 74 | 73 | 76 | 78 | 64 |

Note 1: DCI Format 4 assumes 4 TX antenna ports at UE

It can be seen that for DL MIMO-specific DCI formats 2 through 2D the payload sizes only differ by a few bits. In addition, if asynchronous HARQ (addition of at least 4 bits for HARQ process number) were configured for UL MIMO the format 4 payload size would not be significantly different from DCI formats 2A and 2B.

One proposal discussed in e.g. [1] is to simplify the DCI structure by defining only a small number of DCI formats. The motivation is that since many LTE DCI formats differ by only a few bits, it is simpler for NR to support a select few reference payload sizes. The DCI format could contain a header field which defines the information fields contained in the DCI. Taking LTE DCI formats in Table 1 as an example, the implication is that the actual payload size is quantized to fit one of these reference sizes.

Whilst this scheme obviates the need for padding bits to avoid ambiguity at the decoder, and also reduces specification effort, further refinement is required to actually provide meaningful power savings. In LTE the number of blind decodes depends on the transmission modes, number of aggregation levels (ALs) and number of search spaces. An LTE UE that is not configured for NB-IoT, eMTC or LAA needs to search in the UE-specific search space (UESS) for DCI formats 0/1A and for a DL transmission mode-dependent format such as 1/1B/1D/2/2A/2B/2C/2D. Furthermore, the UE also searches for DCI format 4 if UL MIMO is configured. Since there are 16 control channel candidates in the UESS, the UE performs up to 16 * 3 = 48 blind decodes. Therefore, one way to reduce the number of BDs is to define fewer payload sizes in NR. For instance, a small to moderate payload size may be introduced which can also be used for fallback signaling. A larger reference payload size may be defined for scheduling based on UE-specific RRC configuration.

A possible concern for this scheme can be observed from Table 1. For example, up to 14 padding bits (less for asynchronous UL HARQ) are needed to size match DCI format 4 to DCI Format 2D. This should not affect control channel reliability as a UE configured in Transmission Mode 10 should anyway be able to decode the larger payload size. Another possible concern could be with regards to NR-PDCCH capacity, if a moderate DCI payload then requires more NR-CCEs because it is quantized to a large reference payload size. This is not expected to have a material impact on capacity, or equivalently blocking probability, but would need to be evaluated once a more precise estimate of NR DCI sizes is determined.

- Proposal: Minimize the number of DCI payload sizes in NR.

  - Consider introducing at least one moderate and one larger reference payload size to support fallback and UE-specific scheduling respectively.

# DCI Contents

In this section we provide a preliminary overview of DCI fields based on existing agreements. Some of these agreements are summarized here in no particular order:

- Scheduling: Timing between DL assignment and corresponding DL data transmission,  or between UL assignment and corresponding UL data transmission, or between  DL data reception and corresponding acknowledgement is indicated by a field in the DCI from a set of values configured by higher layer signaling

- HARQ: asynchronous and adaptive HARQ is supported for DL and UL. CBG-based indication.

- PUCCH allocation: A combination of semi-static configuration and (at least for some types of UCI information) dynamic signaling is used to determine the PUCCH resource both for the long and short PUCCH formats. The PUCCH resource includes time, frequency and, when applicable, code domains.

- Data resource allocation: resource allocation for data transmission for a UE not capable of supporting the carrier bandwidth can be derived based on a two-step frequency-domain assignment process

  - 1st step: indication of a bandwidth part

  - 2nd step: indication of the PRBs within the bandwidth part

- UL MIMO: NR supports UL-MIMO scheduling by DCI, which includes at least some of

  - Indication of a SRS resource (SRI) which has been transmitted by this UE in previous time instance

  - Transmit Rank indicator (TRI)

  - Wideband transmit PMI (TPMI). FFS: Subband TPMI

- A-SRS: NR supports aperiodic SRS triggering field in DCI.

- QCL: For reception of unicast DL data channel, support indication of spatial QCL assumption between DL RS antenna port(s) and DMRS antenna port(s) of DL data channel: Information indicating the RS antenna port(s) is indicated via DCI (downlink grants)

- IMR: Aperiodic IMR is triggered by DCI.

Given these agreements we consider what additional fields are required in NR compared to reference LTE DL and UL DCI formats. In addition, existing fields would be enhanced to support advanced functionality, e.g. CSI request may be enhanced to support advanced CSI measurement/reporting. The exact number of bits for each information field is FFS. Table 2 describes the DL DCI contents using LTE DCI Format 2D as a reference. A corresponding description for UL DCI is given in Table 3. Completely new fields that are unique to NR in terms of functionality are shaded.

Table 2 Possible NR DL DCI contents

| Information field | Applicability for NR |
| --- | --- |
| Carrier indicator | Required for CA |
| Resource allocation type | Required if multiple RA types are defined in NR |
| Resource allocation | Possible support of 2-step frequency domain assignment |
| TPC | Required |
| DAI | FFS |
| HARQ Process ID | Required |
| AP/Scrambling ID/Number of Layers | Required |
| SRS request | Required |
| MCS1 | Required |
| NDI1 | Required |
| RV1 | Required |
| MCS2 | Required for more than 4 layers |
| NDI2 | Required for more than 4 layers |
| RV2 | Required for more than 4 layers |
| PQI | Required |
| IMR activation | Required |
| CSI-RS resource activation | Required |
| PUCCH resource allocation | Time-freq-code allocation |
| Reuse of control resources for data | New for NR |
| CBG-based indication | Possible field |
| DL HARQ-ACK timing | New for NR |
| DL Data timing | New for NR. Example is starting symbol and duration |

Table 3 Possible NR UL DCI contents

| Information field | Applicability for NR |
| --- | --- |
| Carrier indicator | Required for CA |
| Flag 0/1A | Not needed |
| Resource allocation type | Required if multiple RA types are defined in NR |
| Resource allocation | Enhanced to support 2-step frequency domain assignment |
| Frequency hopping flag | Needed if frequency hopping is supported |
| TPC | Required |
| DMRS CS/OCC | Depends on final RS design and UL waveform |
| UL Index/DAI | FFS |
| CSI request | Required |
| SRS request | Required |
| HARQ Process ID | Required |
| MCS1 | Required |
| NDI1 | Required |
| RV1 | Required |
| MCS2 | Not needed |
| NDI2 | Not needed |
| TPMI | Required |
| UL transmission timing | Enhancement required |
| SRI | Required |
| TRI | Required |

As this is only a preliminary view of DCI functionality it should be clear that without optimization NR DCI payload sizes may become unwieldy. Candidate fields for optimization include transmission timing, PDSCH/PUSCH/PUCCH resource allocation.

Proposal: target optimization of NR DCI payload sizes. Candidate fields for optimization include transmission timing, PDSCH/PUSCH/PUCCH resource allocation.

# Conclusion

This contribution provided some preliminary assessment of the construction and contents of NR DCI formats. The following proposals are the outcome of the discussion:

- Minimize the number of DCI payload sizes in NR.

- Consider introducing at least one moderate and one larger reference payload size to support fallback and UE-specific scheduling respectively.

- Target optimization of NR DCI payload sizes. Candidate fields for optimization include transmission timing, PDSCH/PUSCH/PUCCH resource allocation.

# References

  - R1-1702094, “Construction and mapping of DCI formats”, CATT, RAN1 #88