3GPP TSG RAN WG1 Meeting #89                                                                         	  R1-1707532

Hangzhou, China 15th  – 19th  May 2017

# Source:              CATT

Title:                  NR Power Headroom Calculation

Agenda Item:     7.1.9

Document for:   Discussion and Decision

Introduction

In RAN1#88bis, the beam-specific power control was agreed as follows,

Agreements:

- For beam specific power control, NR defines beam specific open & closed loop parameters.

  - FFS: details on beam common parameter(s)

  - Note: Agreed on RAN1 #88 FFS details on “beam specific”, especially regarding handling layer/layer-group/panel specific/beam group specific/beam pair link specific power control

- gNB is aware of the power headroom differences for different waveforms, if the UE can be configured for both waveforms.

  - FFS: offset configured/specified, reported,

  - FFS on the details of power control parameters for example, P_c, Max or other open/closed loop parameter

In this contribution, we discuss the NR power headroom calculation based on power control framework for PUCCH [4] and PUSCH with dynamic beam switching in multi-beam configuration [3].

Power Headroom Calculation

The UE power headroom is calculated based on the remaining power after the power used by PUSCH or PUSCH+PUCCH at slot i.

- The UE power headroom type 1 is defined as the remaining power after the power set for PUSCH transmission in a given slot as follows,

[dB]

Based on the proposed PUSCH power control framework in multi-beam configuration [3], the power headroom type 1

- is the  back off factor of operating waveforms , such as  CP-OFDM and DFT-S-OFDM waveforms, of serving cell c at slot I defined in [3]

- is the bandwidth of the PUSCH resource assignment expressed in number of resource blocks valid for slot i.

- is the numerology adjustment factor defined in [3]when the configured numerology of UL PUSCH transmission in slot i  is different to the default numerology.

- The target power consists of a cell specific nominal component and provided from higher layers and a UE specific component  provided by higher layers for j=0 and 1.

- is the scaling factor of fractional power control configured by higher layer

- is the path loss function.  For multiple path loss measurements from multiple detected beams in multi-beam configuration, path loss function is to support dynamic link adaptation or joint reception.  Path loss function of serving cell C could be defined as follows,

- is a UE-specific parameters for different type of traffic, such as UCI on PUSCH,  provided by higher layers

- is the power correction by close-loop power control.  if accumulation is enable and if accumulation is not enabled.    is a UE specific correction value through close-loop power control indicated by DCI.    is the offset with respect to the DL slot where the UL grant was received.

- The UE power headroom type 2 is defined as the remaining power after the power set for PUSCH +PUCCH transmission in a given slot as follows,

[dB]

Based on the proposed PUSCH power control framework in multi-beam configuration [3] and PUCCH power control, the power headroom type 2

- is a PUCCH format dependent value.

- where  is the current PUCCH power control adjustment

Conclusion

In this contribution, we discuss NR power headroom calculation based on the proposed PUSCH power control framework [3] and PUCCH power control [4].  We proposed type 1 and type 2 power headroom calculation in the following,

- Type 1 power headroom

- Type 2 power headroom

References

- R1-1706720	, “Summary of Issues on Power Control”, 	ZTE

- R1-1706841, “WF on uplink power control”, Nokia, ASB, InterDigital, OPPO, NTT DOCOMO, Intel, ZTE, Huawei, Ericsson, LGE

- R1-1707529, “PUSCH Power Control in Multi-beam configuration”, CATT

- R1-1707530	, “PUCCH Power Control and Priority in Power Scaling”, CATT