3GPP TSG RAN WG1 Meeting #89	R1-1707761

Hangzhou, China, 15th - 19th May2017

Agenda item:	7.1.2.3.2

Source: 	AT&T

Title: 	On the details of CSI acquisition

Document for:	Discussion/Decision

# Introduction

On RAN1#88b, several candidate interference measurement methods have been identified. In this paper, we give our view on interference measurement.

Agreements:

- For interference measurement, down selection from options will be conducted.

  - NZP CSI-RS based

    - Opt. A1: Estimation on NZP CSI-RS for channel estimation (by subtracting CSI-RS from Rx signal)

    - Opt. A2: Emulation on NZP CSI-RS which is represented by multiplied value of channel and precoding matrix

  - DM-RS based

    - Opt. B1: Estimation on DM-RS for own data demodulation (by subtracting DM-RS from Rx signal)

    - Opt. B2: Estimation on DM-RS for other UEs

- Criteria for design and down selection are as follows.

  - Required RS densities

  - UE processing latency

  - Support of self-contained CSI reporting (if supported) at least depends on the location of IMR.

- FFS: Whether the emulation is performed at TRP side or UE side

- FFS: RAN1 specification impact, if any, on the options above

# Resources for interference measurement

ZP CSI-RS resource has been used for interference measurement since LTE Rel.11. The benefit of using ZP-CSI-RS for interference has been extensively discussed at that time: Since all the received power on ZP CSI-RS resource REs is interference, there is no need to extract the useful signal out (in comparison to any NZP RS- based interference measurement). Such operation increases the measurement accuracy with an even simpler UE implementation. It is straight forward to adopt ZP CSI-RS for interference measurement in NR.

Then the question is which other RSs can be used as additional resource for interference measurement. Both NZP CSI-RS and DMRS are being considered in RAN1#88b. There are pros and cons to considering DMRS and NZP CSI-RS. Table-1 summarized the pros and cons of each options. DMRS has higher density which means a better channel estimation (Although the detailed RS pattern is not yet decided, it’s very likely that DMRS will have higher density than NZP CSI-RS). On the other hand, DMRS is transmitted along with PDSCH thus unscheduled UEs can’t use it to estimate interference. More importantly, the transmission of NZP CSI-RS can be easily coordinated across multiple gNBs in a semi-static fashion while the coordination of DMRS/PDSCH transmission requires dynamic coordination. Considering that, we prefer to use NZP CSI-RS for interference measurement in NR.

Proposal-1: Support ZP CSI-RS and NZP CSI-RS based interference measurement (Opt.A2 UE emulation) in NR

Proposal-2: Further study how to use DMRS for interference measurement in NR

Table-1 Summary of the pros and cons of RS used for interference measurement

|  | Pros | Cons |
| --- | --- | --- |
| Opt. A1 (NZP-CSI-RS for IM) | No need to have separate RSs for interference measurement | Require high density NZP-CSI-RS to guarantee an accurate own signal and interference measurement System load is incorporated in the measurement in statistic way, it’s hard to create an artificial interference hypothesis for coordination purpose |
| Opt. A2 (Emulation on NZP CSI-RS) | It’s easy to build artificial interference hypothesis between cells for CSI feedback. Reduce the number of needed NZP-CSI-RS when large number of interference hypothesis is needed. Only semi-static coordination is needed on the configuration of NZP-CSI-RS between cells. | Additional complexity: UE need to implement emulation function. |
| Opt. B1 (by subtracting DM-RS from Rx signal) | DMRS density is high enough for interference measurement | DMRS is transmitted together with data. Another interference measurement scheme is needed for the case where no data is transmitted. |
| Opt. B2 (DMRS for other UE) | UE doesn’t need to do emulation DMRS density is high enough | Require tight coordination between gNBs on the scheduling decision. |

# Using CRI to select preferable interference source

In RAN1 NR Adhoc, CSI acquisition framework has been refined: RS and IM settings are merged into resource settings. Furthermore, CRI is agreed to select multiple resources for the CSI report. The combination of these two agreements open a door to use CRI to select both the signal and the interference resources for the CSI report.

Figure 1 illustrates an example use case of using CRI to select both signal and interference sources. When configured by multiple NZP CSI-RS resources as interference measurement sources, the UE can use CRI to select a NZP CSI-RS resource for interference measurement. The selection is based on UE’s measurement on all the interference resources, e.g. UE may find I3 >> I2, therefore, UE selects to report the interference from I3. The detailed feedback of the selected interference source can be further decided in the work item, e.g. PMI of the NZP CSI-RS resource (for interference) can be fed back as a best/worst companion PMI.

NZP CSI-RS Resource-1

NZP CSI-RS Resource-2

NZP CSI-RS Resource-3

S1

I2

I3

ZP CSI-RS Resource

I_out

CQI=S1I2+Iout

CRI to select resources

Figure.1 using CRI to select the signal and interference sources for CSI report

Proposal-3: Consider to use CRI to select the NZP CSI-RS resources for interference measurement.

# Conclusion

In this paper, we give our view on the beam recovery mechanism for connected UE. In particular, we propose:

Proposal-1: Support ZP CSI-RS and NZP CSI-RS based interference measurement (Opt.A2 UE emulation) in NR

Proposal-2: Further study how to use DMRS for interference measurement in NR

Proposal-3: Consider to use CRI to select the NZP CSI-RS resources for interference measurement.

# Reference

- RP-160671, New SID Proposal: Study on New Radio Access Technology, NTT DOCOMO.