3GPP TSG RAN WG1 Meeting #89                                            	R1-1707918

Hangzhou, P.R. China 15th – 19th May 2017

Agenda Item:		6.2.6.2

Source:	 Samsung

Title:	 DL power consumption reduction for eMTC

Document for:	Discussion/Decision

# Introduction

In RAN #75 meeting, a Rel-15 new WID [1] on further efeMTC enhancements was agreed, including:

Improved power consumption:

- Power consumption reduction for physical channels [RAN1 lead, RAN2, RAN4]

  - Study and, if found beneficial for idle mode paging and/or connected mode DRX, specify physical signal/channel that can be efficiently decoded or detected prior to decoding the physical downlink control/data channel.

In RAN 1 #88bis meeting, some techniques were listed to be evaluated as:

Agreements:

- Techniques to be evaluated:

  - Wake-up signal/channel (either relying or not relying on DL synchronization)

  - Go-to-sleep signal/channel (either relying or not relying on DL synchronization)

  - Compact DCI

  - Reduced-bandwidth MPDCCH

  - Dynamic USS periodicity

In this paper, compact DCI was compared with wake-up signal/channel with dynamic USS periodicity.

# Techniques for DL power reduction

The techniques listed in RAN #88bis can be divided into two categories, one is about the use of channel/signal, and the other is about periodicity. Dynamic USS periodicity can be applied to all the above techniques. In this section, we only discuss the use of channel/signals.

For connected-mode DRX, currently eMTC already can set onDurationTimer to psf1 (i.e., 1 subframe) and drx-InactivityTimer and drx-RetransmissionTimer to psf0 (i.e., 0 subframe). In addition, there is DRX Command MAC control element to stop PDCCH monitoring, similar as the concept of “Go-to-sleep signal/channel”. The motivation for introducing a wake-up signal/channel may be using less receiving time to decode a “1” bit information to understandg if there is MPDCCH in the coming MPDCCH search space. However, this “wake-up signal/channel” may also needs some blind decoding, and may occupied similar resource as one MPDCCH candidate. On the other hand, even UE is waked up, it still need to monitor MPDCCH for PDSCH scheduling. Moever, with the transmision of wake-up signal/channel, it may increase “blocking rate” for eNB schedular to ensure no impact on legacy UEs. Considering the above analysis, there is no need to introduce any new signal/channel for connected-mode DRX.

Proposal #1: Not introduce new signal/channel, either wake-up or go-to-sleep signal/channel, for connected-mode DRX.

UE behaviors for paging for each technique assumed in this paper are assumed as:

  - Alt 1: Wake-up signal/channel (either relying or not relying on DL synchronization)

UE first detects for “wake-up signal/channel”, if detected, UE decodes MPDCCH and its scheduled PDSCH carrying paging; otherwise, UE goes to Idle.

  - Alt 2: Go-to-sleep signal/channel (either relying or not relying on DL synchronization)

UE first detects for “go-to-sleep signal/channel”, if detected, UE keeps in Idle; otherwise, UE decodes MPDCCH and its scheduled PDSCH carrying paging.

  - Alt 3: Compact DCI or Reduced-bandwidth MPDCCH (Modified MPDCCH)

UE detects for a modified MPDCCH and maximum maintain legacy behavior. Some fields in DCI are removed to reduce payload, which can be configured in SIB or pre-defined in the spec.

Table 1 summarizes DL receiving time for different techniques. Note that Ramp up and Ramp down are not included, since for each solution, the power consumption is assumed to be the same. Alt 1 and Alt 2 are supposed to have same DL receiving time. However, because the purposes of these two signal/channels are different, the system overhead and probability of false alarm Pfa and probability of missed detection Pmiss are also different. As pointed out in [2], the Pmiss needs to be quite low for wake-up signal/channel and Pfa needs to be quite low for go-to-sleep signal/channel, because the penalty is missing paging transmission.

Observation 1: Wake-up signal/channel and go-to-sleep signal/channel are expected to provide similar performance on power consumption.

Observation 2: Probability of missed detection needs to be very low (e.g., Pmiss <1%) for wake up signal/channel, and probability of false alarm needs to be very low (e.g., Pfa <1%) for go-to-sleep signal/channel.

Compared with Alt 1/ Alt 2 and Alt 3, if WUS or GSS using compact DCI which only indicates the present of PDSCH for the UE (e.g., UE-specific or group-specific “1-bit” information), then Alt 3 provides same DL receiving time (i.e., Twus or TGSS= TNew DCI) when there is no paging record for the UE in PDSCH. When there is UE paging record in following PDSCH, Alt 3 will save the MPDCCH monitoring time TMPDCCH. However, this means no other indication for PDSCH decoding (e.g., time/frequency resource, repetition number, TBS), which need to be pre-configured by eNB or pre-defined in the spec. This will sacrifice scheduling flexibility of PDSCH carrying paging. Some feasibility and pros/cons analysis will be provided in next section.

Observation 3: Introducing a new compact DCI (only indicate present of PDSCH for one group of UEs or one UE) is expected to provide slightly better power consumption performance by skipping MPDCCH detection when there is a PDSCH carrying paging for the UE.

In the summary of Table 1, the receive time for synchronization and MIB reading (for SFN) is assumed to be the same for all three alternatives, that is, WUS and GSS relying on DL synchronization. For the design which WUS or GSS doesn’t relying on DL synchronization, the receiving time or occupied DL resource are expected to be much more than only convey 1-bit information. In addition, for the case of sleeping time is very short (e.g., <0.67s), this feature is over-designed. On the other hand, if WUS and GSS can allow UE to skip reading of PSS/SSS for asynchronous case, the complexity of decoding such new signal is expected to be much higher. For LTE Cat 1 UE, the PSS/SSS module takes 10~15% of BOM cost of baseband [3]. The percentage of for PSS/SSS module is expected to be much higher for Cat M since other parts are reduced. Therefore, it is not recommended to design WUS and GSS, which can provide a functionality of PSS/SSS.

Observation 4: WUS and GSS not relaying on DL synchronization is expected to increase UE cost.

Figure 2 shows the simulation result to reduce DCI payload from 27 bits (DCI format 6-2) to 1 +16 bit. For normal coverage (MCL<144dB), if MPDCCH TTI keeps to be 1 ms, there is no power saving for MPDCCH decoding. For the reduced bandwidth, it may help on UE power saving if open a small RF bandwidth. In order to bring up more power saving, a symbol level based WUS/GSS is needed for normal coverage. However, other than potential UE complexity increasing, it will fragment DL resource.

For MCL =154dB and 164dB, the repetition can be reduced about 30%. This could save both power consumption and DL resource. In addition, since false detection rate is quite high with 16 bits CRC. It can be considered to further reduce CRC length.

Observation 5: Reducing DCI payload from 27 bits to 17 bits can reduce about 30% DL receiving time and DL overhead for UE in extended or extreme coverage.

Based on the analysis and observations above, it is proposed to introduce a compact DCI for paging.

Proposal #2: Introduce a compact DCI for paging to reduce DL power consumption.

Table 1  DL Receive Time for Different Techniques for paging

|  | Without target UE’s paging record in PDSCH | With target UE’s paging record in PDSCH |
| --- | --- | --- |
| Legacy behavior | Tsync+TMIB+TMPDCCH+X% TPDSCH† | Tsync+TMIB+TMPDCCH+TPDSCH |
| Alt 1: WUS | Tsync+TMIB+Twus | Tsync+TMIB+Twus+TMPDCCH+TPDSCH |
| Alt 2: GSS | Tsync+TMIB+TGSS | Tsync+TMIB+TGSS+TMPDCCH+TPDSCH |
| Alt 3: new MPDCCH | Tsync+TMIB+TNew DCI | Tsync+TMIB+TNew DCI+TPDSCH |

† X% is the percentage of paging for another UE.

Figure 1 Candidate techniques for DL power saving

# Compact DCI for DL power saving

Existing DCI for paging contains resource allocation (1~4 bits depends on system BW), MCS (3 bits), repetition number (3 bits), DCI subframe (2 bits). The field for Resource allocation (narrow band indication) can be removed by using the same narrow band as MPDCCH. Considering eMTC support paging on multiple narrow bands, it will not increase much on scheduling flexibility. 3 bits MCS is to determinate TBS. In order not causing collision problem for PDSCH carrying paging, some level of TBS may needs to be kept. It can be further study if MCS can be reduced to 1 or 2 bits. Repetition number of PDSCH may also be reduced. UE can do blindly decoding for different repetition numbers because current UE may early terminate DL reception time for power saving purpose. 2 bits are used to indicate MPDCCH repetition for determination of PDSCH starting subframe. As discussed in Rel-12, starting subframe of PDSCH can be associated with starting subframe of MPDCCH search space. At last, as analyzed in section 2, CRC can be reduced to 8 bits to further reduced DCI size. In total, DCI payload can be further reduced to 10 bits and additional 20% overhead and repetition saving can be expected. For normal coverage UE, a narrow band MPDCCH can be considered to further reduced power consumption without introducing a symbol level MPDCCH structure.

Observation 6: The fields for resource allocation, repetition number for both PDSCH and MPDCCH in DCI format 6-2 for paging can be eliminated. CRC can be reduced to 8 bits.

Dynamic USS periodicity is another technique to reduce DL power. It can combine with any above alternatives. For eDRX, a UE specific parameter is configured by upper layers. For DRX, DRX cycle is determined by the shortest of the UE specific DRX value, if allocated by upper layers, and a default DRX value broadcast in system information.  This is designed for normal LTE UE and reused for eMTC UE. However, for MTC/IoT traffic, the paging requirement may be quite different between UEs. Therefore, it is not reasonable to monitor a cell specific DRX if it is much smaller than the one allocated by upper layers. It is worthwhile to introduce a UE specific DRX cycle similar as eDRX. It could highly reduce the probability of decoding a MPDCCH or PDSCH without its paging.

Observation 7: Introduce UE specific periodicity can help to reduce UE power consumption.

Proposal #3: Send LS to RAN 2 to ask RAN 2 consider introducing a UE specific DRX cycle for eMTC to reduce UE power consumption.

# Conclusion

In this contribution, techniques for DL power consumption reduction for eMTC were discussed. A compact DCI with some pre-configured or pre-defined parameter for PDSCH carrying paging can provide the most power saving compared with new WUS or GSS signal. In additional, it will not increase UE or eNB complexity. Moreover, UE specific DRX cycle can be considered to further reduce DL power consumption. Based on the analysis, we proposed that:

Proposal #1: Not introduce new signal/channel, either wake-up or go-to-sleep signal/channel, for connected-mode DRX.

Proposal #2: Introduce a compact DCI for paging to reduce DL power consumption.

Proposal #3: Send LS to RAN 2 to ask RAN 2 consider introducing a UE specific DRX cycle for eMTC to reduce UE power consumption.

# References

RP-170732, “New WID on Even further enhanced MTC for LTE”, Ericsson, Qualcomm

R1-1705192, “Power consumption reduction for paging and connected-mode DRX”, Ericsson

TR 36.888 v12.00 “Study on provision of low-cost Machine-Type Communications (MTC) User Equipments (UEs) based on LTE”

R1-1703020, “NB-IoT/eMTC enhancement for mMTC”, Samsung

Figure 2