3GPP TSG RAN WG1 #89	R1-1707927

Hangzhou, P.R. China 15th - 19th May 2017

Agenda item:	7.1.1.1.2

Source:	Samsung

Title:	SS block composition and SS burst set composition

Document for:	Discussion

# Introduction

During RAN1#88bis meeting, the following agreements on SS block composition and SS burst set composition are achieved [1].

Agreements:

- Support about 1000 hypotheses provided by NR-PSS/SSS to represent NR physical cell ID for NR-SS design.

  - FFS: indication of radio frame boundary by NR-SSS

- In both single beam and multi-beam scenario, support only time division multiplexing of PSS, SSS and PBCH.

Working assumption:

- NR-PSS, NR-SSS and NR-PBCH are presented in every SS block

  - FFS: deactivated cell case (if defined)

Agreements:

- Number of symbols per SS block

  - 1 symbol NR-PSS

  - 1 symbol NR-SSS

  - 2, 3 or 4 symbols NR-PBCH (to be decided once the payload and NR-PBCH design has been agreed)

- FFS: Multiplexing of TSS/TSCH in SS block if TSS/TSCH is agreed to be introduced

- In a single SS block, the symbols are consecutive

Agreements:

- The considered maximum number of SS-blocks, L, within SS burst set for different frequency ranges are

  - For frequency range up to 3 GHz, the maximum number of SS-blocks, L, within SS burst set is [1, 2, 4]

  - For frequency range from 3GHz to 6 GHz, the maximum number of SS-blocks, L, within SS burst set is [4, 8]

  - For frequency range from 6 GHz to 52.6 GHz, the maximum number of SS-blocks, L, within SS burst set is [64]

- The way the value of L is reflected in specification is FFS

  - Aforementioned values are to be used to facilitate the NR initial access design and evaluate the specification impact

  - Possibility of having unified frequency agnostic signaling design is not precluded

In this contribution, the design considerations on the maximum number of SS blocks within an SS burst set, SS block components and the possible SS block locations are discussed.

# Maximum number of SS blocks in an SS burst set

In the legacy LTE at 2GHz, on average there is one SS block per 5 ms. RAN1 has agreed that the SS burst set periodicity will be 20ms for initial access. However, the SS periodicity for CONNECTED/IDLE & non-standalone cases could be configured as {5, 10, 20, 40, 80, 160} ms. For below 6GHz, the periodicity smaller than 20ms could be used to achieve similar sync acquisition time as that of LTE. Although it may be beneficial for a gNB to improve its cell coverage by transmitting SS blocks using multiple beams for below 6GHz NR, more than 4 SS blocks per SS burst set seems to cost too much overhead as shown in Table 1. Therefore, the maximum number of SS blocks per SS burst set for below 6GHz NR of 2~4 is more reasonable considering the sync cost/performance comparable with that of LTE.

Table 1 Overhead of time resource for different number of SS blocks when SS burst set Periodicity is 5ms(10ms)

|  | SCS (kHz) | Average # of OFDM symbols per SS block | Periodicity of SS burst set (ms) | Overhead in time domain |
| --- | --- | --- | --- | --- |
| LTE | 15 | 4 | 5 | 5.7% |
| 2 SS blocks per burst set | 15 | 4 | 5(10) | 11.4%(5.7%) |
| 4 SS blocks per burst set | 15 | 4 | 5(10) | 22.8% (11.4%) |
| 8 SS blocks per burst set | 15 | 4 | 5(10) | 45.6% (22.8%) |

Therefore, according to the agreements in last meeting, we have the following proposal on the maximum SS block numbers per SS burst set for NR.

Proposal 1:  The maximum number of SS-blocks, L, within SS burst set is carrier frequency dependent

- For frequency range up to 3 GHz, the maximum number of SS-blocks, L, within SS burst set is 2;

- For frequency range from 3 GHz to 6 GHz, the maximum number of SS-blocks, L, within SS burst set is 4;

- For frequency range from 6 GHz to 52.6 GHz, the maximum number of SS-blocks, L, within SS burst set is 64;

# SS block Composition

In RAN1#88bis, TDMed NR-PSS/SSS/PBCH was agreed. Number of OFDM symbols in an SS block should consider the decoding/detection performance of NR-PSS/SSS/PBCH. As agreed in last meeting, there will be 1 symbol of NR-PSS, 1 symbol of NR-SSS per SS block. The number of OFDM symbols for NR-PBCH can be 2, 3 or 4. However, as shown in our simulation results, 2 OFDM symbols per SS block are already good enough for the decoding of NR-PBCH. Therefore, we propose that 2 symbols for NR-PBCH per SS block.

Proposal 2: NR supports 2 symbols for NR-PBCH per SS block.

Figure 1 shows two alternatives on the SS block composition. For illustration, we assume 2RBs as GB and remaining 22RBs with 264REs available in PBCH for illustration.

- Alt1: 127-length NR-SSS sequence mapping on every two subcarriers

  - DMRS is IFDMed in PBCH symbols.

  - DMRS sequence is distributed in 22RBs per PBCH symbol

  - NR-SSS over 22RBs can be used for coherent detected of the DMRS sequence.

- Alt2: 127-length NR-SSS sequence mapping on consecutive subcarriers

  - DMRS is IFDMed in PBCH symbols.

  - DMRS sequence is distributed in the center 11RBs of the two PBCH symbols.

  - NR-SSS over 11 RBs/127REs can be used for coherent detected of the DMRS sequence.

Alt1						Alt2

Figure 1

Figure2 compares the DMRS and DMRS+SSS for PBCH demodulation using multiplexing structure of Alt1 and Alt2 respectively. The SSS and PBCH use same 1-port precoder cycling as the transmission scheme and the 4-shot combining is used within PBCH TTI of 80ms. The 1/4 DMRS overhead is used for fair comparison. With known DMRS sequence, the DMRS+SSS further improves the PBCH detection by around 1dB on top of DMRS only. For 30GHz with small delay spread, the difference between Alt1 and Alt2 is marginal because of the less selective fading channel.

- 4GHz with delay spread=100ns    (b) 30GHz with delay spread =30ns

Figure 2

Based on simulation in [4], we have confirmed no impact of the precoding cycling on the PSS/SSS detection. It is straightforward to use the same single-antenna port for NR-PSS, NR-SSS and NR-PBCH transmission.

Proposal 3: UEs assume same antenna port for NR-PSS, NR-SSS, and NR-PBCH.

Proposal 4: Together with DMRS, the NR-SSS using same antenna port should be considered to improve the channel estimation of NR-PBCH demodulation.

# SS block mapping pattern

For NR, similarly to LTE, we will use the terminology of frame, slot and OFDM symbol to describe the timing of SS blocks. According to the LS from RAN4 [R4-1704223], the potential subcarrier spacing is listed for each frequency range category.

- SCS supported for bands below 1 GHz

  - 15kHz, 30kHz

  - The decision of supporting 60kHz is pending RAN1 check

- SCS supported for bands between 1GHz and 6GHz

  - 15kHz, 30kHz, 60kHz

- SCS supported for bands above 24GHz and below 52.6GHz

  - 60 kHz, 120kHz

  - 240kHz is not applicable for data

- 240kHz for data can be further considered if a clear benefit is shown

- SCS support is band dependent

However, RAN1 has agreed that the default parameter set (SCS, sequence length, NR-SS transmission bandwidth) will be associated with a frequency band. Although the data/control channels may use various numerology of SCS and CP (NCP or ECP), we propose the NR-SS block mapping pattern is fixed/predefined and irrespective of the multiplexed data/control channels. By using the predefined pattern, the UE can detect the NR-SS blocks to get the slot/frame boundary without impact of the different data symbol boundary.

Proposal 5: For each frequency band, the NR-SS block mapping pattern is fixed/predefined and irrespective of the multiplexed data/control channels.

As the numerology of subcarrier spacing can change from 15kHz to 240kHz, the slot length and OFDM symbol length can change due to the change of different numerologies. Therefore, it is necessary for us to define a default slot (NR-SS slot) with predefined subcarrier spacing to describe the mapping of SS blocks. A NR-SS slot for below 6GHz is defined as a slot with subcarrier spacing of 15kHz and slot duration of 1ms. A NR-SS slot for above 6GHz is defined as a slot with subcarrier spacing of 60kHz and slot duration of 0.25ms. For both above 6GHz and below 6GHz, a NR-SS slot can be regarded as a 14-symbol slot with NCP. Figure 3 shows the NR-slot for both below 6GHz and above 6GHz NR.

Figure 3

Proposal 6: For each frequency band, NR-SS slots should be defined to describe the mapping pattern of SS blocks:

- For below 6 GHz, an NR-SS slot has 1ms time duration, equivalent to a slot with 14 OFDM symbols (e.g., #0~#13 OFDM symbol) with SCS=15kHz and NCP. Total number of NR-SS slots in a radio frame is 10.

- For above 6GHz, an NR-SS slot has 0.25ms time duration, equivalent to a slot with 14 OFDM symbols (e.g., #0~#13 OFDM symbol) with SCS=60kHz and NCP. Total number of NR-SS slots in a radio frame is 40.

Within an NR-SS slot duration, the NR-SS block location and the max number of NR-SS blocks are discussed as follows. Here, we assume an NR-SS block consisting of 4 consecutive OFDM symbols with SCS_SS=2n*15kHz and NCP. Considering the potential PDCCH/PUCCH in a NR-SS slot, the following alternatives of NR-SS block pattern are considered:

- Alt1: Consecutive NR-SS block(s) is(are) located in #2~#5 symbol (4 symbols) in the NR-SS slot

- No overlapping between NR-SS blocks and potential PDCCH in #0~#1 symbols and potential PUCCH in #6 symbol per NR-SS slot

  - Maximize no collision with PDCCH/PUCCH in 7-symbol half NR-SS slot

- The #7~#13 symbols may be used for data transmissions, including low latency (e.g., URLLC) packets. Furthermore, #7~#13 symbols allow flexible configuration of dynamic TDD.

- Note that for 120KHz sync SCS, only 32 SS blocks can be mapped within 5ms.

- Alt2: Consecutive NR-SS block(s) is(are) located in #3~#10 symbol (8 symbols) in the NR-SS slot

- No overlapping between NR-SS blocks and potential PDCCH in #0~#2 symbol and potential PUCCH in #11~#13 symbols per NR-SS slot.

  - Maximize no collision with PDCCH/PUCCH (with NCP or ECP, and potential GP before the PUCCH if needed) in 14-symbol NR-SS slot

- Put more NR-SS blocks within a NR-SS slot using doubled resources of Alt1 for NR-SS block mapping.

Figure 4

Table 2 Max number of NR-SS blocks per 1ms-subframe

|  | Max number of NR-SS blocks per 1ms-subframe |  |
| --- | --- | --- |
| for below6GHz | Alt1 | Alt2 |
| If SCS_SS=15kHz | 1 per 1ms | 2 per 1ms |
| If SCS_SS=30kHz | 2 per 1ms | 4 per 1ms |
| for above6GHz | Alt1 | Alt2 |
| If SCS_SS=120kHz | 2 per 0.25ms 4x2=8 per 1ms | 4 per 0.25ms 4x4=16 per 1ms |
| If SCS_SS=240kHz | 4 per 0.25ms 4x4=16 per 1ms | 8 per 0.25ms 4x8=32 per 1ms |

Table 2 illustrate the max number of NR-SS blocks per 1ms-subframe using Alt1 and Alt2 respectively for different SCS value for NR-SS block. If the actual number of NR-SS blocks is smaller than that in Table 2, one subframe is enough to include the NR-SS blocks and all the NR-SS blocks are transmitted at the beginning of NR-SS burst set periodicity, e.g., within 0th subframe within 10ms-frame. Otherwise, if the actual number of NR-SS blocks is larger than that in Table 2, more than one subframe is required within a NR-SS burst set duration.

In case that we need more than one subframe to include the NR-SS slots with NR-SS blocks, there are two options. Further comparison is needed.

- Opt(a): consecutive subframes for NR-SS slots

- Contiguous searching window

- Opt(b): non-consecutive subframes for NR-SS slots

- Non-contiguous searching window or contiguous searching window with longer time

- Time diversity gain across the NR-SS blocks distributed in discontinuous subframes

For example, as shown in Figure 5, the highlighted subframes will be used to include the NR-SS blocks. We assume using SCS=240kHz for NR-SS blocks and there are max 64 NR-SS blocks, the subframe location for NR-SS slot mapping for Alt1 and Alt2 in Figure 4 are illustrated.

- In case of Alt1, subframes include max 16 NR-SS blocks

- Opt (a): 4 subframes in {#0, #1, #2, #3}.

- Opt (b): 4 subframes in {#0, #1} and {#5, #6}.

- In case of Alt2, each subframe include max 32 NR-SS blocks

- Opt (a): 2 subframes in {#0, #1}.

- Opt (b): 2 subframes distributed in {#0, #5}.

Figure 5

# Conclusions

This contribution has examined design issues related to the maximum number of SS blocks, NR SS burst set composition and timing index indication, and made the following observations and proposals.

Proposal 1: The maximum number of SS-blocks, L, within SS burst set is carrier frequency dependent

- For frequency range up to 3 GHz, the maximum number of SS-blocks, L, within SS burst set is 2;

- For frequency range from 3 GHz to 6 GHz, the maximum number of SS-blocks, L, within SS burst set is 4;

- For frequency range from 6 GHz to 52.6 GHz, the maximum number of SS-blocks, L, within SS burst set is 64;

Proposal 2: NR supports 2 symbols for NR-PBCH per SS block.

Proposal 3: UEs assume same antenna port for NR-PSS, NR-SSS, and NR-PBCH.

Proposal 4: Together with DMRS, the NR-SSS using same antenna port should be considered to improve the channel estimation of NR-PBCH demodulation.

Proposal 5: For each frequency band, the NR-SS block mapping pattern is fixed/predefined and irrespective of the multiplexed data/control channels.

Proposal 6: For each frequency band, NR-SS slots should be defined to describe the mapping pattern of SS blocks:

- For below 6 GHz, an NR-SS slot has 1ms time duration, equivalent to a slot with 14 OFDM symbols (e.g., #0~#13 OFDM symbol) with SCS=15kHz and NCP. Total number of NR-SS slots in a radio frame is 10.

- For above 6GHz, an NR-SS slot has 0.25ms time duration, equivalent to a slot with 14 OFDM symbols (e.g., #0~#13 OFDM symbol) with SCS=60kHz and NCP. Total number of NR-SS slots in a radio frame is 40.

# References

- 3GPP, RAN1#88bis, Chairman’s Notes

- R1-1705328, Link budget analysis for SS block, Samsung

[3] R1-1705329, Composite beam transmission for SS block, Samsung

[4] R1-1707926, Remaining Details of SS Sequence Design, Samsung

[5] R1-1707929, Remaining issues of NR-PBCH, Samsung

# Appendix A: Evaluation assumptions for LLS

Table A1: Evaluation assumptions

| Parameters | Below 6GHz |
| --- | --- |
| Bandwidth | 7.65MHz |
| Subcarrier spacing | 30kHz |
| Carrier frequency | 4GHz |
| FFT size | 512 |
| Delay spread | 100ns, 1000ns |
| Modulation | QPSK |
| Antenna configuration | 2x2 |
| Period | 80ms / 4times repetition |
| Tx scheme | 1-port precoder cycling |
| Velocity | 3km/h, 120km/h |
| Channel model | TDL-C (mid corr) |
| Channel estimation | 2D MMSE |
| Channel coding | 1/3 TBCC |
| CFO | TRP: +/- 0.05 ppm, UE: +/- 0.1ppm |

Table A2: Evaluation assumptions

| Parameters | Above 6GHz |
| --- | --- |
| Bandwidth | 69.12MHz |
| Subcarrier spacing | 240kHz |
| Carrier frequency | 30GHz |
| FFT size | 512 |
| Delay spread | 30ns |
| Modulation | QPSK |
| Antenna configuration | gNB: (4,8,2,1,1), HPBW=65° UE: (2,4,2,1,2), HPBW=90° |
| Period | 80ms / 4 times repetition |
| Tx scheme | 1-port precoder cycling |
| Velocity | 3km/h |
| Channel model | CDL-C |
| Beam search | Fixed beam (to boresight direction) |
| Channel estimation | 2D MMSE |
| Channel coding | 1/3 TBCC |