3GPP TSG RAN WG1 Meeting#89	R1-1707955

Hangzhou, China, 15th – 19th May 2017

Agenda item:	7.1.2.2.3

Source: 	Samsung

Title: 	Link level simulation assumptions for beam management

Document for:	Discussion and Decision

# Introduction

In this contribution, we discuss the aspects of beam management that should be considered the link level simulation for beam management. The link level simulation results are shown in our companion contribution [1].

# Some evaluation aspects for beam management

Impact of beam pair link switching latency

In our view, an important aspect to be considered for evaluation would be the impact of beam pair link switching latency which comprises beam pair link measurement and reporting latency as well as beam pair link indication latency. When the UE moves, rotates or is blocked, the best Tx and Rx beam may change. If the gNB and UE can adjust its Tx and Rx beam without significant delay, serious performance degradation can be avoided. But since beam management procedures needs some time for the UE to report the beam measurement information to the gNB, and the gNB to indicate the beam information to the UE, the resulting beam pair link alignment and the impact to system performance should be evaluated.

Proposal 1: Impact of beam pair link switching delay should be considered in the beam management simulation evaluations.

Inducing beam pair link switching with UE rotation

UE rotation is a simple way to induce beam pair link change in link level simulation to evaluate the impact of beam misalignment and beam adjustment latency. We propose to model UE rotation in the azimuth direction with various rotation speeds, e.g. 10/60/120 RPM. Upon rotation, the UE reconfigures the Rx beam weight (equivalent to changing beam pair link) with a delay, where a number of delay values can be modelled to reflect the typical L1/L2/L3 signalling latency. We propose that UE rotation modelling is included in the link level simulation assumptions to simulate the impact of beam pair link switching.

Proposal 2: UE rotation modelling is included in the link level simulation assumptions to simulate the impact of beam pair link switching.

Based on the above discussion, our proposed change to the simulation assumption for link level is given in Table 1 (change is highlighted in red). Table 1 is based upon [2].

Table 1 Simulation assumption for link level

| Attributes | Values |
| --- | --- |
| Carrier Frequency | 4 GHz, 30 GHz |
| Subcarrier Spacing for data | For 4 GHz: 15kHz For 30 GHz: 120kHz, 60kHz (Other subcarrier spacings can be considered) |
| Data allocation | - 8 RBs - First 2 OFDM symbols for PDCCH, and following 12 OFDM symbols for data channel |
| PDCCH decoding | Ideal or Non-ideal (Companies explain how is modeled) |
| Channel Model | CDL-A /B/C model - delay spread =100ns - UE speed=3km/h. - The angles of BS, i.e., AoD, ZoD, are uniformly distributed within [-60, 60]/ [-12.5, 12.5] degrees in azimuth domain and [90, 135]/[103,108] degrees in zenith domain, and those of UE, i.e., AoA, ZoA, are uniformly distributed within [-180, 180]/[-30, 30] degrees in azimuth domain and [45, 90]/[80, 90] in zenith domain, via applying uniform-distribution desired mean angle in Section 7.7.5.1 in TR 38.900 accordingly. |
| TXRU mapping to antenna elements | Companies explain details of the using TXRU mapping to antenna elements. Notes: - 30GHz: 2D DFT based beam per polarization as a baseline; - 4GHz: 1D DFT per vertical dimension per polarization as baseline; |
| TXRU mapping weights | Companies explain details of the using TXRU mapping weights. |
| Procedure of beam sweeping | Companies explain details of procedure of beam sweeping. |
| Criteria for beam selection | Companies explain details of criteria for beam selection. |
| Beam pair link alignment | Companies explain details of beam pair link alignment procedure including latency of the procedure |
| UE reporting | Companies explain details of criteria for UE reporting. |
| BS antenna configurations | For 4 GHz: (M,N,P, Mg,Ng) = (8,8,2,1,1) as baseline. (dV,dH) = (0.8, 0.5)λ. For 30 GHz: (M,N,P,Mg,Ng) = (4,8,2,2,2). (dV,dH) = (0.5, 0.5)λ. (dg,V,dg,H) = (2.0, 4.0)λ Note: important to consider also other antenna configurations to maintain flexibility |
| BS antenna element radiation pattern | For 4GHz: According to TR36.873 For 30 GHz: According to TR38.802 |
| UE antenna configurations | For 4 GHz: Mg = 1, Ng = 1, P =2, (dV,dH) = (0.5, 0.5)λ, MxNxP<=8 (companies report M,N) For 30 GHz: (M, N, P, Mg, Ng) = (2, 4, 2, 1, 2); (dV,dH) = (0.5, 0.5)λ. (dg,V,dg,H) = (0, 0)λ. * Θmg,ng=90; Ω0,1=Ω0,0+180; Note: important to consider also other antenna configurations to maintain flexibility |
| BS array orientation | azimuth 0 degree; mechanic downtilt: 0 degree |
| UE array orientation | ΩUT,a uniformly distributed on [0,360] degree, ΩUT,b = 0 degree, ΩUT,g = 0 degree Companies explain whether UE rotation is modelled and how it is modelled |
| UE antenna element radiation pattern | For 4 GHz: Omni-directional with 5dBi gain For 30 GHz: See Table A.2.1-8 in TR 38.802 |
| Transmission scheme | Multi-antenna port transmission schemes Note: Companies explain details of the using transmission scheme. |
| MIMO mode | SU-MIMO[/MU-MIMO] |
| UE receiver type | MMSE-IRC as baseline; other advanced receiver is not precluded. |
| MCS | LTE MCS |
| Link adaptation | Based on CSI-RS |
| Metrics | - BLER w/ beamforming Proponents are encouraged to provide additional observations on SINR and RSRP. |

# Conclusions

This contribution gives our views on the link level simulation assumptions for beam management. Our proposals are given below.

Proposal 1: Impact of beam pair link switching delay should be considered in the beam management simulation evaluations.

Proposal 2: UE rotation modelling is included in the link level simulation assumptions to simulate the impact of beam pair link switching.

# References

[1] R1-1709034, Link level evaluation results for beam management, Samsung

[2] R1-1703533, Summary of [NRAH-01] email discussion on evaluation on beam management, ZTE