3GPP TSG RAN WG1 Meeting #89                                         	R1-1708023

Hangzhou, China, 15th – 19th May 2017

Agenda Item:		    7.1.3.3.2

Source:			Samsung

Title:		  Transmission of UL Data Channels

Document for:		Discussion and Decision

# Introduction

In the RAN1 AdHoc meeting on NR, the following were agreed [1]:

Agreements:

- For an UL transmission scheme without grant

  - at least semi-static resource (re-)configuration is supported

    - FFS: The resource configuration includes at least physical resource in time and frequency domain and RS parameters

    - Higher-layer signaling could be similar to Rel-8 LTE SPS

    - FFS: MCS

  - RS is transmitted together with data

    - channel structure of grant-based data transmission can be starting point

In the RAN1 #88 meeting, the following were agreed [2]:

Agreements:

- For UL transmission without grant,

  - The resource configuration includes at least the following

    - Time and frequency resources, FFS: including resources for repetitions, implicitly or explicitly

    - Modulation and coding scheme(s), possibly including RV, implicitly or explicitly

    - Reference signal parameters

      - FFS: Details

    - FFS: The number of repetitions K

      - FFS: Whether multiple number of K can be configured to one UE

    - FFS other parameters

  - FFS: A UE may continue repetitions for a TB until one of the following conditions is met

    - An ACK is successfully received from gNB

    - The number of repetitions for the TB reaches K

This contribution considers the transmission of UL data channels related to UL grant-free transmission.

# Discussion

From previous meetings, the grant-free transmission with semi-static resource configuration is considered for UL data transmissions, and the latency is expected to be reduced as the signaling procedure is significantly simplified. However,

One issue associated with grant-free transmission is the trade-off between the resource utilization and the reliability of data transmission. If the resource configured for grant-free transmission is assumed to be UE dedicated, which is similar to LTE SPS transmission, reliability is guaranteed but resource configuration may be inefficient since the UE-dedicated resource may not be frequently used considering the burstiness of URLLC services. UE-group common resource configuration can improve the resource utilization but the contention-based nature of UL data transmissions reduces the respective reliability when multiple UEs transmit in the same resources.

Although grant-free transmission based on semi-static resource configuration can reduce latency, the associated overhead can be large and the packet size for a service, such as URLLC service, can be variable. One pre-defined resource configuration is hard to satisfy all potential services or packet sizes. Therefore, switching between grant-free and grant-based transmissions is beneficial to balance resource utilization and delay/reliability requirement of associated services.

As discussed and proposed in [3], a UE configured for grant-free transmission is also configured to monitor UL grants.

Fig. 1 UE monitors the PDCCH for UL grant while configured with grant free transmission

If a UE is configured for grant-free transmission does not receive UL grant, it will transmit UL data by grant-free transmission for new data and re-transmission data. However, if UE receives UL grant, the grant-based transmission will be performed. This means that the received UL grant can override the grant-free transmission.

Proposal: the received UL grant could override the grant free transmission configuration.

# Conclusion

In this contribution considered resource configuration aspects for grant-free transmission and proposes the following:

Proposal: the received UL grant could override the grant free transmission configuration.

# References

- [1] Chairman’s Notes RAN1 NR AH, Jan. 2017.

- [2] Chairman’s Notes RAN1 #88, Feb. 2017.

- [] R1-1708024, “Grant-free and grant-based UL transmissions”, Samsung, RAN1 #89 meeting, May 2017