3GPP TSG RAN WG1 Meeting #89                                          R1-1708096

Hangzhou, China, 15th – 19th May 2017

Source: 	ETRI

Title: 	   NR MIMO Phase 2 SLS calibration results

Agenda Item:	7.1.2.5

Document for:	Discussion

Introduction

In [86-20] e-mail discussion, a phased approach is agreed for MIMO system/link level calibration [1] and in RAN1 #88, the following summary for NR MIMO calibration was agreed [2].

Agreements: - System level and link level NR MIMO calibration has been conducted through the following phased approaches: - Phase 1: Calibration is to check the link and system level channel models with basic beamforming behavior in terms of SNR/SINR distribution. - Phase 2: Calibration is to check the link/system level performance through observing the metrics of BLER, spectrum efficiency and outage. - Phase 3: Calibration is to check the additional link (if necessary)/system-level features including UE movement, UE rotation and channel blockage in terms of coupling loss, ASA, SINR and spectral efficiency. - Capture following simulation assumptions and results in TR 38.802. - R1-1701823 Evaluation assumptions for Phase 1 NR MIMO link level calibration - R1-1703534 Evaluation assumptions for Phase 1 NR MIMO system level calibration - R1-1703535 Evaluation assumptions for Phase 2 NR MIMO link level calibration - R1-1703536 Evaluation assumptions for Phase 2 NR MIMO system level calibration - R1-1701828 Evaluation assumptions for Phase 3 NR MIMO system level calibration - R1-1701829 Calibration results for Phase 1 NR MIMO link level calibration - R1-1701830 Calibration results for Phase 1 NR MIMO system level calibration

Based on the agreed evaluation assumptions [3], companies are encouraged to submit their calibration results and implementation details. In this contribution, we provide our phase 2 system level NR MIMO calibration results for indoor hotspot and dense urban macro scenarios.

Evaluation results

In this section, we provide user spectral efficiency (SE) results of phase 2 system level calibration for indoor hotspot (30GHz, 12 sites) and dense urban macro (4GHz) scenarios. Table 1 shows 5, 50, and 90-percentile of SE distribution along with mean SE where user SE is calculated as follows:

SE=(Total user throughput through simulation time)(simulation time × simulation bandwith).

Table 1: Spectral efficiency results (in bps/Hz)

|  | 5% SE | 50% SE | 95% SE | Mean SE |  |
| --- | --- | --- | --- | --- | --- |
| Indoor hotspot | 0.10 | 0.36 | 0.60 | 0.31 |
| Dense urban macro | 0.08 | 0.30 | 0.55 | 0.27 |

Figure 1 shows CDF of user SE for each scenario. To obtain the results, MMSE-IRC algorithm is adopted for UE receiver while the MMSE-IRC receiver was calculated based on the estimated channel from LTE DM-RS pattern [3]. On the transmit side, rank-1 SU-MIMO transmission based on SVD precoder is utilized. Detailed evaluation assumptions used for the evaluation can be found in Appendix.

Figure 1. CDF of spectral efficiency for Indoor Hotspot and Dense Urban Macro scenarios

Conclusion

In this contribution, phase 2 system level calibration results for indoor hotspot and dense urban macro scenarios based on the agreed evaluation assumptions has been provided.

References

[1] R1-1608661, “Summary of [86-20] email discussion on MIMO calibration for NR”, ZTE.

[2] Chairman’s Notes, RAN1 #88, February, 2017.

[3] R1-1701517, “Summary of email discussion [87-30] on NR-MIMO calibration”, ZTE. ZTE Microelectronics

- Appendix – Evaluation assumptions

Table A: Evaluation Assumptions for Phase 2 Indoor Hotspot System-level Calibration

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | 30 GHz |
| Mode | DL only |
| Bandwidth | 40MHz |
| Subcarrier Spacing | 60kHz |
| Channel Model | Indoor in TR 38.900 |
| TXRU mapping to antenna elements | One TXRU per panel per polarization |
| TXRU mapping weights | 2D TXRU virtualization weights for each panel is the Kronecker product between vertical and horizontal weight vectors taken from DFT, i.e., 2D sub-array partition model defined in TR36.897. |
| Criteria for selection for serving TRP | Maximizing RSRP with best analog beam pair, where the digital beamforming is not considered. The selection is done per drop, i.e., only at the beginning of drop and kept unchanged. |
| Criteria for beam selection for serving TRP | Select the best beam pair among the limited set of DFT beams, based on the criteria of maximizing receive power after beamforming. The selection is done per drop, i.e., only at the beginning of drop and kept unchanged. |
| Criteria for Beam Selection for interfering TRP | Considering the real traffic in adjacent cells, the actual beam or SVD precoder that is used by the non-serving TRPs in its data transmission is used as interfering beams. |
| Constraints for the range of selective beams per TRP sector | [0, 180] in azimuth domain and [0, 180] in zenith domain |
| Scheduling algorithm | Round robin |
| ISD | 20m |
| BS Tx power | 23dBm |
| BS Antenna Configuration | (M,N,P,Mg,Ng) = (4,8,2,1,1). (dV,dH) = (0.5, 0.5)λ Notes: Boresight direction is perpendicular to the ceiling. |
| UE Configuration | (M, N, P, Mg, Ng) = (2, 4, 2, 1, 2); (dV,dH) = (0.5, 0.5)λ. (dg,V,dg,H) = (0, 0)λ. Θmg,ng=90; Ω0,1=Ω0,0+180; Notes: the polarization angles are 0 and 90 |
| UE array orientation | ΩUT, uniformly distributed on [0,360] degree, ΩUT,= 0 degree, , ΩUT, = 0 degree |
| BS antenna pattern | See single sector pattern wall-mount pattern in Table A.2.1-7 in TR 38.802 |
| UE antenna pattern | See Table A.2.1-8 in TR 38.802 |

Table B: Evaluation Assumptions for Phase 2 Dense-Urban (Macro-layer) System-level Calibration

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | 4 GHz |
| Mode | DL only |
| Bandwidth | 20MHz |
| Subcarrier Spacing | 15kHz |
| Channel Model | 3D-UMa in TR36.873 |
| O2I penetration loss | Follow TR 36.873 |
| TXRU mapping to antenna elements | One TXRU per vertical dimension per polarization |
| TXRU mapping weights | TXRU virtualization only in the vertical dimension (i.e. 1D virtualization) using DFT, i.e., 1D sub-array partition model defined in TR36.897 |
| Criteria for selection for serving TRP | Maximizing RSRP with best analog beam pair, where the digital beamforming is not considered. The selection is done per drop, i.e., only at the beginning of drop and kept unchanged. |
| Criteria for beam selection for serving TRP | Select the best beam pair among the limited set of DFT beams, based on the criteria of maximizing receive power after beamforming. The selection is done per drop, i.e., only at the beginning of drop and kept unchanged. |
| Criteria for Beam Selection for interfering TRP | Considering the real traffic in adjacent cells, the actual beam or SVD precoder that is used by the non-serving TRPs in its data transmission is used as interfering beams. |
| Constraints for the range of selective beams per TRP sector | [-60, 60] in azimuth domain and [100, 160] in zenith domain |
| Scheduling algorithm | Round robin |
| Traffic Model | Full buffer |
| BS antenna element radiation pattern | According to TR36.873 |
| UE antenna element radiation pattern | Omnidirectional |
| ISD | 200m |
| BS Tx power | 44dBm |
| BS Antenna Configuration | (M,N,P,Mg,Ng) = (8,8,2,1,1) (dH,dV) = (0.5, 0.8)λ |
| BS array orientation | azimuth 0 degree; mechanic downtilt: 0 degree |