3GPP TSG RAN WG1 Meeting #89	R1-1708196

Hangzhou, China, 15-19 May, 2017

Agenda Item:	6.2.5.3

Source:	Huawei, HiSilicon

Title:	Evaluation on DL 1024 QAM

Document for:	Discussion/Decision

Introduction

In RAN Plenary #75, the new WI of "Enhancements for High Capacity Stationary Wireless Link and Introduction of 1024 QAM for LTE" has been approved [1]. The main objectives on support of DL 1024 QAM are as follows:

- Study the feasibility and performance benefit, and if justified, specify support for 1024QAM for DL channels [RAN1, RAN4]

  - Document the study result of feasibility and benefit in a TR

  - If normative work starts, introduce new MCS table, signalling and CQI feedback to support 1024QAM for PDSCH

In RAN1 #88bis, agreements on link-level evaluation assumptions were reached for 1024QAM [2]. In this contribution, corresponding evaluation results are presented to show feasibility and benefit of 1024QAM.

Link level results

Crossover point

Link level evaluation are performed for DL 1024QAM to achieve the crossover point between 256QAM and 1024QAM in different channel models, i.e., AWGN, TDL and various Tx/Rx EVM ranging between [0%, 3%]. The simulation assumptions are summarized in Table 5 of Appendix.

AWGN channel

For AWGN channel, the results of the evaluation are shown in Figure 1 and 2, and the crossover points are summarized in Table 1.

Figure 1.Crossover SNR for 1024QAM vs 256QAM with 2T2R

Figure 2.Crossover SNR for 1024QAM vs 256QAM with 2T8R

Table 1. Crossover SNR between 256QAM and 1024QAM

| antennas | Tx/Rx EVM(0%/0%) | Tx/Rx EVM(1%/1%) | Tx/Rx EVM(2%/2%) | Tx/Rx EVM(3%/3%) |
| --- | --- | --- | --- | --- |
| 2T2R | 29.2dB | 29.4dB | 30.4dB | 33.0dB |
| 2T8R | 23.2dB | 23.4dB | 24.0dB | 25.5dB |

The crossover points between 256QAM and 1024QAM with different TX/RX EVM values are summarized in Table 1. As EVM increases, the crossover SNR also increases. It can also be seen that the crossover SNR for 2T8R is 6dB less than that of 2T2R, as more receiving antennas could have larger processing gain.

Observation 1: For 2T2R case under AWGN channel,

  - the crossover SNR with 2% TX/RX EVM is ~30.4dB;

  - the crossover SNR is decreased by ~1dB with 1% TX/RX EVM;

  - the crossover SNR is increased by ~2.5dB with 3% TX/RX EVM.

Observation 2: For 2T8R case under AWGN channel,

  - the crossover SNR with 2% TX/RX EVM is~24dB;

  - the crossover SNR is decreased by ~0.6dB with 1% TX/RX EVM;

  - the crossover SNR is increased by ~1.5dB with 3% TX/RX EVM.

TDL channel

The results for TDL channel are presented in Figure 3, 4, and the crossover SNR are summarized in Table 2.

Figure 3.Crossover SNR for 1024QAM vs 256QAM with 2T2R in TDL channel

Figure 4.Crossover SNR for 1024QAM vs 256QAM with 2T8R in TDL channel

Table 2. Crossover SNR between 256QAM and 1024QAM(TDL channel)

| Delay spread | Tx/Rx EVM(1%/1%) | Tx/Rx EVM(2%/2%) |
| --- | --- | --- |
| 10ns,2T2R | 32.5dB | 35dB |
| 10ns,2T8R | 23.3dB | 24dB |
| 100ns,2T2R | 34dB | 36dB |
| 100ns,2T8R | 24dB | 25dB |

Observation 3: For 2T2R case under TDL, the crossover SNR with 2% TX/RX EVM is

  - ~35dB for 10ns delay spreading,

  - ~36dB for 100ns delay spreading.

  - the crossover SNR is decreased by ~2.5dB for 10ns delay spreading and ~2dB for 100ns delay spreading with 1% TX/RX EVM;

Observation 4: For 2T8R case under TDL, the crossover SNR with 2% TX/RX EVM is

  - ~24dB for 10ns delay spreading,

  - ~25dB for 100ns delay spreading.

  - the crossover SNR is decreased by ~0.7dB for 10ns delay spreading and ~1dB for 100ns delay spreading with 1% TX/RX EVM;

Throughput

We summarize the simulation results of throughput both in AWGN and TDL channel as shown in Table 3, 4.

Table 3. Summary of throughput in AWGN

| Channel | Case | Throughput(1024QAM,CR:0.745/256QAM,CR:0.9) |
| --- | --- | --- |
| AWGN | 2T2R, TX/RX EVM:0%/0% | ~175Mbps/~168Mbps@30dB |
| AWGN | 2T2R, TX/RX EVM:1%/1% | ~175Mbps/~168Mbps@30dB |
| AWGN | 2T2R, TX/RX EVM:2%/2% | ~175Mbps/~168Mbps@31dB |
| AWGN | 2T2R, TX/RX EVM:3%/3% | ~175Mbps/~168Mbps@34dB |
| AWGN | 2T8R, TX/RX EVM:0%/0% | ~175Mbps/~168Mbps@24dB |
| AWGN | 2T8R, TX/RX EVM:1%/1% | ~175Mbps/~168Mbps@24dB |
| AWGN | 2T8R, TX/RX EVM:2%/2% | ~175Mbps/~168Mbps@25dB |
| AWGN | 2T8R, TX/RX EVM:3%/3% | ~175Mbps/~168Mbps@28dB |

Table 4. Summary of throughput in TDL

| Channel | Case | Throughput(1024QAM,CR:0.745/256QAM,CR:0.9) |
| --- | --- | --- |
| TDL | 2T2R, DS:10ns, TX/RX EVM:1%/1% | ~164Mbps/~152Mbps@44dB |
| TDL | 2T2R, DS:10ns, TX/RX EVM:2%/2% | ~160Mbps/~148Mbps@46dB |
| TDL | 2T2R, DS:100ns, TX/RX EVM:1%/1% | ~155Mbps/~149Mbps@44dB |
| TDL | 2T2R, DS:100ns, TX/RX EVM:2%/2% | ~150Mbps/~145Mbps@46dB |
| TDL | 2T8R, DS:10ns, TX/RX EVM:1%/1% | ~175Mbps/~168Mbps@25dB |
| TDL | 2T8R, DS:10ns, TX/RX EVM:2%/2% | ~175Mbps/~168Mbps@26dB |
| TDL | 2T8R, DS:100ns, TX/RX EVM:1%/1% | ~172Mbps/~168Mbps@25dB |
| TDL | 2T8R, DS:100ns, TX/RX EVM:2%/2% | ~169Mbps/~167Mbps@26dB |

Obviously, when SNR reaches crossover point, 1024QAM shows benefit in throughput against 256QAM for different Tx/Rx EVM both in 2T2R and 2T8R case.

Considering the performance benefit, we propose that DL 1024QAM is supported.

Proposal 1: Support 1024QAM for DL PDSCH transmission.

Conclusion

In the contribution, we present evaluation results for DL 1024QAM, with the following observations and proposals:

Observation 1: For 2T2R case under AWGN channel,

  - the crossover SNR with 2% TX/RX EVM is ~30.4dB;

  - the crossover SNR is decreased by ~1dB with 1% TX/RX EVM;

  - the crossover SNR is increased by ~2.5dB with 3% TX/RX EVM.

Observation 2: For 2T8R case under AWGN channel,

  - the crossover SNR with 2% TX/RX EVM is~24dB;

  - the crossover SNR is decreased by ~0.6dB with 1% TX/RX EVM;

  - the crossover SNR is increased by ~1.5dB with 3% TX/RX EVM.

Observation 3: For 2T2R case under TDL, the crossover SNR with 2% TX/RX EVM is

  - ~35dB for 10ns delay spreading,

  - ~36dB for 100ns delay spreading.

  - the crossover SNR is decreased by ~2.5dB for 10ns delay spreading and ~2dB for 100ns delay spreading with 1% TX/RX EVM;

Observation 4: For 2T8R case under TDL, the crossover SNR with 2% TX/RX EVM is

  - ~24dB for 10ns delay spreading,

  - ~25dB for 100ns delay spreading.

  - the crossover SNR is decreased by ~0.7dB for 10ns delay spreading and ~1dB for 100ns delay spreading with 1% TX/RX EVM;

Proposal 1: Support 1024QAM for DL PDSCH transmission.

- References

RP-170838, “New WI proposal: Enhancements for High Capacity Stationary Wireless Link and Introduction of 1024 QAM for LTE”, Huawei, HiSilicon, Qualcomm Incorporated, China Telecom.

“RAN1 Chairman’s Notes”, RAN1#88bis, Spokane, April, 2017.

- Appendix

Table 5. Link-level simulation assumptions

| Bandwidth | 20MHz |
| --- | --- |
| Carrier frequency | 2G |
| Channel model and Doppler frequency | AWGN TDL with delay spread of {10ns,100ns} - Maximum Doppler frequency: 5Hz |
| Transmission mode | TM4 |
| MIMO configuration | 2x2/2x8 |
| Antenna correlation (Tx and Rx) | Uncorrelated |
| CRS configuration | Antenna ports 0,1 |
| Rank number | 2 |
| UE receiver | MMSE-IRC |
| Channel estimation | Practical |
| Interference estimation | Practical |
| Modulation mapping | Gray mapping (described in R1-1705007) |
| Overhead assumption | 3 PDCCH symbols; PBCH/PSS/SSS; 2-port CRS; |
| TX/RX EVM | 0%-3% |
| Metric | BLER(%)/Throughput(Mbps) |