3GPP TSG RAN WG1 Meeting #89	        		              R1-1708271

Hangzhou, P.R. China, 15th – 19th May 2017

Agenda item:	7.1.2.2.3

Source: 	National Instruments

Title: 	Discussion on Robust Beam Management

Document for:	Discussion and Decision

# Introduction

The 5G Study Item regarding the “Study on New Radio Access Technology” was approved at the 3GPP TSG RAN#71 meeting. It has the objective to be able to operate from sub 1 GHz to 100 GHz in a large variety of deployment scenarios in a single technical framework [1]. Beam management   becomes a major functionality and has been discussed in previous RAN WG1 meetings.

The following agreements have been made in RAN1#88b [2]

Agreements:

- For UE RRC connected mode, periodic signal is supported at least for P1 procedure (Tx/Rx beam alignment) using following options in addition to UE-specifically configured CSI-RS. Down selection from following options will be conducted in the next meeting.

  - Opt. 1: SS blocks

  - Opt. 2: Cell-specifically configured CSI-RS

    - Configuration of CSI-RS is obtained from the broadcast message (e.g., MIB, SIB)

  - Opt. 3: No additional option

Agreements:

- Study whether or not support mechanism for UE to provide L1/L2 reports based on SS-block measurements for beam management

  - FFS which channels/signals in SS-block for measurement

  - Especially in light of L3-RSRP

  - Study further whether or not to have a unified format for L1-RSRP measurement report of SS-block and CSI-RS

Agreements:

- For reception of DL data channel, study further at least the following:

  - Whether or not have an effective window of spatial QCL assumption

  - Interaction between higher layer signaling (if supported) and DCI indication

    - FFS the signaling details for higher layer and DCI based approaches (e.g., the corresponding information field in DCI, etc.)

  - Interaction between beam management and PDSCH transmission

  - Whether or not to have a default behavior (e.g., due to DCI miss detection), and if so the default behavior

  - Beam switching time, DCI decoding time, etc.

The following agreements have been made in RAN1#88 on the robustness of the control channel [3].

Agreements:

- Study further multi-beam based NR-PUCCH transmission for robustness against beam pair link blocking

  - E.g., UE transmits NR-PUCCH on different UL Tx beams in different NR-PUCCH OFDM symbols

    - FFS: multi-beam triggering condition/mechanism (e.g. event-triggered, network configured, etc.)

    - FFS: number of OFDM symbols for each beam

In this contribution, we present our views on some open issues on beam management with a focus on the control channel.

# Control Channel Beam Management

The beam based control channel in the DL and UL is required to be robust. In order to increase the robustness beam diversity has been proposed wherein the UE can receive the PDCCH on multiple beams from one or multiple TRP(s). The multiple beams can be in the same symbol if the UE can support multiple simultaneous RX beams, or they can be in successive symbols if the UE can support only a single RX beam at a time.

In addition to beam diversity, the choice of the beam(s) is also important to increase the control channel robustness. The PDCCH beam selection is expected to follow P1/P2/P3 procedure, where the UE will measure the beam and report its measurements back to the gNB. The gNB will then select the PDCCH beam(s) based on the reports from the UE.

It is common understanding that the beam RSRP is the main measurement reported by the UE. However, the beam selection algorithm depends on a variety of factors that need to be considered by the gNB, such as

- UE grouping based on which UEs can share the DL beam for PDCCH within the same symbol(s)

- Spatial multiplexing of PDCCH within each beam

- Robustness and reliability of the beam(s)

- Availability of data/control to be scheduled to/from UE in the given slot

- Other scheduler constraints

From the above list, it is noticed that while many of the factors involved in choosing the beam are scheduler implementation specific, there exists a need to feedback and track more beam metrics than only RSRP. If the beam selected is not robust, then there is higher probability of beam failure which leads to triggering of a time and resource consuming beam failure recovery process.

Observation 1: Selection of the beam(s) for the DL and UL control channel involves a variety of constraints and can be based on many measured beam measurement metrics in addition to RSRP.

We present below some suggested metrics to track robustness of the beam.

Long term beam quality:

It can be measured by filtering the instantaneous RSRP over multiple beam measurement occasions.

Beam CSI:

CSI Reporting on multiple beam can also give additional information such as expected CQI and Rank  on a given beam.

Inter beam interference:

The ability to measure and report the interference between beams, at same/different UEs will aid in scheduling UEs in a robust manner.

Beam Failure History:

Previous instances of beam failure on a given beam can help identify beams that have repetitive blockage patterns.

Reporting of Almost Failures of the Beam:

If the UEs can be configured to log and report (with a lesser periodicity) certain measurements from instances where the beam came close to failing but did not reach the stage of beam failure, this additional data on robustness of the beam can be obtained by the gNB.

Proposal 1: Study supporting of beam measurement metrics in addition to instantaneous RSRP to enable robust beam selection algorithms.

In addition to increasing reliability of the link, improving the robustness of the beam based control channel can impact beam reporting aspects, such as beam reporting periodicity. This in turn can reduce the beam management related overhead.

Proposal 2: Study the specification impact on beam management overhead of more robust beam selection metrics for the beam based control channel.

# Impact of AGC on Beam Sweeping Procedures

The AGC is trained by estimating the received energy level in the time and/or frequency domain. Typically rough training is implemented in the time domain training and fine training in the frequency domain. In LTE some part of the cyclic prefix can be used for AGC training, and as the AGC level changes only slowly over time, the power from previous received OFDM symbols can be used to train the AGC for current symbol.

In NR these assumptions may not hold, especially during a beam sweeping procedure (P1/P2)

- The symbol duration and the CP duration is significantly smaller than for 15 kHz SCS in above 6 GHz.

- Different beams can have a large amount of variation in the channel gain and received power level.

- Beam sweeping should support very fast beam change, upto once per OFDM symbol.

As a result, the successive symbols will need rapid and accurate AGC retraining.

# Conclusion

In this contribution, we make the following observations and proposals:

Observation 1: Selection of the beam(s) for the DL and UL control channel involves a variety of constraints and can be based on many measured beam measurement metrics in addition to RSRP.

Proposal 1: Study supporting of beam measurement metrics in addition to instantaneous RSRP to enable robust beam selection algorithms.

Proposal 2: Study the specification impact on beam management overhead of more robust beam selection metrics for the beam based control channel.

# References

[1]		RP-160671, ”New SID Proposal: Study on New Radio Access Technology”, NTT DoCoMo

[2]	    Chairman’s Notes, RAN1#88b, Final Version.

[3]    R1-1703730,	“Robustness of UL control channel”, 	LG, IDC