3GPP TSG-RAN WG1 #89 	R1-1708281

May 15th – 19th, 2017

Hangzhou, P. R. China

Agenda item:	7.1.3.3.4

Source: 	Apple Inc.

Title: 	Number of HARQ processes and UE category

Document for:	Discussion/Decision

# Introduction

The number of HARQ processes supported in NR is different from LTE, where UE and eNB budget the processing delay and soft buffer size assuming a fixed 8 parallel HARQ processes (at least for FDD).

In NR, it was already agreed that NR will support different number of HARQ processers on a per UE basis, as in the following agreement [1].

Agreements:

- NR UE supports a set of minimum HARQ processing time

  - FFS: set size

- NR supports different minimum HARQ processing time at least for across UEs

  - The HARQ processing time at least includes:

    - Delay between DL data reception timing to the corresponding HARQ-ACK transmission timing

    - Delay between UL grant reception timing to the corresponding 
UL data transmission timing

  - NR UE is required to indicate its capability of minimum HARQ processing time to gNB

    - FFS how the capability is indicated by UE

      - e.g. reported processing time granularity

      - e.g. dependency of DMRS pattern configuration

  - FFS definition of minimum HARQ processing time

Further, it was also agreed that some UEs will be able to support one HARQ operation (e.g. with self-contained slot):

Agreements: [2]

- NR supports operation of one DL HARQ process for some UEs

- NR supports operation of one UL HARQ process for some UEs

- FFS: Conditions on supporting above 2 bullets

- Note: This does not mean the gNB has to schedule back-to-back

- Note: This does not mean the UE has to support K1=0 and/or K2 = 0

Therefore, it is very important for UE to correctly indicate its capabilities to the network, and this also requires appropriate definition of UE category.

What potentially makes such efforts more complicated in NR is that NR can support different (scaled) numerologies at least from SCS=15kHz to 480kHz, which leads to variable TTI lengths accordingly. Therefore, it remains to be decided in RAN1:

- Is the (maximum) supportable number of HARQ processes numerology dependent? And

- How does the UE efficiently notify eNB its capability, i.e. how to efficiently define NR UE categories?

# Maximum Number of HARQ Processes

The number of HARQ processes a UE can support depends on several factors, including at least:

- Service dependent requirements

- numerology

- UE processing power (i.e. processing delay)

- UE soft buffer size

It is expected that NR should enable services with shorter delay compared to existing LTE. Therefore, it is desirable that NR can support smaller number of HARQ processes than 8, at least when configured with 15kHz SCS.

The number of (DL) HARQ processes that a UE can support also depends on the individual UE’s processing power, as well as the pipelining architecture. A simple example is illustrated below:

Figure 1 UE processing delay

For simple illustration, Figure 1 assumes a simplified UE architecture with two steps: front-end processing and back-end processing. Specifically, it also assumes pipelining resolution of TTI. With a fixed TBS per TTI (i.e. same data throughput), UE-1 with faster processing speed can support 1 HARQ processing delay, while UE w with slower processing speed only supports 2 HARQ delay. Obviously, UE-1 will potentially be more complicated and costly.

The number of (DL) HARQ processes that a UE can support also depends on the total soft buffer size available, similar as in LTE. Assuming a fixed block error rate (BLER) operating point at the UE, the soft buffer size is proportional to the data rate a UE can support, as well as the number of parallel HARQ processes:

Nsoft-bits∝bitsTTI∙NHARQ∝Rbps∙THARQs

If a UE is soft buffer size limited, higher throughput can only be supported with a smaller number of HARQ processes. NR is expected to support much wider bandwidth than LTE, and potential peak data rate as high as 20Gbps [2] to meet IMT2020 requirement. To support such a high peak throughput, it is desirable to reduce the number of parallel HARQ processes to reduce the required amount of soft buffer. This doesn’t prevent the scenario where larger number of parallel HARQ processes needs to be configured, but UE needs to reduce the data rate accordingly.

Notice that above statement assumes that HARQ buffer is implemented as on-chip memory. In real modem implementation, it also related to the actually implementation of memory management, and the modem sub-system can leverage off-chip memory to offload/load-back the HARQ soft-bits. Such operation can reduce the requirement on the amount of on-chip memory, but also have implications on power consumption. Exact how UE manage the HARQ buffer should be left as UE implementation decision, and spec should leave enough flexibility. Therefore, we have the following proposal:

Proposal 1:  UE should assume that peak data rate is supported only with a small number of parallel HARQ processes, and possibly reduced data rate is allowed at UE side (i.e. up to UE’s actually implementation and memory management) if a larger number of parallel HARQ processes needs to be configured by the network.

# UE Capability Indication

In LTE, the UE capability is characterised by the UE category, and is reported to eNB for scheduling purpose. The UE capability is simply categorized by:

- Maximum number of DL-SCH transport block bits (as well as per transport block) receives within a TTI

- Total size of DL-SCH soft channel bits

- Total number of layers

The UE category definition is straightforward in LTE due to the facts that:

- LTE supports fixed numerology: e.g. SCS=15kHz

- LTE assumes fixed number parallel HARQ processes: 8 (in FDD), with roughly equal split of UE/eNB processing margin. This translate a fixed processing delay requirement for UE (eNB)

In NR, however, settings can be more flexible. For example:

- TTI length is variable: due to the support of scalable SCS in 15kHz∙2k, including at least from 15kHz to 480kHz.

- Network can configure different number of parallel HARQ processes across different UEs.

- Network can configure different ACK timeline for different UEs

Therefore, it is important for NR to decide on

1) how to efficiently notify eNB what UEs’ capability is under different network settings; and

2) whether the existing way of defining UE category is still sufficient for NR?.

For example, a UE that supports maximal TBS=600k at peak throughput with SCS=15kHz may only support maximal TBS=300k with SCS=30kHz, due to hardware limitation. Similarly, a UE than supports maximal TBS=600k with 1 HARQ interlace may only supports maximal TBS=300k with 2 parallel HARQ interlaces due to soft buffer limitation. Therefore, a single setting of maximum number of DL-SCH transport block bits (as well as per transport block) may not be sufficient for NR UE categorization.

Proposal 2: NR should study how to define UE categories to efficiently indicate to eNB on the maximum supportable number transport block bits corresponding different network configurations (e.g. SCS, UE processing delay, and total number of parallel HARQ processes).

# Conclusions

In this contribution, we consider the factors that impacting the supportable number of parallel HARQ processes. We also consider the potential differences between NR and existing LTE, which may impact how UE capabilities should be indicated to the network. Specifically, we propose the following:

Proposal 1:  UE should support only a small number of parallel HARQ processes at peak data rate, and reduce the supportable data rate accordingly if a larger number of parallel HARQ processes needs to be configured by the network.

Proposal 2: NR should study how to define UE categories to efficiently indicate to eNB on the maximum supportable number transport block bits corresponding different network configurations (e.g. SCS, UE processing delay, and total number of parallel HARQ processes).

# References

- 3GPP RAN1 NR AdHoc Chairman Notes, “Chairman’s Notes RAN1_NRadhoc_final”, January 16th-20th, 2017,  Spokane, USA.

- 3GPP RAN1 #86 Chairman Notes, “Chairman’s Notes RAN1_86_final”, August 22nd-26th, 2016,  Gothenburg, Sweden.

- 3GPP TR 38.913 “Study on scenarios and Requirements for next generation access technologies”.

In practice, the pipeline resolution can be smaller, e.g. per-symbol, which leads to more efficient pipeline architecture with shorter delay.