3GPP TSG RAN WG1 Meeting #89                                     R1-1708329

Hangzhou, P.R. China, 15th – 19th May 2017

Agenda Item:	7.1.1.2.2

Source:	InterDigital Inc.

Title:	On Remaining System Information Delivery

Document for:	Discussion and Decision

# Introduction

It was agreed in the RAN1 meeting #85 [1] that both multi-beam based and single-beam based approaches should be studied for broadcast channel and system information transmission. In the RAN1 meeting NR Ad-Hoc [3], it was agreed to further study NR-PBCH design such as an NR-PBCH which carries information necessary for the UE to perform initial UL transmission. Furthermore in the RAN1 meeting #88bis [2] the following were agreed:

Agreements:

- NR-PDSCH carrying the remaining minimum system information is scheduled using NR-PDCCH.

- NR-PBCH provides configuration information for the NR-PDCCH scheduling the NR-PDSCH carrying the remaining minimum system information

- FFS if a part of configuration information can be derived by specification

Agreements:

- The broadcast delivery of other system information (OSI) is supported by NR-PDSCH transmission. The scheduling information of broadcast NR-PDSCH is considered to be carried by the following option(s):

    - Option 1: NR-PDCCH

    - Option 2: Remaining minimum system information

    - Other options are not precluded

Agreements:

- Down-select one of SCS options for PRACH msg. 3 transmission

  - Option 1: RACH configuration (possibly within PBCH or the remaining minimum system information) provides the SCS of the PRACH msg. 3

  - Option 2: The same SCS applied in PBCH transmission is used for the transmission of the PRACH msg. 3

  - Option 3: RAR can indicate the SCS of the PRACH msg. 3 transmission

- FFS the determination of the SCS for msg 1, 2, and 4

In this contribution, we considered and discussed some design principles for system information delivery for initial access for a beam-based approach in NR. We also discussed options how to deliver the remaining system information including remaining minimum system information and other system information.

# Remaining System Information Delivery

## Remaining Minimum System Information (RMSI) Delivery

NR-PBCH is a non-scheduled broadcast channel carrying at least a part of the minimum system information (MSI) in a fixed payload size. NR-PBCH is transmitted periodically, similar to the PBCH in LTE. The periodicity of NR-PBCH transmission should be predefined in the specification. To avoid a large overhead (especially in beam-based systems that may require repetition of the NR-PBCH for each beam) and for improved coverage, the payload size of the NR-PBCH should be minimized while carrying sufficient system information to enable basic initial access. As such, all of the MSI should not be carried by the NR-PBCH.

In order to minimize the payload size of NR-PBCH, the MSI may be partitioned into multiple parts, for example, the 1st and 2nd parts of the MSI. The 2nd parts of the minimum system information is referred to as remaining minimum system information (RMSI). By balancing the payload size of these two parts, the NR-PBCH performance can be optimized. With this design, there is a need to transmit the RMSI using different channel. In order to carry RMSI, the RMSI can be transmitted in a shared downlink channel via NR-PDSCH. Such a shared downlink channel may be a scheduled shared downlink channel. This is similar to the PDSCH in LTE for transmission of SIBs.

Transmission of RMSI can be indicated by the NR-PBCH for increased flexibility or pre-determined to reduce scheduling complexity. NR-PBCH can provide configuration information for the NR-PDCCH. NR-PDCCH can schedule the shared downlink channel which can transmit the RMSI. The NR-PBCH may be used to configure the RMSI or initial uplink transmission. For example, NR-PBCH could provide the control channel search space, control resource set or scheduling assignment for RMSI transmission, It is also possible that a part of configuration information can be derived by specification. For example, a part of the control channel search space/scheduling assignment could be derived by the specification. The numerology for NR-PDSCH for the RMSI should also be determined.

Proposal 1: The RMSI is transmitted in shared downlink channel via NR-PDSCH. NR-PBCH should carry configuration information for the RMSI transmission.

## Other System Information (OSI) Delivery

The broadcast delivery of other system information (OSI) can be supported by NR-PDSCH transmission. The scheduling information of broadcast NR-PDSCH may be carried by the NR-PDCCH or RMSI. Other options may also be considered. The maximum transport block size for OSI should be investigated.

A UE may use an initial UL transmission or PRACH to request for a beam(s) upon successful synchronization and reception of RMSI. The initial UL transmission or PRACH may be the request for beam(s) while a NR-PBCH may provide configuration or scheduling information necessary for the UE to receive the RMSI. RMSI may provide configuration information for initial UL transmission or PRACH. The initial UL transmission or PRACH may be used to request an activation or deactivation of the beam(s) for OSI transmission. The initial uplink transmission can be transmitted by (but not limited to) the NR-PRACH, e.g. NR-PRACH message 1.

The NR-PBCH may carry information necessary for the UE to receive RMSI. The NR-PBCH contents may include configuration information for transmission of RMSI. Such configuration information may include frequency and/or time resources for the RMSI and/or initial UL transmission. For example, configuration information for the RMSI may include control resource set, control channel search space or scheduling assignment for the RMSI.

Proposal 2: RMSI can carry information necessary for the transmission of OSI and PRACH.

In a beam-based system, the DL transmission of system information such as shared downlink channel may be based on all, or a subset, of the beams in a beam sweep. An initial UL transmission may provide information to the TRP to enable advanced beam-based schemes.

A selective beam sweeping scheme, using a subset of beams, for transmission of system information was evaluated in [4] and a performance gain due to the use of selective beam sweep was shown. This scheme is depicted in Figure 1.

The selective beam sweep may be enabled in stage 2 where each TRP may only transmit from L wide beams (where L beams are a subset of the Mw wide beams at the TRP). This operation may reduce interference, especially for non-uniform UE distributions. The TRP-based hierarchical beam sweep may be enabled to reduce overhead, and processing power, by using a smaller number of wider beams, and narrow beams may be selected in stage 2. Finally, the UE-based hierarchical beam sweep may be enabled to substantially reduce the latency since only NW sync bursts are used to complete a sweep. In stage 2 only the narrow beams contained within the detected wide beam may be used, providing additional savings on processing power.

Figure 1. Hierarchical based Selective Beam Sweeping

An initial UL transmission may be used to provide information to gNB or TRP to enable a selective beam sweep or hierarchical beam sweep procedure. For example, a UE beam-location profile, or information for the desired beam(s), may be provided to gNB or TRP during initial UL transmission.

Proposal 3: DL transmission of OSI may be based on all or a subset of the beams in a beam sweep. NR should study advanced beam-based schemes for OSI delivery.

System level simulations were performed for hierarchical based selective beam sweep. The TRP-based hierarchical beam sweep and UE-based hierarchical beam sweep are as described previously. The first and second stage association SINR is used as the primary metric for performance comparison. Simulation assumptions are provided in Table 2. Both hierarchical procedures were simulated using uniform and non-uniform UE distributions. For the simulations, a beam-pair corresponding to the maximum SINR for each UE is selected for each stage. The CDF of this “association SINR” is shown in Figure 2, with corresponding percentiles shown in Table 1. Note that since the effect of the selective beam sweep is observable only in stage 2, the stage 1 SINRs are similar for uniform and non-uniform UE distributions.  Further note that the gains of stage 2 compared to stage 1 are primarily due to the use of narrow beams at the TRP and UE, with additional gains possible via the selective beam sweep.

Figure 2. Hierarchical Selective Beam Sweep Procedure Performance Comparisons

Table 1. SINR Percentile for Figure 2

| Sweep Stage | Stage 1 |  | Stage 2 |  |
| --- | --- | --- | --- | --- |
| UE Distribution | Uniform | Non-uniform | Uniform | Non-uniform |
| 10% | 10.7 | 10.9 | 18.7 | 22 |
| 50% | 16.7 | 16.6 | 30.9 | 34 |
| 90% | 22.3 | 22 | 40.8 | 43.8 |
| Mean | 16.7 | 16.6 | 30 | 33.4 |

Observation 1: The performance gain is mainly due to 1) enhanced signal strength because of both Tx and Rx beamforming gain by narrower beams at TRP and UE, and 2) reduced interference because of disabled TRP Tx beams, the narrower TRP Tx beam and UE Tx beam. SINR can be improved when some allowable beams are turned off during beam sweep processing.

Observation 2: Higher SINR improvement is obtainable for non-uniform UE distribution than uniform UE distribution since more allowable beams may be turned off in case of non-uniform UE distribution.

# Conclusions

In this contribution, we considered and discussed some design principles for remaining system information delivery for initial access for a beam-based approach in NR. We discussed options how to deliver the RMSI and OSI. We have the following proposals:

Proposal 1: The RMSI is transmitted in shared downlink channel via NR-PDSCH. NR-PBCH should carry configuration information for the RMSI transmission.

Proposal 2: RMSI can carry information necessary for the transmission of OSI and PRACH.

Proposal 3: DL transmission of OSI may be based on all or a subset of the beams in a beam sweep. NR should study advanced beam-based schemes for OSI delivery.

# References

- Chairman’s Notes, 3GPP TSG-RAN WG1 #85, 23rd – 27th May 2016, Nanjing, China

- Chairman’s Notes, 3GPP TSG-RAN WG1 #88bis, 3rd – 7th April 2016, Spokane, USA

- Chairman’s Notes, 3GPP TSG-RAN WG1 NR Ad-Hoc, 16th – 20th January 2017, Spokane, USA

- R1-1612630, “Performance Evaluation for Beam Centric Design for Broadcast Channel”, InterDigital, 3GPP TSG-RAN WG1 Meeting #87, Reno, USA, November 14-18, 2016

# Appendix: Evaluation assumptions

Table 2. Simulation Assumptions

| Parameter | Value(s)/Description |
| --- | --- |
| Evaluation Methodology | System Level Analysis |
| Channel Model and Scenario | 3GPP TR 38.900 v 14.1.0 5GCM, UMi-Street Canyon |
| Percentage of Outdoor UEs | 100% |
| Carrier Frequency | 30 GHz |
| Bandwidth | 100 MHz |
| Number of Sites | 7 |
| Sectors Per Site | 3 |
| Number of UEs per Sector | 20 |
| UE distribution | - Uniformly distributed in cell center between 10 and 40 m from site - Non-Uniformly distributed in a 30° angular width region in the cell center between 10 and 40 m from the site |
| Number of Synch Resource Blocks | 6 |
| Sync Period | 5 ms |
| Base Station Antenna Element | HPBW = 65°, Directivity = 8 dB |
| UE Antenna Element | HPBW = 90°, Directivity = 5 dB |
| Sector Antenna Configuration | (M, N, P, Mg, Ng): Wide Beam = (1,3,2,1,1), Narrow Beam = (1,16,2,1,1) (dv, dH) = (0.5, 0.5)λ |
| UE Antenna Configurations | (M, N, P, Mg, Ng): Wide Beam = (1,2,2,2,1), Narrow Beam = (1,8,2,2,1) (dv, dH) = (0.5, 0.5)λ (dgv, dgH) = (0.0, 0.0)λ Ωmg,ng-→ α0,0=0°, α1,0=180° |
| Beamforming Weights | Phased Weighting |
| Sector Beam Directions (Azimuth Only) | Wide Beams = [-45, -15, 15, 45] Narrow Beams = [-60, -52, -44, 36], [-28, -20, -12, -4], [4, 12, 20, 26], [36, 44, 52, 60] |
| UE Narrow Beam Directions Per Panel (Azimuth Only) | Wide Beams= [-30, 30] Narrow Beams = [-55, -45, -35, -25, -15, -5], [5, 15, 25, 35, 45, 55] |
| UE Orientation | α: U[0,360]°, β: 90°, γ: 0° |