3GPP TSG RAN WG1 Meeting #89                                   	    R1-1708330

Hangzhou, China, 15th – 19th May 2017

Agenda item:		7.1.2.1.2

Source: 		InterDigital Inc.

Title: 			On Selection of DL Transmission Scheme 2

Document for:		Discussion/Decision

# Introduction

In the recent RAN1 meetings #88 and #88b WG1 did not arrive at a decision towards the selection of the downlink open loop transmission scheme. The last decision related to this topic, was made in RAN1 AH-NR meeting, where the following agreements on downlink Transmission Scheme (TS) 2 were reached [1]. The agreements called for a down selection of the proposed schemes for rank 1 and rank >2 transmissions.

- For Transmission scheme 2, down selection(s) on DMRS based transmission schemes will be done in RAN1#88 at least for rank 1

  - For rank 1,

    - Precoder cycling with transparent DMRS

    - Precoder cycling with non-transparent DMRS

    - Small-delay CDD with transparent DMRS

    - DMRS based SFBC

  - For rank>1,

    - Precoder cycling with transparent DMRS

    - Precoder cycling with non-transparent DMRS

    - Layer shifting

    - Precoder cycling with transparent DMRS and layer shifting

    - Small-delay CDD with transparent DMRS

    - Large-delay CDD with non-transparent DMRS

The SFBC transmission is considered among the rank 1 candidate transmission schemes for TS2. It has demonstrated its effectiveness as the preferred transmit diversity scheme in LTE. SFBC still performs well in both noise- and interference-limited scenarios. In this contribution, we present our views and simulation results on SFBC and some of other candidates for TS2.

# Considerations for Selection of a Transmission Scheme for TS2

There are several aspects in selection of a transmission scheme. As the preferred choice for TS2, the candidate scheme should exhibit a robust performance at different operational interference environment. For example, when operating at the cell edge, it has to be resilient enough to counter the in-band interference of neighbouring cells. Moreover, due to other use cases such as URLLC, where interference is not dominant, the preferred scheme should demonstrate a high degree of reliability that often is attributed directly to the available degree of diversity.

Also, due to the higher degree of diversity [3], it can be demonstrated that the SFBC transmission offers a more consistent performance at both low and high code rates.

MMSE-IRC receiver is the baseline receiver considered for interference mitigation and suppression evaluation. An MMSE-IRC requires an accurate estimate of the covariance matrix of the interference to be able to effectively suppress the interference signal. At the receiver, an estimate of the interference is often provided from measurements performed on reference signal (RS) ports [2]. It is known that due to the mismatch between the measured covariance matrix and the covariance statistics required for optimum operation of MMSE-IRC, some performance loss may be expected for SFBC. In [3], an interference randomization approach is presented to make the structure of the interference generated by other SFBC transmissions similar to the one of a spatial multiplexing transmission, and thus avoiding potential mismatch in interference estimation. However, in [4], it has been argued that the loss due to the mismatch may not be significant and could be avoided.

# Simulation Studies

In this section, we provide our evaluation results for the main candidates of the TS2 schemes, namely, SFBC, and precoder cycling. For precoder cycling scheme, the precoder for every RB is cycled at every subframe. For our evaluations, a comprehensive set of channel, interference and code-rate assumptions are considered. Table 1 captures the simulation assumptions used for the performance evaluation.

Figure set 1-3 show our evaluation results at different scenarios. From the presented results, following observations can be made:

Observation 1 – In a fixed INR scenario;

- At a low code rate, precoder cycling generally outperforms SFBC, however at high code rate SFBC is a better solution.

- At high Doppler spread values, SFBC performs as well or only slightly worse than the precoder cycling.

- At all scenarios, SFBC exhibits a higher diversity order than the precoder cycling.

Observation 2 – At a fixed high SIR scenario, SFBC always outperforms precoder cycling regardless of coding and channel assumptions.

Observation 3 – At a fixed low SIR scenario, SFBC generally outperforms precoder cycling regardless of coding and channel assumptions.

Proposal – Due to versatility and performance of SFBC in various scenarios, NR selects SFBC for TS2.

Table 1 Link-level Simulation Assumptions

| Parameter for LLS | Value |
| --- | --- |
| Channel model | TDL-A, TDL-B, TDL-C with DS = 100 ns and 300ns |
|  | Maximum Doppler frequency: 5Hz |
|  | MIMO: Low channel correlation |
| Allocation size | 6 PRB |
| BS antenna configuration | 2 Tx |
| UE antenna configuration | 2 Rx |
| Channel estimation | UE RS |
| Interference source | SFBC, RB-level precoder cycling |
| Noise estimation | Ideal |
| Interference | SIR = 30, 3 dB; INR = 10dB |
| CSI feedback periodicity | No CSI feedback |
| CSI feedback delay | No CSI feedback |
| UE receiver | MMSE-IRC |
| MCS | QPSK 1/3, 3/4 |
| Max HARQ transmissions | 0 |
| Precoder options (every TTI) | 1211,121j,121-1,121-j |

Figure 1 Performance comparison with fixed INR = 10dB

Figure 2 Performance comparison with fixed SIR = 30 dB

Figure 3 Performance comparison with fixed SIR = 3 dB

# Conclusions

Observation 1 – In a fixed INR scenario;

- At a low code rate, precoder cycling outperforms SFBC, however at high code rate SFBC is a better solution.

- At high Doppler spread values, SFBC performs as well or only slightly worse than the precoder cycling.

- At all scenarios, SFBC exhibits a higher diversity order than the precoder cycling.

Observation 2 – At a high SIR scenario, SFBC always outperforms precoder cycling regardless of coding and channel assumptions.

Observation 3 – At a fixed low SIR scenario, SFBC always outperforms precoder cycling regardless of coding and channel assumptions.

Proposal – Due to versatility and performance of SFBC in various scenarios, NR selects SFBC for TS2.

# Reference

Chairman’s Notes, 3GPP TSG RAN WG1 Meeting NR-AH, Spokane, USA, 16th - 20th January, 2017

R1-1612046, “DL Open-Loop MIMO Transmission Schemes”, Qualcomm Incorporated, Reno, NV, USA, 14th –18th November, 2016

R1-1702320, “On Interference Estimation Mismatch in SFBC Transmission”, InterDigital, Athene, Greece, 13th – 17th February, 2017

R1-1700407, “Analysis on interference estimation for open loop transmission scheme”, Huawei and HiSilicon,  Spokane, USA, 16th - 20th January, 2017