3GPP TSG RAN WG1 Meeting #89						R1-1708460

Hangzhou, P. R. China, 15th - 19th May, 2017

Source:	NTT DOCOMO, INC.

Title:	Evaluation results of DM-RS

Agenda Item:	7.1.2.4.2

Document for: 	Discussion and Decision

- Introduction

At the RAN1 #88bis meeting, we have reached the following agreements [1]:

R1-1706310 - Support at least the following design of DL DM-RS for data channels - Support the maximal 12 orthogonal DL DMRS ports for MU-MIMO # Agreements: - At least for slot, the location of front-loaded DL DMRS is fixed regardless of the first symbol location of PDSCH - FFS: Mini-slot case - Support ZC-sequence for UL DFT-S-OFDM DMRS # Conclusions: - Continue discussions/evaluations until the next meeting about following DMRS port multiplexing schemes for 2 adjacent front-loaded DMRS symbols in the time domain, and RAN1 will definitely conclude this down selection in the next meeting - Alt. 1: OCC - Alt. 2: TDM - Alt. 3: Frequency domain multiplexing only with the time domain repetition/ with a pattern shift - Alt. 4: Configure between Alt. 1 and Alt. 2 - Consider phase noise impact in the high frequency band - Alt. 5: Configure between Alt. 1 and Alt. 3 # Agreements: - For CP-OFDM, if one additional DMRS exists - At least for non-self-contained ACK/NAK slot, the time distance between the additional DMRS and front loaded DMRS for 14-symbol slot is larger than that for 7-symbol slot. - FFS additional DMRS position for 14-symbol slot - Consider symbol 12th, 11th, 10th, 9th - Study the location of additional DMRS for self-contained ACK/NAK slots - Evaluations are encouraged for next meeting # Conclusion: - Consider the issue of collision between DC subcarrier and DMRS. - Evaluate and analyze whether it can be solved by implementation or if DMRS design needs to take DC subcarrier into account Agreements: - Companies are encouraged to perform further evaluations on additional DM-RS symbols, using same or lower density compared with front loaded DM-RS, and also identifying use cases associated with the operation - Aim to decide in the next meeting whether to support same density only, or lower density only, or both - FFS at least CP-OFDM, frequency domain density of front loaded DMRS is configurable.

In this contribution, we provide link-level evaluation results for front-loaded DM-RS design.

- Link-level evaluation results

In this section, we provide initial link-level evaluation results. Firstly, we provide the evaluation results on front-loaded DM-RS pattern in frequency domain, and then we show the results on port multiplexing scheme in time domain for front-loaded DM-RS.

  - Front-loaded DM-RS pattern in frequency domain

Figure 1 shows the candidate front-loaded DM-RS patterns assumed in this evaluation. We compared 6 patterns selected based on company’s proposals considering avoidance of collision between DC subcarrier and DM-RS at least for up to two layers [2]. For layer multiplexing, FDM, cyclic shift (CS), frequency domain (FD)-OCC, and time-domain (TD)-OCC are applied. When applying FDM, 2-comb, 3-comb, or 4-comb, power boosting for each DM-RS port is applied. The simulation assumptions are summarized in table 1 (Appendix A). Carrier frequency is assumed to be 4 GHz and subcarrier spacing is 30 kHz. We assume 4-by-4 transmission and the number of transmission layers is set to one, two, and four. The data is assumed to be mapped to 32 PRBs and 12 OFDM symbols, where each PRB is composed of 12 subcarriers. As for channel estimation, MMSE filtering is applied.

(a) 2-comb + CS                          (b) 3-comb + CS                           (c) 4-comb

(d) 4FD-OCC                               (e) FDM + 2FD-OCC                    (f) FDM + 2TD-OCC

Fig. 1  Candidate front-loaded DM-RS patterns assumed in the evaluation.

Figures 2, 3, and 4 show the throughput performance when modulation order is QPSK (R = 1/3), 16QAM (R = 1/2), and 64QAM (R = 3/2) without PRB bundling. In these figures, we evaluate the variety of transmission layer (one, two, and four) and delay spread (10, 100, and 300 ns). Figures 5 and 6 show the throughput performance when QPSK Rank 1 with 4 PRB bundling, and 64QAM Rank 4 with 4 PRB bundling are applied.

The figures 2, 3, and 4 show that the robust performance gain can be achieved when applying pattern (a) compared with the other patterns irrespective of the number of layers, modulation order, and delay spread. As for pattern (d), performance gain can be achieved especially in one and two layer transmission, but large performance degradation is observed in four layer transmission and higher modulation order for larger delay spread case.

Regarding the number of front-loaded DM-RS symbols (one OFDM symbol or two OFDM symbols), the results show that there is no gain when applying two symbol front-loaded DM-RS pattern (pattern (f)) compared with one symbol pattern for up to four DM-RS ports. Thus, from view point of latency reduction, one symbol front-loaded DM-RS pattern is preferred at least for up to four DM-RS ports.

For 4 PRB bundling case (figure 5 and 6), the same tendency as no PRB bundling case is observed.

Based on these discussions, we make the following observation.

Observation 1:

- For pattern (a) (2-comb + cyclic shift), robust performance can be achieved.

- For front-loaded DM-RS pattern for up to four ports, there is no gain from two symbol front-loaded DM-RS pattern compared with one symbol front-loaded DM-RS pattern.

(a) DS = 10 ns                                               (b) DS = 100 ns

(c) DS = 300 ns

Fig. 2  Throughput performance (QPSK (R = 1/3)).

(a) DS = 10 ns                                               (b) DS = 100 ns

(c) DS = 300 ns

Fig. 3 Throughput performance (16QAM (R = 1/2)).

(a) DS = 10 ns                                               (b) DS = 100 ns

(c) DS = 300 ns

Fig. 4  Throughput performance (64QAM (R = 2/3)).

(a) DS = 10 ns                                               (b) DS = 100 ns

(c) DS = 300 ns

Fig. 5  Throughput performance (QPSK (R = 1/3), Rank 1, 4 PRB bundling).

(a) DS = 10 ns                                               (b) DS = 100 ns

Fig. 6  Throughput performance (64QAM (R = 2/3), Rank 4, 4 PRB bundling).

  - Port multiplexing scheme in time domain for front-loaded DM-RS

In the previous meeting, the following conclusions were made [1].

# Conclusions:

- Continue discussions/evaluations until the next meeting about following DMRS port multiplexing schemes for 2 adjacent front-loaded DMRS symbols in the time domain, and RAN1 will definitely conclude this down selection in the next meeting

  - Alt. 1: OCC

  - Alt. 2: TDM

  - Alt. 3: Frequency domain multiplexing only with the time domain repetition/ with a pattern shift

  - Alt. 4: Configure between Alt. 1 and Alt. 2

    - Consider phase noise impact in the high frequency band

  - Alt. 5: Configure between Alt. 1 and Alt. 3

In order to compare the performance of each multiplexing scheme, we conducted the link-level simulation. Figure 7 shows the candidate port multiplexing schemes, TDM, TD-OCC, and repetition based on the above previous conclusion. Regarding the number of symbols for front-loaded DM-RS, we assume that two symbol front-loaded DM-RS is applied for higher layer transmission such as more than 4 or 6 layers. However, for simplicity, we evaluate the port multiplexing scheme in time domain by two layer transmission. The simulation assumptions are summarized in table 2 (Appendix A). Carrier frequency is assumed to be 4 GHz and 30 GHz, and subcarrier spacing is 30 kHz and 120 kHz, respectively. We assume 4-by-4 transmission, and phase noise model proposed in [3] is adopted. In addition, we assume that PT-RS is inserted into the OFDM symbols after the 5th symbol and the influence of phase noise over the symbols is ideally compensated (Phase noise mainly impacts the 3rd and 4th OFDM symbols).

(a) TDM                                 (b) TD-OCC                                 (c) Repetition

(Each layer is multiplexed by F-CS)

Fig. 7  Candidate port multiplexing schemes in time domain assumed in the evaluation.

Figures 8 and 9 show the throughput performance when carrier frequency is assumed to be 4 GHz and 30 GHz. The modulation order is QPSK (R = 1/3), 16QAM (R = 1/2), and 64QAM (R = 3/2). The result shows that the same performance can be achieved even though the influence of phase noise is large such as figure 9 (c) (carrier frequency is 30 GHz and modulation order is 64QAM). This is because even if the channel estimation accuracy of each DM-RS RE is different among multiplexing schemes, channel estimation accuracy after applying MMSE filtering becomes almost the same. Thus, we make the following observation.

Observation 2:

- For port multiplexing scheme in time domain, the same performance can be achieved even if the influence of phase noise is assumed.

(a) QPSK (R = 1/3)                                               (b) 16QAM (R = 1/2)

(c) 64QAM (R = 2/3)

Fig. 8  Throughput performance (Carrier frequency = 4 GHz).

(a) QPSK (R = 1/3)                                               (b) 16QAM (R = 1/2)

(c) 64QAM (R = 2/3)

Fig. 9  Throughput performance (Carrier frequency = 30 GHz).

- Summary

In this contribution, we have presented our evaluation results of DM-RS, and then made the following observations.

Observation 1:

- For pattern (a) (2-comb + cyclic shift), robust performance can be achieved.

- For front-loaded DM-RS pattern for up to four ports, there is no gain from two symbol front-loaded DM-RS pattern compared with one symbol front-loaded DM-RS pattern.

Observation 2:

- For port multiplexing scheme in time domain, the same performance can be achieved even if the influence of phase noise is assumed.

References

- “Chairman’s notes: RAN1 #88bis”, Apr., 2017.

- R1-1708459, “Views on DM-RS,” NTT DOCOMO, INC.

- R1-165005, “On the evaluation of PN model,” Nokia, Alcatel-Lucent Shanghai Bell.

Appendix: Simulation Assumptions

Table 1  Simulation assumptions for figure 2 - 6

Table 2  Simulation assumptions for figure 8 and 9