3GPP TSG RAN WG1 Meeting #89	R1-1708489

Hangzhou, China 15th - 19th May 2017

Source:	NTT DOCOMO

Title:	Sequence design of Polar codes

Agenda Item:	7.1.4.2.1.2

Document for: 	Discussion

Introduction

In RAN1#88bis meeting, it was concluded that the following issues should be considered when designing Polar codes [1].

Conclusions: At least the factors relevant for evaluating the sequences are: - Performance - Information granularity - Compatibility with rate matching - Complexity - Latency

The structure of Polar codes is shown in Fig. 1. In [2], we have proposed a single nested sequence design for Polar codes. In this contribution, we will show more details of the method.

Fig. 1 The structure of Polar codes for NR

The following notations are used in this contribution.

K:		information block size

M:		codelength

N:		mother codelength,

K1:		number of input bits of the basic encoder

ui:		input bits of the basic encoder

xi:		coded bits of the basic encoder

Ui:		PDF of the decoded bits

Li:		PDF of the received symbols

I:			information set

QN:		ordered sequence of codelength N

Sequence of Polar codes

Polar codes have shown good performance compared to other coding schemes. However, as Polar codes are constructed by channel polarization phenomenon which is channel depended, Intuitively, Polar codes should be also constructed depending on the channel conditions.

Fortunately, from BOTH theoretical and practical points of view, a single nested sequence can be applied for Polar codes.

Theoretical point of view

In [3], it has been shown that if the codelength goes to infinite, for the same type of channels, the sequences are nested. Assuming two channel W1, W2 are both AWGN, and the capacity of W1 is larger than W2 i.e., SNRW1 > SNRW2, then the information set IW1 for W1 and IW2 for W2 have the following relationship.

It means that for different information block sizes k1, k2, …, if k1 < k2 < …, we have

Observation 1: For infinite codelength N, only a single sequence is needed for Polar codes.

Practical point of view

Unlike infinite codelength situation, for finite codelength situation, indeed, the information sets are different for different information block sizes. For different information block sizes, even the order of the sequence for the same codelength may be different. For example, for codelength N = 32 and information u = (u0, u1, …, u31), the order of u7 and u24 may be different due to the channel SNR. However, to construct good codes for practical application, the BLER performance is more important than capacity. We can still use the nested feature to construct sequence for Polar codes. In [4, 5], it is shown that even for single sequence, Polar codes can perform well.

Nested sequence design

Two nested structed should be consider when designing the nested sequence.

1. for the same codelength, nested for different information block sizes

2. nested ordered sequence for different mother codelengths

Nested for different information block sizes

Assume 1 bit granularity is supported, then for codelength N, there should be information sets for arbitrary information block sizes smaller than N and these sets should be nested, i.e.,

For example, for codelength N = 4, if , then  are nested sequences. If N goes to infinite, the nested feature will always meet. Note that we have

and

This means that an ordered sequence Q can be used to represented the nested sequences, where

Note that although the sequence Q is ordered, there is no need to keep the order for different information block sets. It is because that as long as the indices are selected as information bits, there is no difference between them during the encoding and decoding.

Each entry of Q is corresponding to an information block size, i.e., a coding rate, inspired by this representation, we can find this ordered sequence Q bit by bit. Then the following steps can be applied repeatedly to obtain the ordered sequence Q.

1) Find the index of I1, which is optimal for coding rate 1/N.

2) Find the index of Ik/Ik-1, which is optimal for coding rate k/N.

Nested for different mother codelength

In order to determine the indices of information bits and frozen bits, ordered sequences for mother codes are needed. Only one basis sequence is needed for polar codes and the other ones can be generated from the basis sequence following a given rule. In this section, we will show how to obtain the nested sequence based on the basis sequence of Nmax

Let  be the binary representation of i. Assume all the channel are i.i.d. Then when using density evolution to calculate the error probability of the sub-channels, f function is applied for b = 0, and g function is applied for b = 1. For each sub-channel Ui, usually the MSB part of “i” will be calculated first. For example, N = 8, for U4 = U100, the calculation order is g function, f function and f function.

Fig. 2 An example of density evolution procedure

Because the  functions are monotonic functions, for codelength N1 = 2n, if two different sub-channels  and  have the same LSB part “0”. Then for the same initial SNR, the order of  and  is the same with the order of  and  for codelength N2 = 2n-1. For example, if the ordered sequence of N1 = 8 is [0, 1, 2, 4, 3, 5, 6, 7], then the ordered sequence of N2 = 4 is [0, 1, 2, 3], which is shown in Fig. 3.

Fig. 3 An example of selecting the ordered sequence

Observation 2: The sequence of a short mother code can be obtained from the sequence of a long mother code.

Based on this observation, a sequence of any length N < Nmax can be selected using a unique basis sequence of length Nmax. The sequence can be obtained by using the MSB where the LSB = 0s. The detail steps are shown as follows.

1) Read the order index QNmax, and mother codelength N

2) Select the indices where the LSB = 0s less than M, and keep the order

Based on the above analysis, we have the following proposal.

Proposal 1: Nested ordered sequence is supported for polar codes

Summary

In this contribution, we described the sequence design of polar codes. Based on the analysis, we have the following Observations and Proposal.

Observation 1: For infinite codelength N, only a single sequence is needed for Polar codes.

Observation 2: The sequence of a short mother code can be obtained from the sequence of a long mother code.

Proposal 1: Nested ordered sequence is supported for polar codes

- References

- 3GPP, Chairman's Notes, RAN1 #88bis, Spokane, USA.

- 3GPP, R1-1705758, “Sequence design of Polar codes”, NTT DOCOMO, RAN1 #88bis, Spokane, USA.

- R. Blasco-Serrano, R. Thobaben, M. Andersson, V. Rathi, and M. Skoglund, “Polar Codes for Cooperative Relaying,” IEEE Trans. Commun., vol. 60, no. 11, pp. 3263 - 3273, Nov. 2012.

- 3GPP, R1-167209, “Polar code design and rate matching”, Huawei, HiSilicon, RAN1 #86, Gothenburg, Sweden.

- 3GPP, R1-1705425, “Design of a Nested Sequence for Polar Codes”, Samsung, RAN1 #88bis, Spokane, USA.

- Appendix

An example of the ordered sequence with N = 1024

|  | +0 | +1 | +2 | +3 | +4 | +5 | +6 | +7 | +8 | +9 | +10 | +11 | +12 | +13 | +14 | +15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 4 | 8 | 12 | 16 | 20 | 24 | 32 | 36 | 40 | 48 | 64 | 68 | 72 | 80 | 128 |
| 16 | 256 | 512 | 132 | 2 | 18 | 34 | 66 | 136 | 10 | 130 | 96 | 26 | 28 | 144 | 42 | 258 |
| 32 | 6 | 44 | 260 | 22 | 38 | 50 | 70 | 74 | 52 | 264 | 14 | 160 | 76 | 1 | 17 | 33 |
| 48 | 65 | 9 | 129 | 25 | 257 | 514 | 41 | 5 | 21 | 82 | 37 | 49 | 56 | 69 | 73 | 13 |
| 64 | 513 | 134 | 81 | 272 | 133 | 516 | 84 | 137 | 138 | 192 | 97 | 3 | 19 | 35 | 67 | 11 |
| 80 | 131 | 98 | 520 | 140 | 88 | 145 | 288 | 29 | 27 | 259 | 261 | 146 | 43 | 45 | 100 | 30 |
| 96 | 7 | 262 | 51 | 46 | 23 | 39 | 265 | 53 | 148 | 75 | 528 | 161 | 104 | 71 | 266 | 77 |
| 112 | 320 | 54 | 162 | 57 | 83 | 515 | 152 | 78 | 273 | 268 | 85 | 517 | 58 | 544 | 164 | 135 |
| 128 | 15 | 112 | 274 | 193 | 139 | 86 | 518 | 99 | 384 | 89 | 521 | 194 | 60 | 141 | 168 | 289 |
| 144 | 276 | 147 | 101 | 576 | 90 | 522 | 142 | 290 | 196 | 263 | 149 | 102 | 280 | 529 | 176 | 105 |
| 160 | 92 | 524 | 47 | 267 | 292 | 150 | 163 | 321 | 200 | 530 | 106 | 153 | 55 | 640 | 269 | 165 |
| 176 | 322 | 113 | 154 | 296 | 275 | 108 | 545 | 79 | 31 | 532 | 208 | 59 | 166 | 270 | 114 | 324 |
| 192 | 87 | 169 | 195 | 385 | 156 | 277 | 546 | 519 | 304 | 61 | 536 | 116 | 170 | 91 | 197 | 224 |
| 208 | 278 | 386 | 291 | 768 | 328 | 523 | 548 | 281 | 577 | 177 | 143 | 103 | 62 | 198 | 172 | 293 |
| 224 | 93 | 120 | 388 | 201 | 525 | 282 | 151 | 578 | 178 | 107 | 336 | 552 | 531 | 294 | 94 | 202 |
| 240 | 526 | 323 | 297 | 284 | 180 | 155 | 392 | 580 | 109 | 641 | 209 | 533 | 167 | 298 | 204 | 560 |
| 256 | 115 | 352 | 325 | 157 | 271 | 210 | 110 | 184 | 642 | 547 | 534 | 305 | 584 | 171 | 326 | 400 |
| 272 | 537 | 300 | 117 | 158 | 329 | 225 | 279 | 306 | 212 | 387 | 549 | 644 | 538 | 173 | 118 | 199 |
| 288 | 769 | 592 | 330 | 226 | 179 | 121 | 550 | 283 | 416 | 308 | 389 | 216 | 579 | 337 | 174 | 553 |
| 304 | 540 | 648 | 295 | 203 | 770 | 332 | 228 | 122 | 181 | 390 | 285 | 608 | 338 | 312 | 554 | 581 |
| 320 | 393 | 527 | 299 | 205 | 448 | 182 | 772 | 561 | 286 | 232 | 656 | 353 | 124 | 211 | 582 | 340 |
| 336 | 185 | 394 | 556 | 95 | 206 | 535 | 585 | 301 | 327 | 643 | 562 | 401 | 354 | 307 | 213 | 186 |
| 352 | 111 | 776 | 396 | 240 | 344 | 586 | 302 | 672 | 539 | 402 | 331 | 564 | 645 | 63 | 227 | 356 |
| 368 | 214 | 309 | 593 | 188 | 551 | 159 | 217 | 588 | 119 | 417 | 784 | 646 | 541 | 333 | 404 | 229 |
| 384 | 310 | 568 | 594 | 175 | 360 | 649 | 218 | 339 | 391 | 771 | 313 | 555 | 704 | 418 | 334 | 542 |
| 400 | 609 | 230 | 123 | 408 | 596 | 650 | 800 | 233 | 314 | 341 | 220 | 183 | 368 | 395 | 449 | 557 |
| 416 | 773 | 583 | 420 | 657 | 610 | 563 | 287 | 355 | 207 | 652 | 342 | 234 | 600 | 316 | 558 | 450 |
| 432 | 125 | 774 | 397 | 345 | 658 | 587 | 187 | 424 | 241 | 612 | 777 | 403 | 565 | 832 | 357 | 303 |
| 448 | 236 | 398 | 215 | 346 | 673 | 452 | 660 | 589 | 242 | 126 | 778 | 566 | 189 | 405 | 358 | 616 |
| 464 | 432 | 647 | 595 | 569 | 361 | 674 | 311 | 348 | 590 | 785 | 219 | 419 | 456 | 244 | 406 | 664 |
| 480 | 190 | 780 | 335 | 570 | 896 | 597 | 409 | 362 | 624 | 231 | 705 | 651 | 676 | 786 | 315 | 421 |
| 496 | 543 | 221 | 369 | 611 | 248 | 598 | 410 | 464 | 572 | 706 | 364 | 653 | 343 | 801 | 601 | 235 |
| 512 | 680 | 788 | 422 | 370 | 222 | 659 | 451 | 317 | 559 | 613 | 425 | 412 | 775 | 654 | 602 | 708 |
| 528 | 802 | 480 | 347 | 237 | 399 | 372 | 792 | 318 | 688 | 661 | 614 | 426 | 453 | 359 | 243 | 567 |
| 544 | 833 | 617 | 604 | 779 | 804 | 433 | 712 | 238 | 675 | 349 | 662 | 454 | 376 | 428 | 591 | 407 |
| 560 | 665 | 618 | 457 | 834 | 245 | 434 | 363 | 781 | 571 | 808 | 350 | 677 | 625 | 720 | 787 | 666 |
| 576 | 458 | 620 | 599 | 411 | 782 | 836 | 246 | 436 | 897 | 365 | 678 | 626 | 573 | 465 | 707 | 423 |
| 592 | 249 | 816 | 668 | 681 | 789 | 371 | 460 | 191 | 413 | 736 | 366 | 898 | 440 | 603 | 840 | 655 |
| 608 | 466 | 628 | 574 | 803 | 682 | 709 | 790 | 250 | 615 | 427 | 373 | 223 | 414 | 793 | 481 | 689 |
| 624 | 900 | 605 | 468 | 632 | 710 | 127 | 684 | 805 | 663 | 848 | 374 | 252 | 455 | 713 | 429 | 794 |
| 640 | 690 | 482 | 619 | 377 | 319 | 606 | 435 | 472 | 904 | 806 | 239 | 835 | 714 | 430 | 796 | 667 |
| 656 | 809 | 692 | 484 | 378 | 864 | 459 | 621 | 351 | 721 | 783 | 437 | 679 | 627 | 716 | 810 | 837 |
| 672 | 912 | 669 | 622 | 696 | 380 | 488 | 722 | 461 | 247 | 438 | 817 | 838 | 812 | 467 | 629 | 683 |
| 688 | 441 | 367 | 670 | 737 | 899 | 791 | 462 | 841 | 724 | 928 | 818 | 496 | 630 | 575 | 442 | 415 |
| 704 | 711 | 738 | 685 | 469 | 251 | 842 | 633 | 901 | 820 | 795 | 728 | 691 | 375 | 849 | 444 | 483 |
| 720 | 686 | 470 | 740 | 807 | 960 | 844 | 634 | 607 | 902 | 715 | 473 | 824 | 431 | 797 | 693 | 905 |
| 736 | 253 | 850 | 485 | 636 | 744 | 379 | 474 | 865 | 811 | 717 | 798 | 694 | 906 | 852 | 254 | 623 |
| 752 | 486 | 723 | 697 | 439 | 718 | 476 | 913 | 866 | 489 | 752 | 839 | 381 | 813 | 908 | 671 | 698 |
| 768 | 856 | 463 | 819 | 725 | 914 | 490 | 868 | 814 | 382 | 631 | 443 | 700 | 497 | 843 | 739 | 726 |
| 784 | 929 | 821 | 916 | 492 | 729 | 872 | 687 | 498 | 471 | 930 | 445 | 845 | 822 | 741 | 635 | 903 |
| 800 | 730 | 920 | 825 | 851 | 880 | 500 | 961 | 446 | 846 | 742 | 932 | 732 | 475 | 695 | 745 | 637 |
| 816 | 799 | 826 | 853 | 907 | 962 | 504 | 487 | 936 | 719 | 638 | 746 | 828 | 477 | 867 | 854 | 753 |
| 832 | 699 | 964 | 909 | 857 | 748 | 944 | 815 | 478 | 915 | 491 | 754 | 869 | 255 | 910 | 858 | 727 |
| 848 | 968 | 701 | 383 | 870 | 756 | 917 | 493 | 860 | 702 | 873 | 976 | 823 | 499 | 931 | 731 | 918 |
| 864 | 760 | 494 | 874 | 921 | 847 | 992 | 743 | 881 | 733 | 447 | 933 | 501 | 827 | 876 | 922 | 882 |
| 880 | 734 | 502 | 934 | 963 | 924 | 747 | 829 | 505 | 855 | 937 | 639 | 884 | 830 | 506 | 965 | 938 |
| 896 | 749 | 888 | 479 | 911 | 755 | 945 | 859 | 966 | 508 | 940 | 750 | 969 | 946 | 871 | 703 | 861 |
| 912 | 757 | 970 | 948 | 862 | 758 | 919 | 977 | 972 | 495 | 952 | 875 | 761 | 978 | 762 | 993 | 877 |
| 928 | 923 | 980 | 764 | 735 | 883 | 878 | 994 | 984 | 935 | 503 | 925 | 996 | 885 | 926 | 831 | 1000 |
| 944 | 886 | 939 | 507 | 1008 | 889 | 751 | 967 | 890 | 941 | 509 | 892 | 942 | 947 | 510 | 971 | 863 |
| 960 | 949 | 759 | 950 | 973 | 953 | 974 | 979 | 954 | 763 | 956 | 981 | 765 | 879 | 982 | 766 | 995 |
| 976 | 985 | 986 | 927 | 997 | 988 | 887 | 998 | 1001 | 1002 | 1009 | 1004 | 891 | 1010 | 943 | 1012 | 893 |
| 992 | 1016 | 894 | 511 | 951 | 975 | 955 | 1020 | 958 | 957 | 990 | 1006 | 1014 | 1018 | 1022 | 767 | 895 |
| 1008 | 959 | 983 | 987 | 989 | 991 | 999 | 1003 | 1005 | 1007 | 1011 | 1013 | 1015 | 1017 | 1019 | 1021 | 1023 |