3GPP TSG RAN WG1#89					          	                 	R1-1708503

Hangzhou, P.R. China, 15th – 19th May 2017

Agenda item:		7.1.3.1.2

Source:	Nokia, Alcatel-Lucent Shanghai Bell

Title:	On the hierarchical search space and UE blind detection

Document for:		Discussion and Decision

# 1	Introduction

Rel-14 NR study item [1] has been closed and a new Rel-15 WI dealing with New Radio Access Technology [2] has been approved. The work item should specify the NR functionalities for enhanced mobile broadband (eMBB) and ultra-reliable low-latency-communication (URLLC) as defined in [3]. The NR under this work item should consider frequency ranges up to 52.6 GHz.

This contribution relates to hierarchical search space and UE blind detection in NR. The following agreements related to PDCCH search space structures were made in RAN1 #87, RAN1 NR ad hoc and RAN1#88 [4][5][6]:

Agreements: [4]

- The time/freq. resource containing at least one search space is obtained from MIB/system information/implicitly derived from initial access information

- Time/freq. resource containing additional search spaces, can be configured using dedicated RRC signaling

- Other solution is not precluded

- when the control resource set spans multiple OFDM symbols, NR support a control channel candidate to be mapped to multiple OFDM symbols or to a single OFDM symbol

  - The gNB can inform UE which control channel candidates are mapped to each subset of OFDM symbols in the control resource set. FFS: details of the signaling (implicit or explicit)

Agreements: [5]

- NR supports at least following functionalities

  - At least for eMBB, in one OFDM symbol, multiple CCEs cannot be transmitted on the same PRB except for spatial multiplexing to different UEs (MU-MIMO)

  - A PDCCH candidate consists of a set of CCEs. A CCE consists of a set of REGs. A REG is one RB during one OFDM symbol.

  - For one UE, the channel estimate obtained for one RE should be reusable across multiple blind decodings involving that RE in at least the same control resource set and type of search space (common or UE-specific).

  - At least for DL data scheduled for a slot, the DL data DMRS location in time is not dynamically varying relative to the start of slot

- A control resource set is defined as a set of REGs under a given numerology

  - Control search space includes at least the following properties

  - Aggregation level(s)

  - Number of decoding candidates for each aggregation level

  - The set of CCEs for each decoding candidate

- FFS: if any of the following properties belong to control resource set or control search space

  - Transmission/diversity scheme

  - CCE to REG mapping

  - RS structure

  - PRB bundling size

- FFS: if the control resource sets can overlap or not

- FFS: whether the mapping between control resource set and control search space is one-to-one or one-to-many

Agreements: [6]

- Multiple control resource sets can be overlapped in frequency and time for a UE.

- A search space in NR is associated with a single control resource set

  - The search spaces in different control resources sets are defined independently.

- The max number of BD candidates for a UE is defined independently of the number of control resource sets and the number of search spaces.

- Further study the following alternatives:

  - Alt 1: For a given control resource set, there is only one CCE to REG mapping scheme

  - Alt 2: For a given search space, there is only one CCE to REG mapping scheme

We provide more details related to the search space structure in a companion contribution [7].

# 2	Discussion

Based on the agreement made in January AH meeting, for one UE, the channel estimate obtained for one RE should be reusable across multiple blind decodings involving that RE in at least the same control resource set and type of search space. This can be achieved with a hierarchical CCE stucture where channel estimate for a lower aggregation levels can be obtained based on channel estimates of the highest aggregation level.

It is noted that when PDCCH occupies the same PRBs in consececutive OFDM symbols, then channel estimate for the second symbol is available also for the first symbol (and vice versa). This will improve the channel estimation performance accordingly, and it may happen not only with time first mapping of REG to CCE, but also in the case of frequency first mapping, provided that different CCEs occupy the same PRBs.

Proposal #1: Consider hierarchical CCE stucture where channel estimate for a lower aggregation level can be obtained based on channel estimates of highest aggregation level

Another agreement made in RAN1#88 is that, the max number of BD candidates for a UE is defined independently of the number of control resource sets and the number of search spaces. We think that in the case of slot based scheduling the max number of BD candidates for a UE per slot should not depend on the number of OFDM sybols in the control resource set. On the other hand, in the case of mini-slot based scheduling when the PDCCH monitoring takes place in the middle of the slot, the UE may have already processed the BDs corresponding to slot based scheduling. Hence, there might be opportunities for increased number of BD candidates for a UE per slot compared to slot based scheduling.

It should be possible for gNB to share the number of BD candidates among different user specific search spaces. There is a need also for rules to share the available BD candidates between different aggretation levels within the serach space, as well as between different OFDM symbols of the slot.

Proposal #2: In the case of slot based scheduling the max number of BD candidates for a UE per slot does not depend on the number of OFDM symbols in the control resource set(s).

Observation #1: Mini-slot based operation may support increased number of BD candidates for a UE per slot compared to slot based scheduling.

One of the issues related to hierarchical control channel structure is increased blocking probability. This depends also on the scenario of interest, and the amount of CCEs configured to each search space. In order to minimize the DL control channel blocking there is a need for search space specific hashing function similarly as in LTE to randomize the overlapping search spaces of different UEs. However, in order to maintain hierarchical CCE stucture, the hashing function of certain aggregation levels must operate within CCEs occupied by highest aggregation level. Furthermore, there is a need to consider also allocation of non-contiguous CCEs at least for certain aggregation levels.

Proposal #3: Consider a search space and AL-specific hashing function to randomize the overlapping search spaces of different UEs.

Observation #2: In order to maintain hierarchical CCE stucture, the hashing function of certain aggregation level must operate within CCEs occupied by highest aggregation level.

# 3	Conclusions

In this contribution we have discussed hierarchical search space and UE blind detection in NR. Based on the discussion, we make the following proposals and an observation:

Proposal #1: Consider hierarchical CCE stucture where channel estimate for higher aggregation levels can be obtained based on channel estimates of lower aggregation level

Proposal #2: In the case of slot based scheduling the max number of BD candidates for a UE per slot does not depend on the number of OFDM symbols in the control resource set(s).

Proposal #3: Consider a search space and AL-specific hashing function to randomize the overlapping search spaces of different UEs.

Observation #1: Mini-slot based operation may support increased number of BD candidates for a UE per slot compared to slot based scheduling.

Observation #2: In order to maintain hierarchical CCE stucture, the hashing function of certain aggregation level must operate within CCEs occupied by highest aggregation level.

# References

- RP-160671, “Study on New Radio Access Technology”, NTT DoCoMo

- RP-170847, “New WID on New Radio Access Technology”, NTT DoCoMo

- TR 38.913, “Study on Scenarios and Requirements for Next Generation Access Technologies”, 3GPP

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #87, 3GPP

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting NR Ad-Hoc, 3GPP.

- RAN1 Chairman’s Notes, 3GPP TSG RAN WG1 Meeting #88, 3GPP

[7]  R1-1708502, “On the PDCCH search space configuration for NR”, Nokia, Alcatel-Lucent Shanghai Bell