3GPP TSG-RAN WG1#89	R1-1708531

Hangzhou, P.R. China, May 15-19, 2017

Agenda item:		7.1.3.3.6

Source:	Nokia, Alcatel-Lucent Shanghai Bell

Title:	On scheduling of mini-slots within slots

Document for:		Discussion and Decision

# 1	Introduction

The newly started work item on New Radio [1] states the following:

According to the outcome of the study item, the NR should specify the followings [TR38.912]:

-	Physical layer aspects including [RAN1]:

[…]

-	Physical layer channels for control and data based on associated waveform, numerologies and frame structure in line with the conclusions of the study item, including mini-slot design;

[…]

The TR38.912 [2] referred to in the above-quoted WID lists some design principles of the mini-slot

Mini-slots having the following lengths are defined.

-	At least above 6 GHz, mini-slot with length 1 symbol supported.

-	Lengths from 2 to slot length -1

-	For URLLC, at least 2 is supported

The following should be taken into account for designing slot-level channels/signals/procedures:

-	Possible occurrence of mini-slot/slot transmission(s) occupying resources scheduled for ongoing slot transmission(s) of a given carrier for the same/different UEs.

-	At least one of DM-RS format/structure/configuration for slot-level data channel is re-used for mini-slot-level data channel.

-	At least one of DL control channel format/structure/configuration for slot-level data scheduling is designed to be applicable to mini-slot-level data scheduling.

-	At least one of UL control channel format/structure/configuration for slot-level UCI feedback is designed to be applicable to mini-slot-level UCI feedback.

Note that some UEs targeting certain use cases may not support all mini-slot lengths and all starting positions. Mini-slot can start at any OFDM symbol, at least above 6 GHz. A mini-slot contains DM-RS at position(s) relative to the start of the mini-slot.

The following targets/use-cases to design mini-slots should be taken into account.

-	Support of very low latency including URLLC for certain slot lengths

-	Target slot lengths are at least 1ms, 0.5ms.

-	Support of finer TDM granularity of scheduling for the same/different UEs within a slot, especially if TRxP uses beam-sweeping (e.g., above 6GHz).

-	NR-LTE co-existence

-	Note that this use case also exists for slot-based scheduling

-	Forward compatibility towards unlicensed spectrum operation

The following should be taken into account as starting point for designing mini-slotlevel channels/signals/procedures:

-	Possible occurrence of mini-slot/slot transmission(s) occupying resources scheduled for ongoing slot transmission(s) of a given carrier for the same/different UEs

-	DM-RS for mini-slot-level data channel is just a re-use of that for slot-level data channel

-	DL control channel for mini-slot-level data scheduling is just a re-use of that for slot-level data scheduling

-	UL control channel for mini-slot-level UCI feedback is just a re-use of that for slot-level UCI feedback

-	Scheduling/HARQ timelines for a mini-slot can be based on scheduling/HARQ timelines for a slot

-	Scheduling/HARQ timelines for a mini-slot can be based on scheduling/HARQ timelines shorter than those for a slot

RAN1#87 (November 2016) made the following agreement:

Agreements:

- NR-PDCCH monitoring at least for single-stage DCI design,

  - NR supports the following minimum granularity of the DCI monitoring occasion:

    - For slots: once per slot

    - When mini-slots are used: FFS if every symbol or every second symbol

      - FFS with respect to which numerology if slot and mini-slot have different numerology (e.g. SCS, CP overhead)

      - Note: slot/mini-slot alignment is not assumed here

      - Note: This may not apply in all cases

RAN1#88bis (April 2017) made the following agreements and working assumptions:

Agreements

- UE can be configured to “monitor DL control channel” in terms of slot or OFDM symbol with respect to the numerology of the DL control channel

  - Specification supports occasion of “DL control channel monitoring” per 1 symbol with respect to the numerology of the DL control channel

    - Note: This may not be applied to all type of the UEs and/or use-cases

    - FFS whether or not total number of blind decodings in a slot when a UE is configured with “DL control channel monitoring” per symbol can exceed the total number of blind decodings in a slot when a UE is configured with “DL control channel monitoring” per slot

- Data channel (PDSCH, PUSCH) duration and starting position

  - Specification supports data channel having minimum duration of 1 OFDM symbol of the data and starting at any OFDM symbol to below-6GHz, in addition to above-6GHz

    - Note: This may not be applied to all type of UEs and/or use-cases

    - UE is not expected to blindly detect the presence of DMRS or PT-RS

    - FFS: Whether a 1 symbol data puncturing can be indicated by preemption indication

  - FFS: combinations of data duration and granularities of data position

  - Specification supports data having frequency-selective assignment with any data duration

- FFS: relations between “DL control channel monitoring” occasions and data channel durations

- Note: this is addition to the agreements at RAN1#86.

- Note : 1-symbol case may be restricted depending on the BW.

Agreements:

- The duration of a data transmission in a data channel can be semi-statically configured and/or dynamically indicated in the PDCCH scheduling the data transmission

  - FFS: the starting/ending position of the data transmission

  - FFS: the indicated duration is the number of symbols

  - FFS: the indicated duration is the number of slots

  - FFS: the indicated duration is the numbers of symbols + slots

  - FFS: in case cross-slot scheduling is used

  - FFS: in case slot aggregation is used

  - FFS: rate-matching details

  - FFS: whether/how to specify UE behavior when the duration of a data transmission in a data channel for the UE is unknown

Working assumption:

- The structure of a data channel (PDSCH and PUSCH (if applicable)) including DMRS is independent from whether it is scheduled on resources that are preempting or not preempting another users data transmission.

# 2	Discussion

## 2.1	Generic principles of scheduling slots and mini-slots

During the SI phase, there were some discussions on how the mini-slot should align to slot-based transmissions and whether the mini-slot may cross slot boundaries. Considering the SI agreements related to subframes, slots and mini-slots, this discussion may not be relevant to the mini-slot design any longer.

- The mini-slot transmissions exist independent of the slot transmissions, and should not be linked to imaginary slot transmissions of possibly different UEs

- The scheduling intervals for different UEs may differ due to potentially different slot lengths, and slot aggregation

- The common ‘tick’ for all UEs in a given cell is the subframe

Keeping in mind the agreements of RAN1#88bis,

- UE can be configured to “monitor DL control channel” in terms of slot or OFDM symbol with respect to the numerology of the DL control channel

- Specification supports occasion of “DL control channel monitoring” per 1 symbol with respect to the numerology of the DL control channel.

The following observations and proposal are made:

Proposal #1: Confirm the RAN1#88bis working assumption: The structure of a data channel (PDSCH and PUSCH (if applicable)) including DMRS is independent from whether it is scheduled on resources that are preempting or not preempting another user’s data transmission

Proposal #2: The PDCCH monitoring occasions, both in terms of slots and in terms of OFDM symbols, are defined relative to the subframe boundary

Observation: The PDCCH monitoring occasions in terms of slot can be seen to refer to slot-based scheduling, and the PDCCH monitoring occasions in terms of OFDM symbols can be seen to refer to mini-slot based scheduling

In addition, we target an L1 design that is agnostic to the used sub-carrier spacing, and specifically strive for the slot and mini-slot being flavours of the same base design as much as possible. In order to maintain generic slot/mini-slot design, also the mini-slot structure and duration should be agnostic to the starting position. Additional conditions, e.g. that a mini-slot cannot be scheduled to cross a sub-frame boundaries can be further considered, but this would be a scheduling restriction rather than mini-slot design-impacting one.

Proposal #3: The basic slot and mini-slot design, such as length location of control and length (in terms of symbols) is agnostic to sub-carrier spacing

Proposal #4: The designed mini-slot structure and possible lengths are the same regardless of the starting symbol of the mini-slot.

Observation: The design principles above make it possible for the gNB scheduler to avoid mini-slot crossing the subframe boundaries, or slot boundaries of other UEs’ slot-based transmissions.

## 2.2	Mini-slot support in TDD

Link direction switching in TDD mode can be facilitated by means of bi-directional slots. Half-duplex constraint will set boundary conditions for mini-slot allocations in TDD. This means that when a regular slot is in DL phase, also mini-slot needs be in the DL phase, and when regular slot is in UL phase, mini-slot needs to be in UL phase, respectively. This principle is shown in Figure 1. Based on that, there seems to be no need for bi-directional mini-slot containing both DL and UL. This approach will also maximize the commonalities for mini-slot design between TDD and FDD.

In addition, the duration of mini-slots would be fairly short. Supporting DL-UL switching within a mini-slot would result in additional non-negligible overhead and could lead to unreasonable DL-UL switching rate requirements for the UE.

Proposal #5: Mini-slot supports only one link direction.

Figure 1. Mini-slot operation following half-duplex constraint.

# 3	Conclusions

In this contribution we have discussed the slot and mini-slot design aspects in the new radio. Based on the discussion, we make the following proposals and observations:

Proposal #1: Confirm the RAN1#88bis working assumption: The structure of a data channel (PDSCH and PUSCH (if applicable)) including DMRS is independent from whether it is scheduled on resources that are preempting or not preempting another user’s data transmission

Proposal #2: The PDCCH monitoring occasions, both in terms of slots and in terms of OFDM symbols, are defined relative to the subframe boundary

Observation: The PDCCH monitoring occasions in terms of slot can be seen to refer to slot-based scheduling, and the PDCCH monitoring occasions in terms of OFDM symbols can be seen to refer to mini-slot based scheduling

Proposal #3: The basic slot and mini-slot design, such as length location of control and length (in terms of symbols) is agnostic to sub-carrier spacing

Proposal #4: The designed mini-slot structure and possible lengths are the same regardless of the starting symbol of the mini-slot.

Observation: The design principles above make it possible for the gNB scheduler to avoid mini-slot crossing the subframe boundaries, or slot boundaries of other UEs’ slot-based transmissions.

Proposal #5: Mini-slot supports only one link direction.

# References

- RP-170855, “New WID on New Radio Access Technology”, NTT DoCoMo

- 3GPP TR38.912, “Study on New Radio (NR) Access Technology” V1.0.0

- R1-1705465, “Mini-slot design considerations”, ZTE, ZTE Microelectronics, Ericsson, Nokia, Qualcomm