3GPP TSG RAN WG1 Meeting #89	 R1-1708547

Hangzhou, China, 15-19 May 2017

Agenda Item:	5.2.3

Source:	Huawei, HiSilicon

Title:	TP on impact on network implementation

Document for:	Discussion and decision

# Introduction

This text proposal is to provide a short discussion on the impact on network implementation for the RAN WGs on the study item, a simplified HS-SCCH for UMTS [1].

# Text proposal

------------------------------------------------------------- Text start -----------------------------------------------------

# XX		Discussion on network implementation

In this Section we discuss potential impact to network implementation for the identified solution is discussed. UE capability reporting is needed to enable network to save power consumption on simplified HS-SCCH. According to the identified solution, UE would know in advance that only a sub-set of part 1’s original HS-SCCH type 1 table would be signaled. With this prior knowledge, part 1 HS-SCCH detection performance is improved. The triggering mechanism is based on legacy CQI reports. When the network receives a CQI below 6, network would choose the values in the original HS-SCCH table which belongs to the sub-set which is also “known in advance” at the UE. It is noted that UE reporting a CQI below 6 does not mean the network would use the sub-set of the table. UE may perform two blind detection attempts, assuming that the network may or may not use the sub-set to identify whether the sub-set is used by the network. According to the analysis above, from network perspective, the MCS selection procedure is not changed, and no impact to the MCS selection and HS-SCCH signaling mechanisms.

However, if network knows that UE may use advanced receiver to enhance HS-SCCH detection performance, it is good for the network to know that UE is capable of doing it. If network knows that the target UE receiving HS-SCCH is capable of the ‘known in advance’ reception, network can choose a smaller power on HS-SCCH when choosing the “known in advance” sub-set in the original table, saving downlink power resource. Otherwise, it is only possible to enhance HS-SCCH reliability, but not saving HS-SCCH transmit power. Hence, network knowing UE’s capability for advanced HS-SCCH part 1 reception can save HS-SCCH part 1 DL transmit power when the “known in advance” subset of the HS-SCCH part 1 table was selected.

For a UE capable of advanced HS-SCCH part 1 reception, it would automatically turn its advanced receiver on only when the UE is in poor coverage, e.g., soft handover region of a cell, as the simplified HS-SCCH solution is most helpful in poor coverage. When the UE is not within the SHO region, network always assumes that legacy receiver is used.

------------------------------------------------------------- Text end -----------------------------------------------------

# Conclusion

It is proposed:

Proposal: Agree and capture the text proposal on the TR for the study on simplified HS-SCCH for UMTS.

# References

- [1] RP-170725, Study on a simplified HS-SCCH for UMTS