3GPP TSG-RAN WG1 #89	R1-1708700

Hangzhou, China, 15th-19th May, 2017

Source:	Ericsson

Title:	Further details on measurement restriction configuration

Agenda Item:	7.1.2.3.6

Document for:	Discussion and Decision

# Introduction

In RAN1#86 bis, 87 and NR Ad Hoc, agreements related to measurement restrictions (MR) were made and here the relevant decisions are extracted and highlighted:

Agreements:

- For channel and interference measurement in NR,

  - For interference measurement, support at least one of following schemes:

    - Measurement subsets in both time and frequency domain

    - Interference measurement restriction in both time and frequency domain

    - […]

Agreements:

- NR supports CSI calculation based on one-shot measurement of interference.

- For one-shot interference measurement, study at least one of the following alternatives:

  - […]

Agreements:

- [..]

- At least the following configuration parameters are signaled via RRC at least for CSI acquisition:

  - N, M, and L – indicated either implicitly or explicitly

  - In each CSI reporting setting, at least: reported CSI parameter(s), CSI Type (I or II) if reported, codebook configuration including codebook subset restriction, time-domain behavior, frequency granularity for CQI and PMI, measurement restriction configurations

  - [..]

- At least following are dynamically selected by L1 or L2 signaling, if appilicable

  - One or multiple CSI reporting settings within the CSI measurement setting

  - One or multiple CSI-RS resource sets selected from at least one Resource setting

  - One or multiple CSI-RS resources selected from at least one CSI-RS resource set

- FFS until the next meeting about details of dynamic triggering

To summarize the current situation in RAN1, we have agreed to have interference MR for both in time and frequency domain while channel MR is not discussed yet. Another open issue is whether interference MR use subsets, referring to subframe subsets which are used for CSI measurements in LTE. CSI calculation can be based on a single shot (i.e. a single slot) measurement, so in some sense time restriction to a single measurement of channel and interference is supported.

We agreed that the UE can be configured with N≥1  CSI reporting setting and each reporting setting contains a configuration of MR. Moreover, the CSI reporting settings can be dynamically selected by L1 or L2 signalling.

In this contribution, we discuss further around the open issues of measurement restrictions in NR.

# Interference Measurement Restriction

Interference measurement restriction (MR) in LTE is a useful tool, and is supported in the time dimension. For NR, we see value in the agreement to extend the notion of measurement restriction also to the frequency dimension. NR will support different services, potentially with different numerologies, in different parts of the band. For example, eMTC could be supported in one part of the band and eMBB in another part. If an eMBB UE is configured with a wideband interference measurement resource (IMR), MR in the frequency domain would allow the UE to measure only on that portion of the band occupied by eMBB traffic, thus increasing the accuracy of the interference measurement.

Hence partial band interference measurement should be supported in NR, where a partial band is smaller than the full bandwidth the UE can receive. This can be accomplished by a configuration of the interference measurement resource to cover a certain bandwidth:

Frequency domain interference measurement restriction configuration is achieved by an IMR definition that extends a configurable bandwidth that can be the full or a part of the receiver bandwidth the UE supports

In addition, one shot measurement of interference has been agreed which is interpreted as measurement using a single measurement resource in a single slot. Hence, time domain restriction is supported in NR,

Time domain interference measurement restriction configuration is achieved by configuring the IMR either to measure interference in a single slot (one shot) or an unrestricted number of slots (no restriction).

## Dynamics of interference measurement restrictions

Since NR support a flexible CSI framework using CSI reporting settings and Resource settings and configuration of links between these, plus the key principle of avoiding always on signals, the use of aperiodic CSI feedback will be a common tool. The flexible framework also allows pooling of resources and thus re-using CSI-RS for multiple UEs so as to save RS overhead.

Likewise, IMR can be re-used across UEs. However, the use of measurement restriction may lead to worse performance compared to unrestricted measurements in case the interference is stable and the UE can utilize this by averaging or other means. In some other cases, where interference is bursty, enabling interference MR could be beneficial. Also in the case of reusing IMR over time for measuring different CoMP hypotheses, then MR enabled for interference is beneficial.

For these reasons, it was agreed for NR to allow a somewhat more dynamic selection of interference measurement restriction, compared to RRC signalling, through the use of two CSI report settings where they are configured to have MR enabled and MR disabled respectively.

Figure 1 Dynamic selection of MR for interference using two CSI report settings

The network may want to get UE feedback of certain MU-MIMO hypotheses and thus would like to make a single shot measurement while under large packet data transfers spanning many slots using MU-MIMO to multiple UEs, the interference is stable and unrestricted measurement would improve CSI estimates as the UE could average IMR across multiple slots. In the example shown in Figure 1, periodic IMR resource is configured and when triggering an aperiodic CSI report using this IMR using DCI, either the MR enabled or disabled can be selected and if MR is enabled, only the latest (one shot) measurement should be used, i.e. the UE should flush the IMR buffer. This means that the UE stops accumulating and averaging interference measurements of a periodic IMR resource, and begins over again with an empty buffer to collect new measurements. Hence, Alt.2 from the agreement “Configuration of periodic/semi-persistent IMR with dynamic indication of interference measurement restriction.” should be one of the schemes supported for interference measurement.

Additionally, when the MR is switched to enabled from disabled and then back to disabled again, meaning no measurement restriction, it is useful if the UE restart the averaging of the interference. This allows the network to reset the averaging of interference in the UE, to avoid that measurements from a past interference (e.g. CoMP) hypothesis is included in the CSI report.

When MR is dynamically signaled to be disabled, the UE shall reset any memory of accumulated interference averaging across previous measurment slots.

This also has the consequence that the UE need not maintain two interference measurements (for disabled and enabled MR), which simplifies UE implementation.

# Channel Measurement restrictions

Channel measurement restriction in LTE is a useful tool, and we believe that it is useful that this is carried over to NR as well. For example, if a UE is configured to measure on a semi-persistent/periodic CSI-RS, and UE specific beamforming of CSI-RS is employed, then it is important to have channel measurement restriction in place to ensure accurate CSI. The UE must be instructed not to average over multiple CSI-RS transmissions, since the beamforming weights in different slots correspond to different users. Channel measurement restriction in the frequency domain is also beneficial, for the same reasons described in the previous section regarding different services operating in different parts of the band.

Although aperiodic CSI-RS and CSI reporting is supported already, it could be clarified that a single slot channel measurement is supported and can be extended also to periodic and semi-persistent CSI-RS. Hence, the proposal is:

Support channel measurement restriction to a single slot

Frequency domain channel measurement restriction configuration is achieved by a CSI-RS definition that extends a configurable bandwidth that can be the full or a part of the receiver bandwidth the UE supports

In addition, one shot measurement of interference has been agreed which is interpreted as measurement using a single measurement resource in a single slot. Hence, time domain restriction is supported in NR for measurement on periodic or semi-persistent signals,

Time domain channel measurement restriction configuration for periodic and semi-persistently configured CSI-RS is achieved by configuring channel MR in the CSI report setting to either be one slot or unrestricted.

# Measurement subsets

In NR, there has been significant discussion on the need for flexible configuration of CSI-RS since it must support multiple use cases. In companion contributions [1]-[2], we discuss how this flexibility may be achieved through a pooling of CSI-RS resources which are allocated/de-allocated to users on a dynamic basis. This can support both aperiodic and semi-persistent CSI reporting. With the introduction of such flexible and dynamic configurations, we currently do not see a use case where measurement subsets in time would be needed. If down selection between measurement restriction and measurement subsets is required (as implied by the RAN1 agreement), then our preference is that MR is supported as stated in Proposal 2 at least with respect to measurement slot subsets.

No current use case for time domain measurement subsets in NR

There may however be a use case for frequency domain measurement subsets in the CSI measurement bandwidth assigned to the UE (which may be the full or partial bandwidth the UE support). This can be used to increase the CSI resource capacity, especially in flat channels where a sparser frequency domain sampling of the channel can be useful to lower the overhead and/or increase the re-use distance of a CSI-RS resource.

Figure 2 Measurement subsets in frequency domain.

This can be achieved through CSI measurement subset, i.e. CSI measurements are performed on a subset of PRBs that is indicated to a UE.

Support measurement subsets in the frequency domain (across PRB). Do not support measurement subset in time domain (across slots).

# Conclusions

Based on the discussion in this contribution we propose the following:

Proposal 1	Frequency domain interference measurement restriction configuration is achieved by an IMR definition that extends a configurable bandwidth that can be the full or a part of the receiver bandwidth the UE supports

Proposal 2	Time domain interference measurement restriction configuration is achieved by configuring the IMR either to measure interference in a single slot (one shot) or an unrestricted number of slots (no restriction).

Proposal 3	When MR is dynamically signaled to be disabled, the UE shall reset any memory of accumulated interference averaging across previous measurment slots.

Proposal 4	Support channel measurement restriction to a single slot

Proposal 5	Frequency domain channel measurement restriction configuration is achieved by a CSI-RS definition that extends a configurable bandwidth that can be the full or a part of the receiver bandwidth the UE supports

Proposal 6	Time domain channel measurement restriction configuration for periodic and semi-persistently configured CSI-RS is achieved by configuring channel MR in the CSI report setting to either be one slot or unrestricted.

Proposal 7	Support measurement subsets in the frequency domain (across PRB). Do not support measurement subset in time domain (across slots).

# References

R1-1705895, “Further Details of CSI Framework”, Ericsson

R1-1705919, “On Dynamic Triggering for CSI Reports and CSI-RS”, Ericsson