3GPP TSG-RAN WG1 #89	R1-1708706

Hangzhou, China, 15-19 May 2017

Source:	Ericsson

Title:	Evaluation Results for UL DMRS

Agenda Item:	7.1.2.4.2

Document for:	Discussion and Decision

# Introduction

In RAN1#88 [1], the following agreement was made:

- Number of orthogonal ports for DL MU-MIMO

  - Maximum number of orthogonal ports supported by front loaded DMRS

- Port multiplexing in front-load DMRS

  - IFDM structure (comb) with frequency CDM

  - OCC in frequency in adjacent REs

- DMRS for high Doppler scenarios:

  - Additional DMRS with the same density compared to front-loaded DMRS

  - Additional DMRS with a lower density compared to front-loaded DMRS

- Number of ports in front-load DMRS

Regarding the number of ports in the 1-symbol front-load DMRS

- Collision of DC subcarrier and DMRS

In this contribution, we focus on the following evaluations for UL DMRS patterns:

- Performance comparison of DMRS patterns with frequency domain CDM (F-CDM) across adjacent and non-adjacent REs

- Performance comparison of single-symbol and double-symbol front loaded DMRS for UL MIMO transmission

# DMRS link level evaluations: Comparison of F-CDM on adjacent and non-adjacent REs

In this section, we focus on the UL link level evaluations of DMRS patterns, specifically, of two kinds:

- Frequency domain CDM at regular non-adjacent combs, i.e., discontinuous F-CDM pattern

- Frequency domain CDM at adjacent sub-carriers, i.e., continuous F-CDM pattern

Those two types of DMRS patterns are illustrated in the following figure:

Figure 1 DMRS patterns considered for link-level evaluations

Figure 1 illustrates two patterns supporting up to 4 orthogonal DMRS ports which we will evaluate in this section. F-CDM is applied at the resource elements with the same colour.

- Pattern-1: FDM factor 2 with regular comb, and F-CDM with length 2 is applied to every 2 combs (non-adjacent REs) with the same colour.

- Pattern-2: FDM factor 2 with irregular comb, and F-CDM with length 2 is applied to every adjacent 2 resource elements with the same colour.

By differing from the frequency comb structure and F-CDM group allocations, there are following considerations which are triggered by the above two different types of DMRS patterns:

- Pattern-1 will be more flexible in type of receiver that can be used since it allows for time domain as well as frequency domain operations at the receiver side. This is because of the regular sampling in the frequency domain, such that the sampled signal can be easily transformed into the time domain. Therefore, Pattern-1 enables the additional degrees of freedom for channel estimation in the time domain, timing and delay spread estimation in the time domain, and frequency domain filter tuning, etc. Moreover, DMRS pattern with F-CDM on adjacent REs have better CM/PAPR property, see [2] which is important for UL transmission.

- Pattern-2 utilize the advantages of the continuous F-CDM group, but on the other hand increases the interpolation distance across different F-CDM groups. Therefore, it is difficult to judge by inspection whether it performs better in terms of channel estimation compared to Pattern-1. Moreover, because of the irregular sampling in the frequency domain, it will bring more complexity for carrying out time domain operations.

Another aspect is that there may exist many antennas at the gNB in NR for receiving and processing the UL transmissions. The performance difference of pattern 1 and 2 will likely be negligible in the case of large number of receiver antenna processing.

The DMRS pattern with F-CDM on regular combs provides more flexibility in receiver implementation as it allows for time domain operations at the receiver side.

The DMRS pattern with F-CDM on regular combs provides CM/PAPR benefits for UL transmission.

In the evaluations, we mainly target on low Doppler scenarios. We first compare two patterns based frequency domain LMMSE channel estimator.

## DMRS evaluations for UL transmission

In this section, we provide simulation results based on two patterns for UL 4-layer transmission with frequency LMMSE channel estimator. UE speed is 3km/h. The RMS delay spread 30ns, 100ns, and 300ns are investigated with the channel model CDL-A. MCS (QPSK, 0.5), (QAM16, 0.75), and (QAM64, 5/6) are used in transmissions. Detailed simulation assumptions are listed in Appendix 6.1.

Figure 2 BLER for 4-layer UL transmission with DMRS pattern-1 and 2. CDL-A channel model, Ds=30ns, Tx antennas 4, Rx antennas 16

Figure 3 BLER for 4-layer UL transmission with DMRS pattern-1 and 2. CDL-A channel model, Ds=100ns, Tx antennas 4, Rx antennas 16

Figure 4 BLER comparison of UL 4-layer transmission using DMRS pattern-1 and 2. CDL-A channel model, Ds=300ns, Tx antenna 4, Rx antenna 16

Figure 2-4 show the BLER performance of UL 4-layer transmissions with DMRS pattern-1 and 2.

The DMRS pattern 1 with F-CDM on regular combs is robust to highly frequency-selective channels for UL transmission.

For delay spread from 30ns to 300ns, pattern 2 with consecutive F-CDM and pattern 2 with non-adjacent F-CDM provide similar BLER performance when there are a large number of receive antennas at the gNB.

## UL transmission with PA impairments

In this section, we provide the throughput comparison of two studied patterns under transmitter side PA impairments. It has been stated in [2] that the IFDMA based pattern has advantages in CM/PAPR property. Therefore, the PA will have less power back-off for pattern-1 while transmission compared with pattern-2. The following simulation results show the throughput comparison with link adaptation. The detailed simulation parameters are in Appendix 6.2.

Figure 5 Throughput comparison of two studied patterns under transmitter PA impairment

Figure 6 Throughput comparison of two studied patterns under transmitter PA impairment

The IFDMA pattern (pattern 1) provides higher throughput than the pattern with CDM on consecutive REs (pattern 2) when considering PA impairment at the transmitter.

Combining observations 1-5, we propose:

NR support DMRS patterns across regular combs (non-adjacent REs) for uplink.

NR support F-CDM for DMRS in uplink

# DMRS link level evaluations: Single-symbol and double-symbol front loaded DMRS

In this section, we will evaluate the following four DMRS patterns for 4 layers uplink transmission in an scenario with very low Doppler (3 km/h):

- Pattern-1: single-symbol front-loaded at symbol 3 and FDM4.

- Pattern-2: two-symbol front-loaded at symbol 3 and 4, FDM2, time CDM2 and frequency CDM2 for 8 layers

- Pattern-3: LTE DL-resembling pattern, FDM2 and time CDM2. The only difference of this pattern with the LTE-legacy downlink pattern is the regular comb structure within one PRB.

- Pattern-4: two-symbol front-loaded at symbol 3 and 4, FDM2 and time CDM2.

Figure 5 DMRS patterns for UL evaluations

## Simulation results

Figure 6-8 show the simulation results for the throughput of 4-layer UL transmission using DMRS patterns 1-4 with link adaptation. The TDL-A/C channel are assumed with 10ns, 300ns, and 1000ns delay spread for simulations. The number of Tx and Rx antenna are 4 and 16, respectively. The sub-carrier spacing is 15kHz. The detailed simulation parameters can be found in Appendix 6.3.

Figure 6 Throughput performance comparison of DMRS patterns 1-4, TDL-A, Ds = 10ns

Figure 7 Throughput comparison of DMRS patterns 1-4. TDL-C, Ds = 300ns

Figure 8 Throughput comparison of DMRS patterns 1-4. TDL-C, Ds = 1000ns

For low delay spread channels, single-symbol front loaded DMRS provides better throughput than double-symbol front loaded DMRS.

For large delay spread channels, single-symbol and double-symbol front loaded DMRS provide similar throughput performance.

In the low SNR region, single-symbol front-loaded DMRS provides slightly better throughput than double-symbol front-loaded DMRS.

Based on the above evaluations, we propose the following:

Single-symbol front-loaded DMRS is supported for NR UL

# Conclusions

Based on the discussion in this contribution we propose the following:

Proposal 1	NR support DMRS patterns across regular combs (non-adjacent REs) for uplink.

Proposal 2	NR support F-CDM for DMRS in uplink

Proposal 3	Single-symbol front-loaded DMRS is supported for NR UL

# References

Chairman’s notes RAN-1_88 final, 3GPP

R1-1708705, On UL DMRS design, Ericsson

# Appendix

## Simulation Parameters:

| Parameter | Value |
| --- | --- |
| Channel Model | CDL-A |
| Numerology | 15KHz |
| Carrier frequency | 4GHz |
| Transmission Slot Length | 14 symbols, with first 2 symbols reserved for PDCCH |
| Transmission mode | FDD |
| Number of UE | 1 |
| UE speed | 3 km/h |
| Delay spread | 30ns, 100ns, 300ns |
| Allocated bandwidth | 8 PRBs |
| Link Adaptation | Disabled |
| Antenna configuration | 4Tx, 16Rx |
| MIMO Layers | 4 |
| Channel estimator | Practical LMMSE |
| PRB bundled size | 4 PRBs |

## Simulation Parameters:

| Parameter | Value |
| --- | --- |
| Channel Model | CDL-A |
| Numerology | 15KHz |
| Carrier frequency | 4GHz |
| Transmission Slot Length | 14 symbols, with first 2 symbols reserved for PDCCH |
| Transmission mode | FDD |
| Number of UE | 1 |
| UE speed | 3km/h |
| Delay spread | 300ns, 1000ns |
| Allocated bandwidth | 8 PRBs |
| Link Adaptation | Enable |
| Antenna configuration | 8Tx, 8Rx |
| MIMO Layers | 4 |
| Channel estimator | LMMSE |
| PRB bundled size | 4 PRBs |
| Noise Covariance Estimator | Ideal |
| PA impairment model | Practical IEEE model |

## Simulation Parameters:

| Parameter | Value |
| --- | --- |
| Channel Model | TDL-A, TDL-C |
| Numerology | 15KHz |
| Carrier frequency | 4GHz |
| Transmission Slot Length | 14 symbols, with first 2 symbols reserved for PDCCH |
| Transmission mode | FDD |
| Number of UE | 1 |
| UE speed | 3km/h |
| Delay spread | 10ns, 300ns, 1000ns |
| Allocated bandwidth | 8 PRBs |
| Link Adaptation | Enable |
| Antenna configuration | 4Tx, 16Rx |
| MIMO Layers | 4 |
| Channel estimator | LMMSE |
| PRB bundled size | 4 PRBs |