3GPP TSG RAN WG1 Meeting #89                                           	R1-1708796

Hangzhou, China, 15th – 19th May 2017

Agenda item:    6.2.6.2

Source:             Qualcomm Incorporated

Title:                  Efficient monitoring of DL control channels

Document for:  Discussion and Decision

- Introduction

In RAN#75 a new work item (WI) named even further enhanced MTC for LTE [1] was introduced. The objectives of the WI is to study and, if found beneficial for idle mode paging and/or connected mode DRX, specify physical signal/channel that can be efficiently decoded or detected prior to decoding the physical downlink control/data channel.

In RAN1 #86bis the following was agreed:

Techniques to be evaluated:

  - Wake-up signal/channel (either relying or not relying on DL synchronization)

  - Go-to-sleep signal/channel (either relying or not relying on DL synchronization)

  - Compact DCI

  - Reduced-bandwidth MPDCCH

  - Dynamic USS periodicity

Additionally, the reference scenarios, power models, and simulation assumptions to be used were also agreed in [5].

In this contribution, we provide our views on introducing a new physical channel (“wake up signal”) that UE can monitor prior to monitoring MPDCCH which informs UE whether to monitor MPDCCH or not. We cover use cases, analysis of power savings, as well as some PHY/MAC design aspects of the wake-up signal. In this contribution, we focus on the paging use case. However, the techniques do apply to connected mode DRX as well.

- Wake-up Receiver Architecture

Figure 1: Typical idle mode processing (no paging case)

Typical processing at UE when it is in idle mode is shown in Fig. 1. The UE monitors for page in its paging occasions which are configured periodically with a period equal to the DRX cycle. The UE is in a sleep mode prior to its paging occasion. It wakes up slightly earlier than its paging occasion to obtain timing/frequency sync and reconfirm the serving cell. Once it has timing/frequency sync, it goes on to monitor the MPDCCH. If there is no page (which is the case in Fig. 1), the UE then goes back to sleep until the next paging occasion. The amount of time UE is awake includes the time for sync, time for monitoring MPDCCH, as well as some warm up and ramp down time. Since the MPDCCH payload contains several bits, the time to decode MPDCCH can span several 10s of subframes.

The awake time of the UE can be substantially reduced by sending, for example, a “1-bit” wake up signal that lets the UE know whether it should monitor the MPDCCH or not. As shown in Fig. 2, if UE doesn’t detect the wake-up signal (or it decodes it and the wake-up signal indicates no MPDCCH present), it does not monitor the MPDCCH. Since the length of the wake-up signal is expected to be much smaller than that of MPDCCH (as it effectively conveys much fewer bits) the UE awake time is significantly reduced and hence the corresponding power consumption.

Figure 2 : Idle mode processing with wake-up signal (no paging case)

Fig. 3 shows the processing when a page is sent to the UE. The UE is expected to detect / decodes the wake-up signal, go on to monitor MPDCCH, and then associated PDSCH.

Figure 3 : Idle mode processing with wake-up signal when UE is paged

The wake-up signal also enables use of much more power efficient HW architectures. One example is shown in Fig. 4. MPDCCH monitoring involves complex basedband processing but detection of wake-up signal may be possible by new low power wake up receiver (e.g. that maybe does just correlations). The wake-up receiver runs only to detect the wake-up signal. The full baseband modem is turned on only when the wake-up signal is detected. This enables extracting even more gains than just those obtained by reducing the awake time. Note that the wake-up receiver and baseband modem block-level partitioning shown in the Fig. 4 is conceptual and there are a number of ways to realize in actual HW implementation, including operating WUS detection as a low power mode functionality of the modem.

Figure 4 : Wake up receiver architecture

- Use Cases and Power Analysis

  - Use Case

The wake-up signal / receiver is primarily targeted towards MT data traffic use case where page monitoring power is a dominant source of power consumption. The following was agreed in [5] for the reference use case – “The reference scenario is a UE that monitors the MPDCCH transmission, but is not receiving a PDSCH. This includes both idle and connected mode. In idle mode a page for another UE is present 0% and x% of the cases.” We focus on the 0% case for now as we believe x can be made fairly small by configuring sufficient number of wake-up signal resources.

The agreements in [5] however didn’t cover the DRX cycles for which the power savings have to be studied. The wearables use case from [2] can probably be considered to study the potential benefits of the wake-up signal. The details for this use case is summarized in Table 1. We however assume no UL data transfers as well in line with the agreements in [5]. Since only a small amount of data is sent once every 2 hours, we don’t expect the trend in the results to change significantly due to this simplification.

Table 1: Use Case

| Use case | Wearable [2] |
| --- | --- |
| Traffic Characteristic Description | MT 100 bytes + 20 byte ACK. 1 message per 2 hours |
| Latency (paging cycle) | 5 seconds |
| MCL | 144 dB |
| Battery life | Coin Cell : 1 year Rechargeable : 2 weeks |

  - Power Models

The power models and simulation assumptions to be used for analysing the power savings were also agreed in [5] and are shown in Table 2 and Table 3 below. Note that these models are used to obtain a rough estimate of power savings and may not represent the actual power consumption/savings.

Table 2: Power Model

| Operating mode | Power [units/ms] | Notes |
| --- | --- | --- |
| Receive | 100 | RF and baseband circuitry |
| Light sleep | 1 | Corresponds to maintaining accurate timing by keeping RF frequency reference active. |
| Idle, deep sleep | 0.015 | Deep sleep during PSM and eDRX |
| Transitions between states | To be declared (We assume 50) | Boot, reload memory etc. See below illustration. |

Table 3: Simulation Assumptions

| Parameter | Value |
| --- | --- |
| BS TX antenna configuration | 2 Tx |
| BS power | 43 dBm per TX port |
| System BW | 10 MHz |
| Band | Band 8 (900 MHz) |
| Channel model | ETU |
| Doppler spread | 1 Hz |
| Time/frequency drift, in idle mode when not relying on DL synchronization | 0.05 ppm/s |
| Maximum frequency error, in idle mode when not relying on DL synchronization | ±5 ppm |
| Frequency error, when relying on DL synchronization | ±30 Hz |
| UE RX antenna configuration | 1 Rx |
| UE NF | 9 dB |
| Coupling loss | 144, 154, 164 dB |

Note that in the power model the light sleep mode power and the standby/deep sleep mode power are substantially different. The wake-up time from the stand-by/deep sleep mode to active mode can be very significant (e.g. in [4] it is assumed to be 150ms) and hence UE is expected to move to deep sleep mode only when the savings in sleep power is more than the power spent on wake-up every DRX cycle. For example, assuming 150ms warm up time, UE would use deep sleep mode only when the sleep duration T ms is such that T x difference in sleep power (1-0.015) units is larger than 150ms (warm up time) x warm up power (100/2) units (assuming warm up power is half of active power), i.e., T >= 7.5 seconds.

We assume a total time of 20ms for warm up and ramp down from light sleep to receive and study both 200ms and 2 second switching time for deep sleep to receive.

  - Power Analysis

In this section, we analyze the power savings with a wake-up signal assuming a simplistic model of UE only monitoring paging but not receiving any page. The savings are studied as a function of different MPDCCH Rmax and DRX cycles. The other assumptions made are as follows. We assume that the sync duration is 10ms. For the wake-up signal we assume the length is equal to length of time needed for SYNC+ Rmax/16. The inherent assumption here is that the wake-up signal has only one bit but MPDCCH has 30+ bits payload including CRC and hence Rmax/16 number of subframes should be sufficient for the wake-up signal. The detailed assumptions are also summarized below:-

Table 4: Power Analysis Parameters

| Parameter | Value |
| --- | --- |
| Rmax | [8,16,32,64,128,256] |
| DRX | 128,256,512,1024 frames |
| SYNC duration (after light sleep) | 10ms |
| SYNC duration (after deep sleep) | 500ms |
| Wake up signal length | SYNC duration + Rmax/16 |
| FA rate for WU signal detection | 2% |

The resulting power savings computed due to savings in awake time is shown in the Fig. 5. The results are assuming the sleep mode is light sleep mode (i.e. not the deep sleep/standby mode). The savings of course will be higher (50% or more across all cases in the plot) if we can assume UE can go to deep sleep mode without significant switching time.

Figure 5: Power savings with wake-up signal due to reduced awake time

The assumptions in Table 3 are used to determine the Rmax of interest. In 36.101, MPDCCH with repetition of 64 (with frequency hopping) for ETU1Hz is expected to be decoded at -12 dB SNR. The repetition levels in the table below are extrapolated based on that and assuming 2.5dB gain per doubling of repetition level.

Table 5: Mapping of MCL to SNR and Rmax

| MCL (dB) | SNR (dB) | Required Rmax (assuming ETU 1Hz) |
| --- | --- | --- |
| 144 | -3 | 4-8 |
| 154 | -13 | 64-128 |
| 164 | -23 | 1024-2048 (max Rmax is 256 though) |

Hence for the 5 second DRX cycle case at MCL of 154 dB we see savings of 40-60%. The power distribution and savings with wake up signalling for Rmax = 64 and 5 second DRX cycle is shown in the figure below. We should also note that for the lower MCL, although the required Rmax is small for scheduling flexibility the eNB may configure larger Rmax than the minimum required Rmax and hence the power savings there could also be substantial.

Figure 6 : Power breakdown with and without use of wake-up signal (Rmax = 64, DRX cycle = 5sec)

Observation 1: In the page monitoring dominated use cases, wake up receiver signalling provides gains of 40-60% for the 5 second DRX cycle case at MCL of 154 dB.

    - Power consumption as function of Rmax

In Fig. 7 we plot the power consumed for a given DRX cycle as function of Rmax. As can be seen, since the wake-up signal is much smaller in length than the MPDCCH, the impact of Rmax on the power consumption is significantly reduced. Hence, use case such as wearables can be enabled for deeper coverage without significantly impacting the overall power consumption and battery life of the device.

Figure 7: Idle mode power consumption as function of Rmax

Observation 2: Use of wake-up signal makes the power consumption less sensitive to MCL enabling the use cases to operate at higher MCL without significantly impacting the battery life.

    - Power savings with eDRX

The percentage power savings in eDRX mode depends on the switching time between deep sleep mode to sleep mode which depend on the HW architecture. In the following figures we show the power savings and power breakup for switching time of 200ms and 2 seconds as function of the number of PDCCH search spaces within one eDRX cycle (i.e. as function of PTW).

Figure 8 : Power savings with eDRX cycle of ~30mins as function of PTW

Figure 9 : Power breakup with eDRX cycle of ~30mins assuming deep sleep to receive switch time of 200ms

Figure 10 : Power breakup with eDRX cycle of ~30mins assuming deep sleep to receive switch time of 2 secs.

The percentage savings when UE is configured in eDRX mode are smaller but still quite significant. For example, depending on the assumptions on the switching time, the savings for Rmax of 64 and PTW of 4 are between 10 to 20%.

Observation 3: In the page monitoring dominated use cases, wake up receiver signalling provides gains of 10-20% in eDRX mode with eDRX cycle of 30 mins and PTW of 4 at MCL of 154 dB.

Based on observation 1, 2, and 3 we do see clear benefits of introducing wake up signal. We hence make the following proposal.

Proposal 1: A new physical channel that carries the wake-up signal is introduced for eFeMTC. If the wake-up signal is configured for the UE in idle mode/connected mode DRX, the UE is required to decode the control channel only when the wake-up signal is detected.

- Wake-up signal resource configuration

In this section, we present our views on configuration of the wake-up signal resource (WUSR) and mapping to UE groups/MPDCCH search space to it. The proposed operation is shown in Fig. 11. We propose that a group of WUS resource occasions is configured with a certain periodicity. In each occasion, there are one or more WUS resources configured. UE determines the WUS resource to monitor based on UE-ID/RNTI/Rmax etc. The WUS indicates the presence or absence of MPDCCH for a group of subframes. For example, in Fig. 11, the WUS resources shown in a particular colour indicate the presence / absence of MPDCCH on subframes of the same colour.

The motivation for having multiple WUS resources per occasion is as follows. If the paging probability system wide itself is very low (e.g. only 1 occasion out of 10 have any paging present), we can just configure 1 WUS resource in each occasion. All UEs would wake up in the occasion if any UE got paged. However, since this is infrequent UE would still save power. On the other hand, if every occasion has at least one UE being paged, with just 1 WUS resource in each occasion, all UEs will be on all the time – no power saving (actually extra power consumed for WUS resource monitoring). It is hence important to split UEs into different WUS resources.

Figure 11: Configuration of wake-up signal resource

Proposal 2: Wake up signal resource occasions are configured with certain periodicity. In each occasion, multiple wake up signal resources can be configured which all correspond to the same set of subframes. Different UEs may monitor different wake up signal resources within the same occasion.

- Summary

The observations and proposals made in this contribution are summarized below.

Observation 1: In the page monitoring dominated use cases, wake up receiver signalling provides gains of 40-60% for the 5 second DRX cycle case at MCL of 154 dB.

Observation 2: Use of wake-up signal makes the power consumption less sensitive to MCL enabling the use cases to operate at higher MCL without significantly impacting the battery life.

Observation 3: In the page monitoring dominated use cases, wake up receiver signalling provides gains of 10-20% in eDRX mode with eDRX cycle of 30 mins and PTW of 4 at MCL of 154 dB.

Proposal 1: A new physical channel that carries the wake-up signal is introduced for eFeMTC. If the wake-up signal is configured for the UE in idle mode/connected mode DRX, the UE is required to decode the control channel only when the wake-up signal is detected.

Proposal 2: Wake up signal resource occasions are configured with certain periodicity. In each occasion, multiple wake up signal resources can be configured which all correspond to the same set of subframes. Different UEs may monitor different wake up signal resources within the same occasion.

References

[1] RP-170732, “New WID on Even further enhanced MTC for LTE”, Ericsson, Qualcomm, 3GPP TSG RAN Meeting #75, Dubrovnik, Croatia, March 6 - 9, 2017.

[2] R1-168485, “Battery Lifetime Model for NR”, Sony,  u-blox, Sierra Wireless, 3GPP TSG RAN WG1 Meeting #86, Gothenburg, Sweden 22nd - 26th August 2016.

[3] 3GPP Technical Report 45.820, “Cellular system support for ultra-low complexity and low throughput Internet of Things (CIoT)”, December 2015.

[4] R2-151142, “eDRX performance”, Ericsson, 3GPP TSG RAN WG2 #89bis, Bratislava, Slovakia, April 20-24, 2015

[5] R1-1706704, “Way forward on Downlink channel power efficiency”, Ericsson, Qualcomm, Orange, 3GPP RAN1#88bis, Spokane, WA, USA, Apr 3-7, 2017.