3GPP TSG RAN WG1 Meeting #89                                     R1-1708836

Hangzhou, P.R. China 15th – 19th May 2017

Agenda item:		7.1.4.3

Source:	Nokia, Alcatel-Lucent Shanghai Bell

Title:	Coding scheme for PBCH

Document for:		Discussion and Decision

# 1	Introduction

In RAN1 #88bis meeting, Ran1 agreed [1] that

Agreement:

- The primary candidates for PBCH channel coding are:

  - Polar control channel coding scheme, with Nmax <= 512, reusing same decoder

  - LDPC data channel coding scheme, reusing same decoder – i.e. no new shift network, but a new base graph may be considered

- LTE TBCC may also be considered if fundamental problems are unresolved with the above candidates

- Evaluate BLER and FAR performance until RAN1#89, with the following assumptions:

  - Implementable decoders, i.e.:

    - For polar decoding: Lmax = 8

    - For LDPC decoding: min-sum variants, flooding 50 iterations

  - Info + CRC = 40-100 bits

  - Target FAR is that achieved with CRC size = 16

  - Starting code rate <= 1/6

  - Performance to be compared based on a single transmission with no combining

    - Note that it is assumed that PBCH uses Chase combining – i.e. IR is not supported.

  - Decoder power may optionally also be considered

In this contribution, we compare the BLER performance of LDPC and Polar coding for PBCH when they are providing the same FAR.

# 2	Performance

## 2.1	Proposed LDPC codes

The LDPC codes we propose for PBCH are defined with the following PCM’s. The first code is for R = 1/6:

24 29  0 -1 -1 -1 -1 -1 -1 -1 -1

4 35  0  0 -1 -1 -1 -1 -1 -1 -1

27 -1 -1 32  0 -1 -1 -1 -1 -1 -1

33 -1 -1 10 -1  0 -1 -1 -1 -1 -1

-1 30 -1 34 -1 16  0 -1 -1 -1 -1

-1 -1 13 21 -1 -1 29  0 -1 -1 -1

3 32 -1 -1 -1 12 -1 -1  0 -1 -1

21 -1 22 -1 -1 -1 29 -1 -1  0 -1

-1 -1 -1 23 -1 -1 16 19 -1 -1  0

The second PCM is for R = 1/12:

24 29  0 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1

4 35  0  0 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1

-1 -1 -1 32  0 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1

33 -1 -1 10 -1  0 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1

-1 30 -1 34 -1 16  0 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1

-1 -1 13 -1 -1 -1 29  0 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1

3 -1 -1 -1 -1 12 -1 -1  0 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1

-1 -1 22 -1 -1 -1 29 -1 -1  0 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1

-1 -1 -1 23 -1 -1 16 -1 -1 -1  0 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1

-1 -1 -1 17 -1 -1  4 -1 -1 -1 -1  0 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1

-1 -1 -1 -1 -1 16 -1 20 -1 -1 -1 -1  0 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1

-1 -1 -1 -1 -1 -1 35 32 -1 -1 -1 -1 -1  0 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1

-1 -1 -1 -1 -1 19 -1  6 -1 -1 -1 -1 -1 -1  0 -1 -1 -1 -1 -1 -1 -1 -1 -1

21 -1 -1 -1 -1 26 -1 -1 -1 -1 -1 -1 -1 -1 -1  0 -1 -1 -1 -1 -1 -1 -1 -1

-1 -1 -1 10 -1 11 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1  0 -1 -1 -1 -1 -1 -1 -1

26 -1 -1 -1 -1 -1  7 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1  0 -1 -1 -1 -1 -1 -1

24 -1 -1 -1 -1 -1  8 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1  0 -1 -1 -1 -1 -1

-1 32 14 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1  0 -1 -1 -1 -1

31 -1 -1 -1 -1 29 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1  0 -1 -1 -1

20 -1 -1 -1 -1 -1 28 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1  0 -1 -1

21 -1 -1  5 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1  0 -1

15 33 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1 -1  0

Rate matching and modulo lifting are used to get the code for arbitrary number of info block sizes K. Optimized performance is obtained for 20 <= K <= 100. LDPC codes have inherent error check in the form of syndrome check, i.e. the for every code word v, it holds that Hv = 0. This alone does not provide sufficient protection against undetectable errors, but when used with CRC check, less bits can be allocated to CRC. As a result, effective coding rate is decreased, improving performance and coverage.

## 2.2	Simulation parameters

Based on the analysis on LDPC BLER and FAR performance, results are given in the Appendix, we obtain following simulation parameters, as in Table I, which are suitable to evaluate LDPC and Polar as the FARs are similar.

For polar codes, we used CA-Polar with 19 CRC bits and advanced repetition scheme as in [2] when coded block size exceeds 512. LTE like repetition is used for LDPC codes to get rate 1/18.

Table I: Simulation parameters

|  | LDPC (OMS with 50 iteration) |  |  |  | Polar (CA Polar) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Information bits (I) | CRC bits (C ) | K (I + C) | Code rate | Coded block (M) | CRC bits (C ) | K (I + C) | Coded block (M) | Mother codeword size (N) |
| 28 | 12 | 40 | 1/6 | 240 | 19 | 47 | 240 | 256 |
| 38 | 12 | 50 | 1/6 | 300 | 19 | 57 | 300 | 512 |
| 50 | 10 | 60 | 1/6 | 360 | 19 | 69 | 360 | 512 |
| 40 | 10 | 50 | 1/12 | 600 | 19 | 59 | 600 | 512 |
| 51 | 9 | 60 | 1/12 | 720 | 19 | 70 | 720 | 512 |
| 29 | 11 | 40 | 1/18 | 720 | 19 | 48 | 720 | 512 |
| 40 | 10 | 50 | 1/18 | 900 | 19 | 59 | 900 | 512 |
| 60 | 10 | 70 | 1/18 | 1260 | 19 | 79 | 1260 | 512 |
| 70 | 10 | 80 | 1/18 | 1440 | 19 | 89 | 1440 | 512 |

## 2.3	Comparison between Polar and LDPC

Performance results are illustrated in Figure 1-3.

Figure 1: BLER versus SNR for rate 1/6

Figure 2: BLER versus SNR for rate 1/12

Figure 3: BLER versus SNR for rate 1/18

Observation 1: CA-polar codes outperform LDPC codes for the considered block sizes and code rates.

# 3	Conclusion

In this contribution, we compared polar and LDPC codes. With the current LDPC constructions we have following observation,

Observation 1: Polar codes outperform LDPC codes for the considered block sizes and code rates.

# References

- RAN1 Chairman’s notes, RAN1 #88bis, Spokane, USA

- R1-1708835,	“Rate matching for Polar codes”, Nokia, ASB

# Appendix

LDPC rate matching parameters are given in Table II and evaluation results of LDPC BLER and FAR for different block sizes are given as Figure 4 and 5.

Table 2: Rate matching parameters.

|  | K | z | nShortBits | nParityPunct | nRepeatedBits | nSystPunct |
| --- | --- | --- | --- | --- | --- | --- |
| Rate 1/6 | 30 | 15 | 0 | 0 | 15 | 0 |
|  | 40 | 22 | 4 | 0 | 2 | 0 |
|  | 50 | 36 | 22 | 38 | 0 | 36 |
|  | 60 | 36 | 12 | 0 | 0 | 24 |
|  | 70 | 36 | 2 | 0 | 26 | 0 |
|  | 80 | 48 | 16 | 0 | 0 | 32 |
|  | 90 | 60 | 30 | 30 | 0 | 60 |
| Rate 1/12 | 30 | 15 | 0 | 0 | 0 | 0 |
|  | 40 | 22 | 4 | 22 | 0 | 22 |
|  | 50 | 28 | 6 | 38 | 0 | 28 |
|  | 60 | 36 | 12 | 96 | 0 | 36 |
|  | 70 | 36 | 2 | 0 | 0 | 22 |
|  | 80 | 48 | 16 | 128 | 0 | 48 |
|  | 90 | 48 | 6 | 18 | 0 | 48 |

(a)

(b)

Figure 4: LDPC BLER and FAR versus SNR for rate 1/6.

(a)

(b)

Figure 5: LDPC BLER and FAR versus SNR for rate 1/12.