3GPP TSG-RAN WG1 Meeting #89	R1-1708847

Hangzhou, P.R. China, 15th – 19th May 2017

Agenda Item:	6.2.1.2.5.1

Source:	Ericsson

Title:	Design aspects of sPUSCH

Document for:	Discussion, Decision

# Introduction

In RAN1#86bis the following was agreed among other agreements.

- For UL transmission for sTTI

  - TM1 and TM2 are supported

In RAN1#88, the following was agreed:

- For both 2-symbol and 1-slot sPUSCH transmission

  - Up to 2-layer transmission is supported.

  - FFS 4-layer is supported.

In this contribution, we discuss the number of codewords for TM2, the design of resource allocation and TBS determination for sPUSCH.

# Discussion

## Supported number of codewords for TM2

For legacy uplink transmission with 1ms TTI, up to four layers can be supported for TM2. And it is possible to use two codewords for TM2 when the number of layers is greater or equal to two. For two-codeword transmission case, the UE feedback (CQI and HARQ ACK/NACK) is doubled compared to the case of a single-codeword. The downlink control information (DCI) associated with the scheduled modulation and coding scheme (MCS) and the new data indicator (NDI) for the two codewords is also twice that for a single-codeword. Since asynchronous HARQ is supported for sPUSCH, a redundancy version field needs to be introduced in the DCI for sPUSCH [3], and this for each transmitted codeword. In order to reduce the signaling overhead for sTTI transmissions, it is proposed that only single-codeword is supported for uplink short TTI transmission. Note that this is also in line with the NR PUSCH codeword design. The mapping of one codeword to up to a 4-layer short TTI transmission can be done by reusing the legacy principle. That is, the codeword is mapped to across layer first, then to the physical resources.

- Proposal 1: Support only single-codeword for uplink short TTI transmission.

## Resource allocation

In several sTTI scheduling options discussed so far the resource allocation for sPUSCH is included in the sTTI DCI that can be sent in each DL sTTI. To limit the control overhead increase due to sTTI, there is a will to reduce the number of bits required for sPUSCH resource allocation in the DCI.  For simplicity, it can be assumed that the resource allocation types for sPUSCH are based on the existing resource allocation types for 1 ms TTI.

The basic UL resource allocation scheme in LTE is uplink resource allocation type 0, specifying a contiguous frequency interval. Adopting the same scheme also here would require a number of bits which from 3GPP TS 36.212 is given by .

In Table 1, the number of bits required for sPUSCH resource allocation are given for different system bandwidths. In order to reduce the number of bits, the uplink resource allocation type 0 could also be modified to include a contiguous range of groups, not specifying individual resource blocks. For instance, the same groups of PRB as defined for downlink allocation type 0 could be used but with a larger granularity (i.e. an increased RB group size). An example is given in Table 1. For a 10MHz UL bandwidth an eNB could allocate one or up to 7 contiguous groups of 6 PRBs. Thereby 2 to 3 bits are saved compared to the bitmap in allocation type 0 as shown in Table 1.

- Proposal 2: Support uplink resource allocation type 0 for sPUSCH resource allocation. Extend uplink resource allocation type 0 to cover allocation of consecutive groups of PRB.

Table 1. Number of bits for different allocation types.

| Bandwidth (RB) | UL RB group size (RB) | Number of groups | Required number of bits for UL allocation type 0 | Required number of bits for UL allocation type 0 with groups |
| --- | --- | --- | --- | --- |
| 25 | 4 | 7 | 9 | 5 |
| 50 | 6 | 9 | 11 | 6 |
| 100 | 12 | 9 | 13 | 6 |

The other UL resource allocation scheme in LTE for 1ms TTI is uplink resource allocation type 1, specifying two sets of RBGs, with each set consists of a continuous frequency interval. In a frequency selective fading channel, Type 1 based resource allocation allows more flexible scheduling, and can improve the reliability of data transmissions by harvesting more frequency diversity gain. Therefore, type 1 based resource allocation should also be supported for sPUSCH. Similar to type 0 based resource allocation. The granularity of type 1 based resource allocation can be increased by increasing the RBG size.

- Proposal 3: Support uplink resource allocation type 1 for sPUSCH resource allocation. Increase the granularity of uplink resource allocation type 1 by increasing the RBG size.

## Transport Block Size

The transport block size (TBS) for short TTI is discussed in detail in [2]. The main proposals regarding TBS for short TTI can be summarized as follows.

- Proposal 4: The TBS for short TTI operation is calculated by scaling a legacy TBS value for the number of RBs and MCS index according to the number of data symbols within the short TTI.

- Proposal 5: The scaled TBS value for short TTI operation is adjusted for CRC to achieve the intended code rate.

- Proposal 6: The scaled TBS value for short TTI operation is rounded off to the closest valid TBS table value.

# Conclusion

Based on the discussion in section 2, we have the following proposals:

Proposal 1: Support only single-codeword for uplink short TTI transmission.

Proposal 2: Support uplink resource allocation type 0 for sPUSCH resource allocation. Extend uplink resource allocation type 0 to cover allocation of consecutive groups of PRB.

Proposal 3: Support uplink resource allocation type 1 for sPUSCH resource allocation. Increase the granularity of uplink resource allocation type 1 by increasing the RBG size.

Proposal 4: The TBS for short TTI operation is calculated by scaling a legacy TBS value for the number of RBs and MCS index according to the number of data symbols within the short TTI.

Proposal 5: The scaled TBS value for short TTI operation is adjusted for CRC to achieve the intended code rate.

Proposal 6: The scaled TBS value for short TTI operation is rounded off to the closest valid TBS table value.

# References

TR 36.881,	Study on latency reduction techniques for LTE.

R1-1708849, TBS scaling for short TTI, Ericsson, RAN1#89

R1-1708869, Asynchronous HARQ for UL retransmissions with short TTI, RAN1#89, May 2017