3GPP TSG RAN WG1 Meeting #89		      R1-1708949

Hangzhou, P.R China 3rd - 7th April 2017

Agenda Item:	7.1.2.5

Source: 	CATR

Title: 	Phase 2 calibration Results of NR MIMO

Document for:	Discussion and Decision

Introduction

In RAN1 88 meeting, the following agreements on the phased approach for NR MIMO calibration have been made:

- System level and link level NR MIMO calibration has been conducted through the following phased approaches:

- Phase 1: Calibration is to check the link and system level channel models with basic beamforming behavior in terms of SNR/SINR distribution.

- Phase 2: Calibration is to check the link/system level performance through observing the metrics of BLER, spectrum efficiency and outage.

- Phase 3: Calibration is to check the additional link (if necessary)/system-level features including UE movement, UE rotation and channel blockage in terms of coupling loss, ASA, SINR and spectral efficiency.

The evaluation assumptions of phase 1 and phase 2 link level and system level assumptions are given in the reference. In this contribution, we give our LLS and SLS phase 2 calibration results of NR MIMO.

Initial link level calibration results of NR MIMO

For link level beam selection, there are two methods for calibration agreed as following:

- Beam selection method 1: The DFT beam directly is pointing to the strongest cluster.

- Beam selection method 2: Select a beam among the limited set of DFT beams with oversampling factor =1. Select the best beam pair at the same time based on the criteria of maximizing receive power after beamforming.

In our link level results, we adopt beam selection method 1, the channel models we used here are 30 GHz CDL-A.The detailed simulation assumptions are shown in the appendix.

Figure 1 Phase 2 calibration result of link level

Initial system simulation results of NR MIMO

In this section, we give our phase 2 calibration results of NR.

Figure 2 Phase 2 calibration result of link level

Conclusion

In this contribution, we give our Phase 2 link level and system level calibration results of NR MIMO based on the discussions and agreements of NR MIMO calibration.

References

- R1-1701823 Evaluation assumptions for Phase 1 NR MIMO link level calibration

- R1-1703534 Evaluation assumptions for Phase 1 NR MIMO system level calibration

- R1-1703535 Evaluation assumptions for Phase 2 NR MIMO link level calibration

Appendix simulation assumptions

Table 1 Simulation assumptions for Phase 2 link-level calibration

|  | Below 6GHz | Above 6GHz |
| --- | --- | --- |
| Carrier Frequency | 4 GHz | 30 GHz |
| Subcarrier Spacing | 15kHz | 60kHz |
| Data allocation | 8 RBs - Note: Error free PDCCH decoding is assumed. - First 2 OFDM symbols for PDCCH, and following 12 OFDM symbols for data channel - One-port DMRS (12 REs) |  |
| Channel Model | CDL-A and CDL-B - delay spread =100ns - UE speed=3km/h. - The angles of BS, i.e., AoD, ZoD, are uniformly distributed within [-60, 60] degrees in azimuth domain and [90, 135] degrees in zenith domain, and those of UE, i.e., AoA, ZoA, are uniformly distributed within [-180, 180] degrees in azimuth domain and [45, 90] in zenith domain, via applying uniform-distribution desired mean angle in Section 7.7.5.1 in TR 38.900 accordingly. |  |
| TXRU mapping to antenna elements | One TXRU per vertical dimension per polarization | One TXRU per panel per polarization |
| TXRU mapping weights | TXRU virtualization only in the vertical dimension (i.e. 1D virtualization) using DFT, i.e., 1D sub-array partition model defined in TR36.897 | 2D TXRU virtualization weights for each panel is the Kronecker product between vertical and horizontal weight vectors taken from DFT, i.e., 2D sub-array partition model defined in TR36.897. |
| Criteria for beam selection | Select the best beam pair among the limited set of DFT beams, based on the criteria of maximizing receive power after beamforming1). |  |
| BS antenna configurations | (M,N,P,Mg,Ng) = (4,4,2,1,1). (dV,dH) = (0.5, 0.5)λ | (M,N,P,Mg,Ng) = (4,8,2,2,2). (dV,dH) = (0.5, 0.5)λ. (dg,V,dg,H) = (2.0, 4.0)λ |
| BS antenna element radiation pattern | According to TR36.873 | See Table A.2.1-6 in TR 38.802 |
| UE antenna configurations | (M,N,P,Mg,Ng) = (1,1,2,1,1) | (M, N, P, Mg, Ng) = (2, 4, 2, 1, 2); (dV,dH) = (0.5, 0.5)λ. (dg,V,dg,H) = (0, 0)λ. 2) Θmg,ng=90; Ω0,1=Ω0,0+180; |
| BS array orientation | azimuth 0 degree; mechanic downtilt: 0 degree |  |
| UE array orientation | ΩUT, uniformly distributed on [0,360] degree, ΩUT,= 0 degree, ΩUT, = 0 degree |  |
| UE antenna element radiation pattern | Omnidirectional | See Table A.2.1-8 in TR 38.802 |
| BF scheme | Analog BF based on beam selection + Digital BF based on ideal SVD |  |
| MIMO mode | SU-MIMO with rank=1 |  |
| UE receiver type | MMSE-IRC |  |
| MCS | LTE MCS = 1 |  |
| DMRS channel estimation | 2DMMSE Based on LTE antenna port 7 |  |
| Metrics | BLER w/ beamforming as a function of transmission SNR ranging from -20 dB to 0dB |  |

Table 2 Simulation assumptions for Phase 2 urban-macro system-level calibration

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | 30 GHz |
| Mode | DL only |
| Bandwidth | 40MHz |
| Subcarrier Spacing | 60kHz |
| Channel Model | UMa in TR 38.900 |
| TXRU mapping to antenna elements | One TXRU per panel per polarization |
| TXRU mapping weights | 2D TXRU virtualization weights for each panel is the Kronecker product between vertical and horizontal weight vectors taken from DFT, i.e., 2D sub-array partition model defined in TR36.897. |
| Criteria for selection for serving TRP | Maximizing RSRP with best analog beam pair, where the digital beamforming is not considered. |
| Criteria for beam selection for serving TRP | Select the best beam pair among the limited set of DFT beams, based on the criteria of maximizing receive power after beamforming1). |
| Criteria for Beam Selection for interfering TRP | Considering the real traffic in adjacent cells, the actual beam or SVD precoder that is used by the non-serving TRPs in its data transmission is used as interfering beams. |
| Constraints for the range of selective beams per TRP sector | [-60, 60] in azimuth domain and [100, 160] in zenith domain |
| Scheduling algorithm | Round robin |
| Traffic Model | Full buffer |
| ISD | 500m |
| BS Tx power | 43dBm |
| BS Antenna Configuration | (M,N,P,Mg,Ng) = (4,8,2,2,2) (dH,dV) = (0.5, 0.5)λ (dg,H,dg,V) = (4.0, 2.0)λ |
| BS array orientation | azimuth 0 degree; mechanic downtilt: 0 degree |
| UE Configuration | (M, N, P, Mg, Ng) = (2, 4, 2, 1, 2); (dV,dH) = (0.5, 0.5)λ. (dg,V,dg,H) = (0, 0)λ. Θmg,ng=90; Ω0,1=Ω0,0+180; Notes: the polarization angles are 0 and 90 |
| UE array orientation | ΩUT, uniformly distributed on [0,360] degree, ΩUT,= 0 degree, ΩUT, = 0 degree |
| BS antenna pattern | See wall-mount in Table A.2.1-6 in TR 38.802 |
| UE antenna pattern | See Table A.2.1-8 in TR 38.802 |
| BS antenna height | 25m |
| UE antenna height | Same as 3D-UMa in TR36.873 |
| UE antenna gain | 5dBi |
| Noise figure for BS | 7dB |
| UE receiver noise figure | 10dB |
| UE distribution | 20% Outdoor in cars2): 30km/h, 80% Indoor in houses: 3km/h 10 users per TRP |
| Metric | - Outage3) - Spectral efficiency |