3GPP TSG-RAN WG1#89	R1-1709002

Hangzhou, P.R. China, May 15-19, 2017

Agenda item:		7.1.5

Source:	Nokia, Alcatel-Lucent Shanghai Bell

Title:	On spectrum shaping for uplink Pi/2 BPSK with DFT-S-OFDM

Document for:		Discussion and Decision

# 1	Introduction

RAN1 NR Ad Hoc in January 2017 agreed to introduce Pi/2 BPSK modulation for DFT-S-OFDM uplink as a low-PAPR modulation scheme:

Agreement:

- NR supports 0.5*pi BPSK modulation for DFT-s-OFDM

Further, RAN1#88bis in April 2017 agreed that Pi/2 BPSK modulation supports spectrum shaping

Agreements:

- pi/2 BPSK DFT-s-OFDM supports spectrum shaping without spectrum expansion of pi/2 BPSK data at least for uplink data for carrier frequencies above 6 GHz and below 52.6 GHz

  - Note that UE still has to fulfill all RAN4 requirements

  - FFS: Whether it will have RAN1 spec impact

  - FFS: Applicability below 6 GHz

    - Note that RAN1 needs to consider at least spectrum efficiency, PA efficiency, complexity, and coverage

In this contribution we briefly look into the next steps on the topic.

# 2	Discussion

Figure 1 [3] and Figure 2 [5] illustrate pre-DFT and post-DFT spectrum shaping architectures of a DFT-S-OFDM transmitter, respectively. In principle whether the spectrum shaping is taking place before or after is a matter of implementation even if some specific spectrum shaping algorithms may be more suitable for one or the other architecture.

BPSK  data

Pi/2 Constellation rotation

Precoder

M-point DFT

IFFT

CP

Subcarrier mapping

Figure 1: pi/2 BPSK with time-domain pulse shaping. Source: IITH et al [5]

Figure 2: pi/2 BPSK with frequency domain pulse shaping. Source, ZTE et al [6]

If the transmitter spectally shapes the DMRS signals, the spectrum shaping can be seen as part of the channel and the receiver does not need any knowledge of the shaping function.

Figure 3 shows PAPR and CM as function of roll-off. The transition band bins are weighted by the RRC function sampled appropriately before mapping to the IFFT input.

Figure 3: PAPR and CM for different RRC roll-off points for Pi/2-BPSK, QPSK and 16QAM

For 1 PRB transmission a roll-off of 0, 0.17, and 0.33 are shown. The roll-off of 0.33 with two addional subcarriers in the edge provides good PAR properties with small power loss. Leakage power for adjcant PRB is such low that no guard band needed.

| n_TX | n_Tx(1+roll-off) | roll-off | P_ex/ P_tot | P_loss [dB] |
| --- | --- | --- | --- | --- |
| 12 | 12 | 0,00 | 0,00 | 0,00 |
| 12 | 14 | 0,17 | 0,02 | -0,11 |
| 12 | 16 | 0,33 | 0,06 | -0,26 |

For 8 PRB transmission a roll-off of 0, 0.08, 0.17, 0.25, and 0.33 are shown. The roll-off of 0.25 provides good PAR properties with small power loss. Leakage power to adjacent PRB may be significant and one PRB guard band may be benefical depending on the allocation granularity and modulation used in adjacent PRBs. However, guard band could be BS scheduler issue and not need to be standardized.

| n_Tx | n_Tx(1+roll-off) |  | roll-off | P_ex/P_tot | P_loss [dB] |
| --- | --- | --- | --- | --- | --- |
| 96 |  | 96 | 0,00 | 0,00 | 0,00 |
| 96 |  | 104 | 0,08 | 0,01 | -0,06 |
| 96 |  | 112 | 0,17 | 0,03 | -0,13 |
| 96 |  | 120 | 0,25 | 0,05 | -0,20 |
| 96 |  | 128 | 0,33 | 0,06 | -0,27 |

# 3	Conclusions

The spectral shaping can supported without RAN1 specification impact by:

- DMRS symbols go through the specrum shaping filter, the shaping becomes part of the channel and there is no need for the receiver to know of the actual shaping function employed

- Setting sufficient in-band emissions and EVM for pi/2 BPSK for best power efficiency gain with flexible implementation choice.

# References

- RP-170855, “New WID on New Radio Access Technology”, NTT DoCoMo

- 3GPP TR38.912, “Study on New Radio (NR) Access Technology” V14.0.0

- R1-050702, “DFT-Spread OFDM with Pulse Shaping Filter in Frequency Domain in Evolved UTRA Uplink”, NTT DoCoMo, NEC, Sharp

- R1-050817, “RF signal aspects to frequency and time domain implementations of SC-FDMA”, Nokia

- R1-1704782, “pi/2 BPSK with precoding/post-DFT subcarrier spectrum shaping”, IITH, CEWiT, Reliance Jio, IITM, Tejas Networks

- R1-1704389, “Discussion On modulation schemes for NR”, ZTE, ZTE Microelectronics