3GPP TSG RAN WG1 Meeting#89	R1-1709034

Hangzhou, China, 15th – 19th May 2017

Agenda item:	7.1.2.2.3

Source: 	Samsung

Title: 	Link level evaluation results for beam management

Document for:	Discussion and Decision

# Introduction

In this contribution, we provide the link level simulation results of BLER vs SNR for beam management with respect to the Rx beam pair link switching latency.

# Simulation Assumptions

In our simulation, to simulate beam pair link switching, we assume the UE rotates in the azimuth direction with a rotation speed of 10/20/60/120 RPM respectively. Upon rotation, the UE realigns its Rx beam with a latency. This latency can be interpreted as the beam pair link switching delay which comprises beam pair link measurement and reporting latency as well as beam pair link indication latency. The latency in the simulation is selected as 0/100/200/500/1000/2000 subframes (0/25/50/125/250/500 ms) long.

The simulation assumptions in our link level simulation is as below in Table 1. Cases with 4 panels at the BS and 2 panels at the UE are simulated. The link level simulation assumptions consideration in our view is shown in our companion contribution [1].

Table 1: Simulation assumptions for link level simulation

| Attributes | Values |
| --- | --- |
| Carrier Frequency | 30 GHz |
| Subcarrier Spacing for data | 60kHz |
| Data allocation | - 8 RBs - First 2 OFDM symbols for PDCCH, and following 12 OFDM symbols for data channel |
| PDCCH decoding | Ideal |
| Channel Model | CDL-A model - delay spread =100ns - UE speed=3km/h. |
| TXRU mapping to antenna elements | - Subarray mapping, DFT BF weights - Each TXRU one-to-one mapped to antenna elements with the same polarization in each panel. - 8 ports at BS; 4 ports at UE. |
| Criteria for beam selection | Beam direction pointing to the strongest Cluster for BS and UE |
| UE reporting/indication | Simulate 0/100/200/500/1000/2000 subframes beam pair link switching latency |
| BS antenna configurations | (M,N,P,Mg,Ng) = (4,8,2,2,2). (dV,dH) = (0.5, 0.5)λ. (dg,V,dg,H) = (2.0, 4.0)λ |
| BS antenna element radiation pattern | According to TR38.802 |
| UE antenna configurations | (M, N, P, Mg, Ng) = (2, 4, 2, 1, 2); (dV,dH) = (0.5, 0.5)λ. (dg,V,dg,H) = (0, 0)λ. * Θmg,ng=90; Ω0,1=Ω0,0+180; |
| BS array orientation | azimuth 0 degree; mechanic downtilt: 0 degree |
| UE array orientation | ΩUT,a= 0 degree, ΩUT,b = 0 degree, ΩUT,g = 0 degree |
| UE rotation | 10/20/60/120 RPM |
| UE antenna element radiation pattern | See Table A.2.1-8 in TR 38.802 |
| Transmission scheme | Precoding is based on ideal SVD |
| MIMO mode | SU-MIMO (Rank 1) |
| UE receiver type | MMSE receiver |
| MCS | LTE MCS = 16 |
| Link adaptation | No Link adaptation |
| Metrics | BLER with beamforming vs SNR curve |

# Simulation Results

Figure 1 and 2 shows the BLER vs SNR for 0/100/200/500 subframes beam pair link switching latency for 120 RPM and 60 RPM.

Figure 1: 120 RPM

Observation 1: For UE rotation of 120 RPM, performance starts to degrade from 100 subframes (25ms) beam pair link switching latency, and significant performance degradation (~3dB loss at 0.1 BLER) is observed at 200 subframes (50ms) beam pair link switching latency.

Figure 2: 60 RPM

Observation 2: For UE rotation of 60 RPM, performance starts to degrade from 200 subframes (50ms) beam pair link switching latency, and significant performance degradation (>3dB loss at 0.1 BLER) is observed at 500 subframes (125ms) beam pair link switching latency.

Figure 3 shows the BLER vs SNR for 0/200/500/1000 subframes beam pair link switching latency for 20 RPM.

Figure 3: 20 RPM

Observation 3: For UE rotation of 20 RPM, performance starts to degrade from 200 subframes (50ms) beam pair link switching latency, and significant performance degradation (~ 5dB loss at 0.1 BLER) is observed at 1000 subframes (250ms) beam pair link switching latency.

Figure 4 shows the BLER vs SNR for 0/200/500/1000/2000 subframes beam pair link switching latency for 10 RPM.

Figure 4: 10 RPM

Observation 4: For UE rotation of 10 RPM, performance starts to degrade from 500 subframes (125ms) beam pair link switching latency, and significant performance degradation (~ 5dB loss at 0.1 BLER) is observed at 2000 subframes (500ms) beam pair link switching latency.

# Conclusions

This contribution gives the link level simulation results for beam management. The followings are observed.

Observation 1: For UE rotation of 120 RPM, performance starts to degrade from 100 subframes (25ms) beam pair link switching latency, and significant performance degradation (~3dB loss at 0.1 BLER) is observed at 200 subframes (50ms) beam pair link switching latency.

Observation 2: For UE rotation of 60 RPM, performance starts to degrade from 200 subframes (50ms) beam pair link switching latency, and significant performance degradation (>3dB loss at 0.1 BLER) is observed at 500 subframes (125ms) beam pair link switching latency.

Observation 3: For UE rotation of 20 RPM, performance starts to degrade from 200 subframes (50ms) beam pair link switching latency, and significant performance degradation (~ 5dB loss at 0.1 BLER) is observed at 1000 subframes (250ms) beam pair link switching latency.

Observation 4: For UE rotation of 10 RPM, performance starts to degrade from 500 subframes (125ms) beam pair link switching latency, and significant performance degradation (~ 5dB loss at 0.1 BLER) is observed at 2000 subframes (500ms) beam pair link switching latency.

# References

[1] R1-1707955, Link level simulation assumptions for beam management, Samsung