3GPP TSG RAN WG1 Meeting #89		       R1-1709037

Hangzhou, P,R. China, 15th – 19th May 2017

Agenda item:	7.1.2.5

Source:	Samsung

Title:	MIMO Phase-2 SLS calibration results on 30GHz frequency band

Document for:	Discussion and Decision

# Introduction

In RAN1#86, the agreements of starting MIMO calibration for NR have been made [1] and for stepwise calibration, following agreements on phased approach have been made in [86-20] email discussion [2].

Agreements:

Phase 1: Calibration can be used to check the channel model and the basic beamforming behavior, e.g., by looking at the SNR/SINR distribution (aim to finish it in RAN1#86bis)

Phase 2:  Start discussion on whether and how to establish the baseline.  Further discuss simulation assumptions for Phase 2 and Phase 3. Calibration can be used to check the link/system level performances, e.g., by looking at the BLER and spectrum efficiency (aim to finish it in RAN1#87)

Phase 3: Calibration can be used to check the UE movement/rotation/blockage (aim to finish it after RAN1#87)

This contribution provides phase 2 calibration results for urban macro, indoor hotspot and dense urban scenario updated from [3] based on simulation assumptions in [4].

# Spectral efficiency evaluation results

In this section, we presents the spectral efficiency (SE) results of phase 2 calibration per scenarios. In this results, we down-scaled the minimum scheduling time unit as 1/n ms, where subcarrier spacing is n×15kHz according to difference in subcarrier spacing per scenarios. In urban macro and indoor hotspot scenarios, the subcarrier spacing is given by 60kHz (i,e., n=4). Table 1 shows average SE and outage denoting cell throughput per bandwidth and the percentage of users that has zero throughput for each scenarios, respectively. Figure 1 shows the cumulative distribution function of SE per scenarios. Specific parameters for simulation can be found in Annex A.

Table 1: Spectral efficiency results

|  | Urban macro | Indoor hotspot | Dense urban (macro layer) | Dense urban (micro layer) |  |
| --- | --- | --- | --- | --- | --- |
| Average SE per UE | 0.18 bps/Hz | 0.41 bps/Hz | 0.36 bps/Hz | 0.30 bps/Hz |
| Outage | 25 % | 0 % | 0 % | 7 % |

Figure 1: Spectral efficiency statistics

# Conclusion

This contribution discussed the phase 2 system level calibration results for urban macro, indoor hotspot, and dense urban scenarios. These results would be taken into collections for the calibration comparison.

# References

[1] RAN1 Chairman’s note, 3GPP TSG RAN WG1 Meeting#86, Göteborg, Sweden, August, 2016.

[2] R1-1608661, “Summary of [86-20] email discussion on MIMO calibration for NR”, ZTE, 3GPP RAN WG1 Meeting#86bis, Lisbon, Portugal, October, 2016.

[3] R1-1705364, “MIMO Phase-2 SLS calibration results on 30GHz frequency band”, Samsung, 3GPP RAN WG1 Meeting#88bis,Spokane, USA, April, 2017.

[4] R1-1700395, “Evaluation assumptions for Phase 2 NR MIMO system level calibration”, ZTE, 3GPP RAN WG1 Meeting#88, Athens, Greece, February, 2017.

# Annex – A: Evaluation assumptions

## Urban macro scenario

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | 30 GHz |
| Mode | DL only |
| Bandwidth | 40MHz |
| Subcarrier Spacing | 60kHz |
| Channel Model | UMa in TR 38.900 |
| O2I penetration loss | 50% low loss and 50% high loss |
| TXRU mapping to antenna elements | One TXRU per panel per polarization |
| TXRU mapping weights | 2D TXRU virtualization weights for each panel is the Kronecker product between vertical and horizontal weight vectors taken from DFT, i.e., 2D sub-array partition model defined in TR36.897. |
| Criteria for selection for serving TRP | Maximizing RSRP with best analog beam pair, where the digital beamforming is not considered. The selection is done per drop, i.e., only at the beginning of drop and kept unchanged. |
| Criteria for beam selection for serving TRP | Select the best beam pair among the limited set of DFT beams, based on the criteria of maximizing receive power after beamforming*. The selection is done per drop, i.e., only at the beginning of drop and kept unchanged. |
| Criteria for Beam Selection for interfering TRP | Considering the real traffic in adjacent cells, the actual beam or SVD precoder that is used by the non-serving TRPs in its data transmission is used as interfering beams. |
| Constraints for the range of selective beams per TRP sector | [-60, 60] in azimuth domain and [100, 160] in zenith domain |
| Scheduling algorithm | Round robin |
| Traffic Model | Full buffer |
| ISD | 500m |
| BS Tx power | 43dBm |
| BS Antenna Configuration | (M,N,P,Mg,Ng) = (4,8,2,2,2) (dH,dV) = (0.5, 0.5)λ (dg,H,dg,V) = (4.0, 2.0)λ |
| BS array orientation | azimuth 0 degree; mechanic downtilt: 0 degree |
| UE Configuration | (M, N, P, Mg, Ng) = (2, 4, 2, 1, 2); (dV,dH) = (0.5, 0.5)λ. (dg,V,dg,H) = (0, 0)λ. Θmg,ng=90; Ω0,1=Ω0,0+180; Notes: the polarization angles are 0 and 90 |
| UE array orientation | ΩUT, uniformly distributed on [0,360] degree, ΩUT,= 0 degree, ΩUT, = 0 degree |
| BS antenna pattern | See Table A.2.1-6 in TR 38.802 |
| UE antenna pattern | See Table A.2.1-8 in TR 38.802 |
| BS antenna height | 25m |
| UE antenna height | Same as 3D-UMa in TR36.873 |
| UE antenna gain | 5dBi |
| Noise figure for BS | 7dB |
| UE receiver noise figure | 10dB |
| UE distribution | 20% Outdoor in cars: 30km/h, 80% Indoor in houses: 3km/h 10 users per TRP |
| Overhead | DMRS overhead (12/168) No control overhead |
| Metric | - Outage** - Spectral efficiency |

## Indoor hotspot scenario

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | 30 GHz |
| Mode | DL only |
| Bandwidth | 40MHz |
| Subcarrier Spacing | 60kHz |
| Channel Model | Indoor in TR 38.900 |
| TXRU mapping to antenna elements | One TXRU per panel per polarization |
| TXRU mapping weights | 2D TXRU virtualization weights for each panel is the Kronecker product between vertical and horizontal weight vectors taken from DFT, i.e., 2D sub-array partition model defined in TR36.897. |
| Criteria for selection for serving TRP | Maximizing RSRP with best analog beam pair, where the digital beamforming is not considered. The selection is done per drop, i.e., only at the beginning of drop and kept unchanged. |
| Criteria for beam selection for serving TRP | Select the best beam pair among the limited set of DFT beams, based on the criteria of maximizing receive power after beamforming**. The selection is done per drop, i.e., only at the beginning of drop and kept unchanged. |
| Criteria for Beam Selection for interfering TRP | Considering the real traffic in adjacent cells, the actual beam or SVD precoder that is used by the non-serving TRPs in its data transmission is used as interfering beams. |
| Constraints for the range of selective beams per TRP sector | [0, 180] in azimuth domain and [0, 180] in zenith domain |
| Scheduling algorithm | Round robin |
| ISD | 20m |
| BS Tx power | 23dBm |
| BS Antenna Configuration | (M,N,P,Mg,Ng) = (4,8,2,1,1). (dV,dH) = (0.5, 0.5)λ Notes: Boresight direction is perpendicular to the ceiling. |
| UE Configuration | (M, N, P, Mg, Ng) = (2, 4, 2, 1, 2); (dV,dH) = (0.5, 0.5)λ. (dg,V,dg,H) = (0, 0)λ. Θmg,ng=90; Ω0,1=Ω0,0+180; Notes: the polarization angles are 0 and 90 |
| UE array orientation | ΩUT, uniformly distributed on [0,360] degree, ΩUT,= 0 degree, ΩUT, = 0 degree |
| BS antenna pattern | See single sector pattern wall-mount pattern in Table A.2.1-7 in TR 38.802 |
| UE antenna pattern | See Table A.2.1-8 in TR 38.802 |
| BS antenna height | 3m |
| UE antenna height | 1.5 m |
| UE antenna gain | 5dBi |
| Noise figure for BS | 7dB |
| UE receiver noise figure | 10dB |
| UE distribution | 100% Indoor, 3km/h, 10 users per BS |
| Overhead | DMRS overhead (12/168) No control overhead |
| Metric | spectral efficiency |

## Dense urban(macro layer only) scenario

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | 4 GHz |
| Mode | DL only |
| Bandwidth | 20MHz |
| Subcarrier Spacing | 15kHz |
| Channel Model | 3D-UMa in TR36.873 |
| O2I penetration loss | Follow TR 36.873 |
| TXRU mapping to antenna elements | One TXRU per vertical dimension per polarization |
| TXRU mapping weights | TXRU virtualization only in the vertical dimension (i.e. 1D virtualization) using DFT, i.e., 1D sub-array partition model defined in TR36.897 |
| Criteria for selection for serving TRP | Maximizing RSRP with best analog beam pair, where the digital beamforming is not considered. The selection is done per drop, i.e., only at the beginning of drop and kept unchanged. |
| Criteria for beam selection for serving TRP | Select the best beam pair among the limited set of DFT beams, based on the criteria of maximizing receive power after beamforming**. The selection is done per drop, i.e., only at the beginning of drop and kept unchanged. |
| Criteria for Beam Selection for interfering TRP | Considering the real traffic in adjacent cells, the actual beam or SVD precoder that is used by the non-serving TRPs in its data transmission is used as interfering beams. |
| Constraints for the range of selective beams per TRP sector | [-60, 60] in azimuth domain and [100, 160] in zenith domain |
| Scheduling algorithm | Round robin |
| Traffic Model | Full buffer |
| BS antenna element radiation pattern | According to TR36.873 |
| UE antenna element radiation pattern | Omnidirectional |
| ISD | 200m |
| BS Tx power | 44dBm |
| BS Antenna Configuration | (M,N,P,Mg,Ng) = (8,8,2,1,1) (dH,dV) = (0.5, 0.8)λ |
| BS array orientation | azimuth 0 degree; mechanic downtilt: 0 degree |
| UE Configuration | (M,N,P,Mg,Ng) = (1,1,2,1,1) Notes: the polarization angles are 0 and 90 |
| BS antenna height | 25m |
| UE antenna height | Same as 3D-UMa in TR36.873 |
| Noise figure for BS | 5dB |
| UE receiver noise figure | 9dB |
| UE distribution | 20% Outdoor in cars: 30km/h, 80% Indoor in houses: 3km/h 10 users per TRP |
| Overhead | DMRS overhead (12/168) No control overhead |
| Metric | spectral efficiency |

## Dense urban(micro layer) scenario

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | 30 GHz |
| Mode | DL only |
| Bandwidth | 40MHz |
| Subcarrier Spacing | 60kHz |
| Channel Model | 3D-UMi in TR38.900 |
| TXRU mapping to antenna elements | One TXRU per panel per polarization |
| TXRU mapping weights | 2D TXRU virtualization weights for each panel is the Kronecker product between vertical and horizontal weight vectors taken from DFT, i.e., 2D sub-array partition model defined in TR36.897. |
| Criteria for selection for serving TRP | Maximizing RSRP with best analog beam pair, where the digital beamforming is not considered. |
| Criteria for beam selection for serving TRP | Select the best beam pair among the limited set of DFT beams, based on the criteria of maximizing receive power after beamforming**. |
| Criteria for Beam Selection for interfering TRP | Considering the real traffic in adjacent cells, the actual beam or SVD precoder that is used by the non-serving TRPs in its data transmission is used as interfering beams. |
| Constraints for the range of selective beams per TRP sector | [-60, 60] in azimuth domain and [35, 135] in zenith domain |
| Scheduling algorithm | Round robin |
| Traffic Model | Full buffer |
| BS Tx power | 33dBm |
| BS Antenna Configuration | (M,N,P,Mg,Ng) = (4,8,2,2,2), (dH,dV) = (0.5, 0.5)λ, (dg,H,dg,V) = (4.0, 2.0)λ |
| BS array orientation | azimuth 0 degree; mechanic downtilt: 0 degree |
| UE Configuration | (M, N, P, Mg, Ng) = (2, 4, 2, 1, 2); (dV,dH) = (0.5, 0.5)λ. (dg,V,dg,H) = (0, 0)λ. Θmg,ng=90; Ω0,1=Ω0,0+180; Notes: the polarization angles are 0 and 90 |
| UE array orientation | ΩUT,α uniformly distributed on [0,360] degree, ΩUT,β = 0 degree, ΩUT,γ = 0 degree |
| BS antenna height | 10m |
| UE antenna height | Same as 3D-UMi in TR36.873 |
| Noise figure for BS | 5dB |
| UE receiver noise figure | 9dB |
| UE distribution | 20% Outdoor in cars: 30km/h, 80% Indoor in houses: 3km/h 10 users per TRP |
| Number of the micro TRPs per macro TRP | 3 |
| TRP placement option | One-sector deployment ： Dropping of TRP and TRP antenna orientation according to TR 36.897 (non co-channel hetnet deployment) |
| BS antenna element radiation pattern | Directional in horizontal, directional in vertical (8dBi gain, HPBW = 65°, vertical tilt 90°, Am=30dB, SLAv=30dB ) |
| Minimum distance between Micro TRPs | 40m |
| Radius of UE dropping within a cluster | 50m |
| Minimum distance between Macro TRP and UE | 10m |
| Minimum distance between Micro TRP and UE | 10m |
| Minimum distance between Micro TRP and Macro TRP | 20m |
| Minimum distance between cluster center and Macro TRP | 20m |
| Radius for micro TRP dropping in a cluster | 55m |
| Minimum distance between small cell cluster centers | 40m |
| Overhead | DMRS overhead (12/168) No control overhead |
| Metric | spectral efficiency |