3GPP TSG-RAN WG1 Meeting#89	R1-1709064

Hangzhou, China, 15th – 19th May, 2017

Source:	Ericsson

Title:	On DM-RS for NR-PDCCH

Agenda Item:	7.1.3.1.1

Document for:	Discussion and Decision

# Introduction

The NR Study Item was completed and closed in RAN plenary meeting#75 where the TR 38.802 was approved and is available in [1]. The NR Work Item was also approved in [2] where the WI targets meeting both eMBB and URLLC related 5G requirements in Rel-15. The design of DL control channel is an essential part of the WI.

In RAN1#88bis, the following working assumptions were made [3].

Working assumption:

- One-port transmit diversity scheme with REG bundling per CCE is used for NR-PDCCH

  - FFS the bundling size

  - FFS: REG bundling is also for localized mapping in time and/or frequency-domain

- Companies are encouraged to provide evaluation results for 10 MHz and 20 MHz for larger aggregation levels and 5 MHz and 10 MHz for smaller aggregation levels

Working assumption:

  - A NR-CCE is defined as 6 REGs

    - Candidate bundle sizes for distributed REG-to-CCE mapping: 2 or 3 REGs if NR-CCE is defined as 6 REGs

  - FFS: impact of the NR-CCE definition on CORESET size, CCE aggregation levels, data resource allocation granularity, etc.

In this contribution, we discuss the DMRS density for the PDCCH and whether it should be configurable. We also discuss the number of REGs that may be bundled together for channel estimation.

# Discussion

## DM-RS in REGs

As discussed in [5], there are benefits to supporting both localized and distributed NR-PDCCH transmissions. One design consideration is whether the DM-RS density should be different depending on the use of one or more OFDM symbols for the transmission of the NR-PDCCH. While it may seem that there are some benefits of lowering the density in some cases, any potential gain obtained from this comes with significant disadvantages resulting from the fact that the structure of the REG differs based on where the REG appears in the NR-PDCCH. In some frequency bands, DM-RS may be needed in multiple OFDM symbols when the PDCCH spans multiple OFDM symbols in order to enhance phase tracking. Also, when MU-MIMO transmissions are used, there is a potential for the DM-RS REs of one UE to be overlapped with payload REs from the other UE which will impact performance. Furthermore, when mini-slot transmissions are considered, then, the UE may have to test for multiple hypotheses where the REG may or may not have DM-RS depending on the hypothesis which adds to the channel estimation complexity at the UE. Therefore, it is best to have a single unified design where a REG consistently contains a fixed number of DM-RS REs located in the same location always. This has the benefit of ensuring that DM-RS REs only overlap with other DM-RS REs for MU-MIMO transmission and also simplifies channel estimation at the UE.

## DM-RS density

To determine suitable DMRS density value, the performance of PDCCH is evaluated for different parameters as introduced next. The density of DMRS impacts the channel estimation quality. However, channel estimation quality also depends on the number of PRBs with DMRS that can be aggregated together for purpose of channel estimation with a greater number of contiguous PRBs resulting in better channel estimation performance On the other hand, when distributed transmissions are used, frequency diversity also is important to boost performance. Therefore, to benefit both from good channel estimation as well as frequency diversity when distributed transmission is used, it is important to determine number of REGs that should be kept together, in addition to DMRS density. Hence, the performance of PDCCH is simulated for values of DMRS densities in combination with different numbers of REGs that are kept together for distributed transmissions. Single-port precoder cycling is used as the transmit diversity scheme as per the current working assumption.

For the NR-PDCCH performance evaluation, we have made the following assumptions:

- Single-port precoder cycling as TxD scheme

- CCE of 6 REGs consisting of 6x12 sub-carriers  [4]

- Aggregation levels AL= 1, 2, 4 and 8

- DCI sizes of PL=20 and 60 bits with 16 CRC bits

- DMRS density of Rd= 16.55, 25% and 33.33%

- X=2 contiguous REGs in a CCE

- Distributed transmissions

With distributed transmissions the clusters of contiguous REGs are equally distributed in the frequency domain within the control resource set.  Based on the above assumptions, Table 1 shows the number of clusters of contiguous REGs denoted by Nc in frequency domain for a given aggregation level.

Table 1: The number of clusters, Nc, of X=2 contiguous REGs distributed in freq. domain for distributed transmission

| Aggregation Level | Number of clusters with X= 2 contiguous REGs per cluster |
| --- | --- |
| 1 | 3 |
| 2 | 6 |
| 4 | 12 |
| 8 | 24 |

Figures 1-Figure 4 show the BLER performance of PDCCH on TDL A-30ns@3km/h and TDL B-300ns@3km/h channel models for different DMRS density values

Figure 1: PDCCH BLER based on single port precoder cycling and TDL A-30ns channel model@3 km/h, assuming PL=60 payload bits, with CCE of 6 REGs and Nc=24 clusters of X=2 contiguous REGs and different DMRS densities

Figure 2: PDCCH BLER based on single port precoder cycling and TDL B-300ns channel model@3 km/h, assuming PL=60 payload bits, with CCE of 6 REGs and Nc=24 clusters of X=2 contiguous REGs and different DMRS densities

Figure 3: PDCCH BLER based on single port precoder cycling and TDL A-30ns channel model@3 km/h, assuming PL=20 payload bits, with CCE of 6 REGs and Nc=24 clusters of X=2 contiguous REGs and different DMRS densities

Figure 4: PDCCH BLER based on single port precoder cycling and TDL B-300ns channel model@3 km/h, assuming PL=20 payload bits, with CCE of 6 REGs and Nc=24 clusters of X=2 contiguous REGs and different DMRS densities

Considering all the results, we can observe from the evaluations that the use of a pilot density of 25% is the best option since a higher pilot density doesn’t provide any additional meaningful gain at higher aggregation levels and the loss compared to the higher density is not too great at aggregation level 1. Based on the discussion in this and the previous section, we propose the following.

Proposal: A UE assumes three DM-RS REs per REG in fixed locations within the REG for all REGs for both localized and distributed transmission.

## DMRS for single/multi-beam control channels

In NR, PDCCH may be sent to different UEs on different beams or with different precoders. Typically, the PDCCH can be transmitted in the same beams as the PDSCH it scheduled. For PDSCH, up to 12 orthogonal DMRS ports has been agreed to be supported for DL, which allows for a high degree of MU-MIMO. This can be even further extended by using different DMRS scrambling sequences to different co-scheduled users.  The assignment of DM-RS sequences that are non-orthogonal may be adequate when there is enough spatial separation between the beams or channels received at the different users which typically is the case when the base station has a very large number of antennas. However, orthogonal resources gives the highest degree of robustness. Aspects related to operation with analog beamforming in higher frequency bands must be carefully considered in the design of the DM-RS sequence set.

# Conclusions

In this contribution, we evaluated the NR-PDCCH performance for different DM-RS densities and channel conditions to determine the number of contiguous REGs and the corresponding RS density. The investigation results in the following proposals for the NR-PDCCH design.

Proposals:

- A UE assumes three DM-RS REs per REG in fixed locations within the REG for all REGs for both localized and distributed transmission.

# References

RP-170377		TR 38.802 v2.0.0 on Study on New Radio (NR) Access Technology Physical Layer Aspects; NTT DOCOMO

RP-170855		New WID on New Radio Access Technology; NTT DOCOMO

Chairman’s Notes	3GPP TSG RAN WG1 #88bis April 2017

R1-1709066		On REG Bundle and CCE Size for NR-PDCCH, Ericsson

R1-1709063 		On NR-PDCCH structure, Ericsson

Chairman’s Notes	3GPP TSG RAN WG1 Meeting#88

# Appendix

## Simulation assumptions

In RAN1#88, the following agreement was made on the evaluation assumptions for down-selection of TxD schemes [6]. Moreover, it was recommended to harmonize the evaluation assumption for other PDCCH related evaluations with the below agreements when possible. Hence, the link level simulation parameters used for the NR-PDCCH performance evaluations in this contribution are aligned with the following agreement and listed in Table 2.

Agreements:

- Evaluation assumption guidelines for down selection of TxD scheme for DL control channel:

- Aggregation levels: 1, 2, 4, 8 (Proponents can evaluate higher aggregation levels in addition, e.g., 16, 32)

- DCI size: 20 and 60 bits + 16 bit CRC

- CCE size: Proponents can choose within the agreed initial estimate of 4 to 8 REGs per CCE

- Practical channel estimation

  - MMSE for reference, other schemes can be evaluated in addition

  - Proponents should state assumptions on

    - Number of RS used for interpolation in time and frequency

    - PRB bundling assumption

- Antenna configurations and correlations corresponding to models at carrier frequencies of 4 GHz and 30 GHz (Prioritize 4 GHz)

- DMRS density 33% (other densities can be evaluated in addition)

- Number of OFDM symbols for transmission of PDCCH: 1 (companies may additionally evaluate for other values)

- Subcarrier spacing: 15 kHz (Other subcarriers spacing may be evaluated in addition)

- Channel model

  - TDL-A, TDL-C

    - Delay spread 30 ns, UE speed 3 km/h, (proponents can also evaluate 70 and 500 km/hr)

    - Delay spread 300 ns, UE spread 3 km/h

    - Delay spread 1000 ns, UE spread 3km/h

Table 2: Link level simulation parameters

| Parameters | Values |
| --- | --- |
| Carrier Frequency | 4 GHz |
| Control Resource Set Bandwidth | 20 MHz |
| Sub-carrier Spacing | 15 kHz |
| DCI Payload Size | 20 bits and 16 CRC bits |
| Modulation | QPSK |
| Channel Coding | Tail-biting Convolutional Code as in Rel-8 |
| Aggregation Level | 8 |
| CCE size | 6 REGs or 72 subcarriers |
| Number of OFDM symbols for NR-PDCCH | 1 |
| Channel Model | TDL-A, Delay spread 30 ns, UE speed 3 km/h TDL-B, Delay spread 300 ns, UE spread 3 km/h |
| gNB antenna configuration | 2Tx, 2 Rx |
| UE antenna configuration | 1Tx, 2Rx |
| Channel Estimation | MMSE |
| Noise Estimation | Ideal |
| Transmission Diversity Scheme | SFBC |