3GPP TSG RAN WG1 Meeting #89	R1-1709204

Hangzhou, China, 15-19 May 2017

Agenda Item:	7.1.2.4.7

Source:	Huawei, HiSilicon, Samsung

Title:	Summary of remaining issues and views on NR CSI-RS

Document for:	Discussion and decision

Introduction

During NR SI, CSI-RS have been discussed for various purposes such as CSI acquisition, beam management, mobility support, as captured by [1]. According to the agreements from RAN1#85 to RAN1#88bis in Appendix [2]-[8], potential functionalities of CSI-RS can be summarized as follows:

- CSI acquisition

- Beam management

- L3 CONNECTED mode mobility support

- Others, e.g., fine time/frequency tracking, beam failure monitoring, power control, etc.

The details of remaining issues of CSI-RS for each of these functionalities have been summarized in contribution [9]. As the CSI-RS related issues for L3 CONNECTED mode mobility are being discussed in another agenda item “7.1.1.5.2 Measurement based on CSI-RS for L3 mobility”, this contribution focuses on summarizing both remaining issues and main views from companies contributions  [10]-[35] on CSI-RS for CSI acquisition and beam management.

CSI-RS for CSI Acquisition

CSI-RS RE pattern

Open issues: CSI-RS RE patterns within a PRB and slot for 1-port and >=8-port up to 32-port CSI-RS including RE pattern aggregation details.

Contribution summary:

The views of the above issues along with their supporters can be summarized as follows.

- Structure of CSI-RS pattern

- Component pattern based: Huawei, HiSilicon, Intel, CATT, Nokia, Alcatel-Lucent Shanghai Bell, ZTE, Qualcomm, Samsung, LGE

- RE-level comb pattern based: Ericsson, Nokia, Alcatel-Lucent Shanghai Bell

  - Note 1: Component-based pattern can be described by one or more interleaved RE-level combs and is thus supported

  - Note 2: A PRB-level comb is a special case of one or more interleaved RE-level combs and is thus supported

- Component CSI-RS RE pattern(s) (Y,Z) for X>4 ports

- The size of YZ

  - YZ>2, e.g. (Y,Z)=(4,1), (2,2), (8,1), (2,4), (4,2), (8,2): Huawei, HiSilicon, Nokia, Alcatel-Lucent Shanghai Bell, ZTE

  - YZ=2, i.e. (Y,Z)=(2,1): Samsung, Qualcomm

- The number of supported pairs of (Y,Z) in total (i.e., not within an X port resource)

  - Multiple pairs of (Y,Z), e.g. (Y,Z)=(4,1), (2,2), (8,1), (2,4), (4,2), (8,2): Huawei, HiSilicon, ZTE, LGE

  - Single pair of (Y,Z), e.g. (Y,Z)=(2,1), (4,1), (2,2): Samsung, Qualcomm, Intel, CATT, Nokia, Alcatel-Lucent Shanghai Bell

- The number of adjacent OFDM symbols L for one CSI-RS resource spanning in N OFDM symbols

- Support L=N for N=2, 4: Huawei, HiSilicon

- Support L=N for N=2, and L=2 or 4 for N=4: CATT, Nokia, Alcatel-Lucent Shanghai Bell, Qualcomm, ZTE, LGE

- The RE mapping pattern of one CSI-RS resource is the same in all N symbols

- Support for at least N=2: Huawei, HiSilicon, ZTE, CATT

- Support for N=2, 4: Ericsson, Qualcomm

- Support for N=2, 4 with adjacent OFDM symbols, FFS for N=4 with non-adjacent OFDM symbols: Samsung, LGE

Based on the above views, the following proposals can be considered.

Proposal 1: At least for CSI acquisition, for N=2 OFDM symbols, support adjacent OFDM symbols for one CSI-RS resource

- For N=4 OFDM symbols, down-selection between 2 and 4 for the minimum number of adjacent OFDM symbols for one CSI-RS resource

Proposal 2: At least for CSI acquisition, for N=2 OFDM symbols, support a uniform RE mapping pattern wherein the same sub-carriers are occupied in each symbol of one CSI-RS resource

- FFS: For N=4 OFDM symbols, the RE mapping pattern of one CSI-RS resource

CDM

Open issues: Details on CDM-2/-4/-8 including CDM pattern, configuration, etc.

Contribution summary:

The views of the above issues along with their supporters can be summarized as follows.

- Whether or not to support “no CDM” in addition to the already agreed CDM-2/-4/-8

- Support “no CDM”: Huawei, HiSilicon, Nokia, Alcatel-Lucent Shanghai Bell, ZTE

  - For at least X=1 port: Huawei, HiSilicon, CATT

  - For X=1, 2, 4, 8, 12, 16, 32 ports: Nokia, Alcatel-Lucent Shanghai Bell, ZTE

- Support “no CDM” for beam management: LGE

- Not support “no CDM” for CSI acquisition: LGE

- CDM sequence

- OCC: Huawei, HiSilicon, Ericsson, Nokia, Alcatel-Lucent Shanghai Bell, CATT, ZTE, LGE

- CS: Ericsson

- CDM pattern

- For CDM 2:

  - Number of ports X

    - Support X=2 port: Nokia, Alcatel-Lucent Shanghai Bell

    - Support X=2, 4, 8 ports: Huawei, HiSilicon

    - Support X=2, 4, 8, 12, 16, 32 ports: CATT

    - Support X=4, 8, 16, 32 ports: ZTE

  - Time- and/or Frequency-domain CDM

    - Support time-domain CDM 2: CATT, ZTE

    - Support frequency-domain CDM 2: Huawei, HiSilicon, ZTE, CATT

- For CDM 4:

  - Number of ports

    - Support X=4, 8, 12, 16 ports: Nokia, Alcatel-Lucent Shanghai Bell, Huawei, HiSilicon

    - Support X=4, 8, 12, 16, 32 ports: CATT

    - Support X=16, 32 ports: ZTE

  - Time- and/or Frequency-domain CDM

    - Support time-domain CDM 4 : CATT, ZTE

    - Support frequency-domain CDM 4: Ericsson, Huawei, HiSilicon, CATT

    - Support time- and frequency-domain CDM 4: Ericsson, Huawei, HiSilicon, CATT, ZTE

- For CDM 8:

  - Number of ports

    - Support X=16, 32 ports: Nokia, Alcatel-Lucent Shanghai Bell

    - Support X=32 ports: Huawei, HiSilicon, CATT, ZTE

  - Time- and/or Frequency-domain CDM

    - Support frequency-domain CDM 8: Ericsson,

    - Support time- and frequency-domain CDM 8: Ericsson, Huawei, HiSilicon, ZTE, CATT

- A CDM pattern within/between component CSI-RS RE pattern(s)

- Support a CDM pattern within a component CSI-RS RE pattern: Huawei (N=1 and 2), HiSilicon, ZTE

- Support a CDM pattern across multiple component CSI-RS RE patterns: Samsung, Intel, CATT, Qualcomm, Nokia, Alcatel-Lucent Shanghai Bell

Based on the above views, the following proposals can be considered.

Proposal 3: Support the following CDM design for CSI-RS

- Support “no CDM” for CSI-RS in additional to the agreed CDM-2, CDM-4 and CDM-8.

  - FFS: which value(s) of X for X-port CSI-RS can be configured with “no CDM” by taking into account its use case(s) such as fine time and frequency tracking, beam management, phase compensation for CSI acquisition etc

- Support at least OCC as one type of CDM sequence

  - FFS: CDM of CSI-RS antenna ports on an RE-level comb based on phase rotation (cyclic shift) sequences

- Support at least CDM 8 for an X-port CSI-RS resource at least when X=32

  - FFS:  Support for other pairings of CDM value and number of ports X

RS RE densities

Open issues:

  - Values of D other than {1, 1/2} RE/port/PRB.

  - Detailed configurations for each value of D, including comb offset values for D<1, etc.

  - Possible combinations of value(s) of D and the number of CSI-RS ports X.

Contribution summary:

The views of the above issues along with their supporters can be summarized as follows.

Comb offset values

- Different comb offset values for different ports: Huawei, HiSilicon, CATT, Ericsson (RE-level combs)

- Same comb offset values for different ports: Huawei, HiSilicon, Nokia, Alcatel-Lucent Shanghai Bell

CSI-RS for Beam Management

Number of ports and reporting mechanism for the preferred beam

Contribution summary:

The number(s) of ports X in one CSI-RS resource, and the number(s) of CSI-RS resources K in N OFDM symbols for beam management

- If an antenna port(s) corresponds to a beam

- Up to X=[16, 32] ports per resource, and K = 1 resource in N = 1 OFDM symbol: Samsung

- Up to X=[32] ports per resource, and K = 1 resource in N = Y (Y>=1) OFDM symbol: LGE

- If an CSI-RS resource corresponds to a beam

- Up to X=2 ports per resource, and up to K = 8 resources in N = 1 OFDM symbol: Samsung

- Up to X=1 port per resource in N = 1 OFDM symbol: LGE

CSI-RS RE pattern

Open issues: CSI-RS RE patterns and configuration details of CSI-RS.

  - Tx and Rx beam sweeping patterns

  - Sub-time unit details including down-selection among options when sub-time units smaller than an OFDM symbol in a reference numerology, e.g. IFDMA, larger subcarrier spacing, DFT-based methods

Contribution summary:

The views of the above issues along with their supporters can be summarized as follows.

- Relations between the reference numerology, the CSI-RS numerology and the numerologies of channels sharing the same slot

- Reference numerology is the data channel numerology: Ericsson, CATT

- Reference numerology is the CSI-RS numerology for beam management: Samsung

- Size of one time unit

- Equal to an OFDM symbol in the reference numerology: Ericsson

- Equal to or larger than an OFDM symbol in the reference numerology: CATT, LGE

- Equal to an OFDM symbol in the CSI-RS numerology for beam management: Samsung

- Size of one sub-time unit

- Smaller than an OFDM symbol in the reference numerology: Ericsson, Intel, Nokia, Alcatel-Lucent Shanghai Bell, KT Corp, Spreadtrum, NEC, ZTE, OPPO, InterDigital

- Equal to and smaller than an OFDM symbol in the reference numerology: Huawei, HiSilicon, CATT, LGE

- Method to achieve sub-time units smaller than an OFDM symbol in a reference numerology if supported

- Support option 1 (IFDMA): Ericsson, Intel, Nokia, Alcatel-Lucent Shanghai Bell, LGE, KT Corp, Spreadtrum, NEC, Samsung

- Support option 2 (larger subcarrier spacing): Intel, CATT, ZTE, OPPO, NEC

- Support option 3 (DFT-based): InterDigital

Based on the above views, the following proposals can be considered.

Proposal 4: Make a decision on the relations among the reference numerology, the CSI-RS numerology and the numerologies of channels sharing the same slot; and make a decision on the size of one time unit

Proposal 5: Make a decision on whether or not to confirm the following WA including the details of sub-time units:

- Working assumption:

  - For CSI-RS for Beam Management, NR supports sub-time units equal to and smaller than an OFDM symbol in a reference numerology

Multiplexing of ports

Open issues: How to support multiplexing of CSI-RS ports, e.g. FDM/TDM/CDM.

Contribution summary:

- FDM: CATT, LGE, Nokia, Alcatel-Lucent Shanghai Bell, , Samsung

- Frequency-domain CDM: CATT (CDM in terms of CS)

Others

Besides of the issues in section 3.1~3.3, other remaining issues of CSI-RS for beam management are also identified as follows:

- RS RE densities

- Whether CSI-RS for beam management is UE-specifically configurable only or cell-specific options are also possible

Common Issues of CSI-RS

Structure of CSI-RS pattern

Open issues: Design attributes for multiple identified functionalities based on CSI-RS

Contribution summary:

- Multiple CSI-RS patterns have been discussed in multiple companies’ contributions for multiple use cases.

Proposal 6:  To harmonize CSI-RS design to the extent possible, NR studies design attributes that can be common across multiple identified functionalities based on CSI-RS.

Note: Further discussion on potential common attributes will occur during RAN1#89

Mapping of port numbers

Open issues: Mapping of port numbers to RE pattern.

Density D>1

Open issues:

  - Whether or not to support density D>1 RE/port/RB for CSI-RS

  - If D>1 RE/port/RB is supported, figure out the applied functionalities and the supported value(s) of D

Contribution summary:

The views of the above issues along with their supporters can be summarized as follows.

Density D>1 RE/port/PRB

- Support: Ericsson, Qualcomm, Samsung, Huawei, HiSilicon

- Not support: Nokia, Alcatel-Lucent Shanghai Bell

Location of CSI-RS

Open issues: Location of OFDM symbols for CSI-RS transmission.

Contribution summary:

- Location of CSI-RS in a slot

- Early and/or late part in a slot

  - Support both early and late part in a slot: Huawei, HiSilicon, Nokia, Alcatel-Lucent Shanghai Bell, Qualcomm, Samsung, CATT

  - Support  at least early part in a slot: LGE

- Indexes of OFDM symbols

  - Support {5th, 6th, 7th, 8th, 13th, 14th} for 2-symbol PDCCH: Samsung

  - Support {6th, 7th, 8th, 9th, 13th, 14th } for 3-symbol PDCCH: Samsung

  - Support {2nd, 4th, 5th, 13th, 14th }: Qualcomm

- Whether support or not overlapping between CSI-RS and other channels/RS

- Support overlapping: Nokia , Alcatel-Lucent Shanghai Bell, LGE

- Not support overlapping as starting point: Samsung, ZTE ( non-overlapping with PTRS)

Based on the above views, the following proposals can be considered.

Proposal 7: Support the following location of CSI-RS in a slot

- Early part of a slot after PDCCH and front-loaded DMRS

  - FFS: the exact location of OFDM symbols

- Late part of a slot

  - FFS: the exact location of OFDM symbols

CSI-RS sequence

Open issues: Design of CSI-RS sequence, including sequence initialization and generation.

Contribution summary:

- Support configurable CSI-RS sequence ID: ,Huawei, HiSilicon

Configuration of CSI-RS parameters

Open issues: Details on configuration of CSI-RS parameters including at least transmission bandwidth, bandwidth granularity, timing related configurations, numbers of configured Resource settings, Resource sets, Resources, and CSI-RS ports, etc.

Contribution summary:

- At least one of following options can be considered as CSI-RS transmission bandwidth: Samsung

  - System bandwidth

  - Bandwidth part

  - CORESET bandwidth

  - UE minimum bandwidth

  - N times of subband size (e.g. RBG or PRB bundling size)

  - Explicit signalling of starting PRB and CSI-RS bandwidth

- At least one of following options can be considered for CSI-RS transmission timing: Samsung

  - Specify the periodicity and timing offset as absolute time

  - Specify the periodicity and timing offset as the slot or mini-slot indices

  - Specify the periodicity and timing offset in sub-time unit (or OFDM symbol)

Transmission power related configuration

Open issues: How to define transmission power related configuration of CSI-RS (e.g. EPRE ratio between CSI-RS and PDSCH)? What would be the corresponding values?

Contribution summary:

- As a baseline, EPRE ratio between CSI-RS and PDSCH and the corresponding values in LTE are considered for the CSI-RS transmission power related configuration: Samsung

Activation/deactivation mechanism(s) for semi-persistent CSI-RS

Open issues: How to support activation/deactivation mechanism(s) for semi-persistent CSI-RS?

Contribution summary:

- Semi-persistent CSI-RS is activated/deactivated by MAC CE: LGE, Samsung, Huawei, HiSilicon

Conclusions

Based on the summary of both remaining issues and main views from companies’ contributions [10]-[35] for CSI-RS for CSI acquisition and beam management, the following proposals can be considered.

Proposal 1: At least for CSI acquisition, for N=2 OFDM symbols, support adjacent OFDM symbols for one CSI-RS resource

- For N=4 OFDM symbols, down-selection between 2 and 4 for the minimum number of adjacent OFDM symbols for one CSI-RS resource

Proposal 2: At least for CSI acquisition, for N=2 OFDM symbols, support a uniform RE mapping pattern wherein the same sub-carriers are occupied in each symbol of one CSI-RS resource

- FFS: For N=4 OFDM symbols, the RE mapping pattern of one CSI-RS resource

Proposal 3: Support the following CDM design for CSI-RS

- Support “no CDM” for CSI-RS in additional to the agreed CDM-2, CDM-4 and CDM-8.

  - FFS: which value(s) of X for X-port CSI-RS can be configured with “no CDM” by taking into account its use case(s) such as fine time and frequency tracking, beam management, phase compensation for CSI acquisition etc

- Support at least OCC as one type of CDM sequence

  - FFS: CDM of CSI-RS antenna ports on an RE-level comb based on phase rotation (cyclic shift) sequences

- Support at least CDM 8 for an X-port CSI-RS resource at least when X=32

  - FFS:  Support for other pairings of CDM value and number of ports X

Proposal 4: Make a decision on the relations among the reference numerology, the CSI-RS numerology and the numerologies of channels sharing the same slot; and make a decision on the size of one time unit

Proposal 5: Make a decision on whether or not to confirm the following WA including the details of sub-time units:

- Working assumption:

  - For CSI-RS for Beam Management, NR supports sub-time units equal to and smaller than an OFDM symbol in a reference numerology

Proposal 6:  To harmonize CSI-RS design to the extent possible, NR studies design attributes that can be common across multiple identified functionalities based on CSI-RS.

Proposal 7: Support the following location of CSI-RS in a slot

- Early part of a slot after PDCCH and front-loaded DMRS

  - FFS: the exact location of OFDM symbols

- Late part of a slot

  - FFS: the exact location of OFDM symbols

- References

- 3GPP, Study on new radio access technology physical layer aspects, TR 38.802 V14.0.0

- 3GPP, RAN #85, Chairman’s Notes

- 3GPP, RAN1 #86, Chairman’s Notes

- 3GPP, RAN1 #86bis, Chairman’s Notes

- 3GPP, RAN1 #87, Chairman’s Notes

- 3GPP, RAN1 NR-AH, Chairman’s Notes

- 3GPP, RAN1 #88, Chairman’s Notes

- 3GPP, RAN1 #88bis, Chairman’s Notes

- Samsung, Huawei, “Remaining issues on NR CSI-RS”, R1-1707981, Hangzhou, China, May 15-19, 2017.

- Huawei, “CSI-RS design for beam management”, R1-1706932, Hangzhou, China, May 15-19, 2017.

- Huawei, “CSI-RS design for CSI acquisition”, R1-1706931, Hangzhou, China, May 15-19, 2017.

- ZTE, “On CSI-RS for beam management”, R1-1707128, Hangzhou, China, May 15-19, 2017.

- ZTE, “On CSI-RS for CSI acquisition”, R1-1707129, Hangzhou, China, May 15-19, 2017.

- VIVO, “On CSI-RS design for NR”, R1-1707246, Hangzhou, China, May 15-19, 2017.

- Intel, “On CSI-RS for beam management”, R1-1707362, Hangzhou, China, May 15-19, 2017.

- Intel, “On CSI-RS for CSI acquisition”, R1-1707363, Hangzhou, China, May 15-19, 2017.

- CATT, “Discussion on CSI-RS for CSI acquisition”, R1-1707485, Hangzhou, China, May 15-19, 2017.

- CATT, “Discussion on CSI-RS for beam management”, R1-1707486, Hangzhou, China, May 15-19, 2017.

- LG Electronics, “On CSI-RS design for beam management”, R1-1707612, Hangzhou, China, May 15-19, 2017.

- LG Electronics, “On CSI-RS design for CSI acquisition”, R1-1707613, Hangzhou, China, May 15-19, 2017.

- OPPO, “CSI-RS design for beam management”, R1-1707700, Hangzhou, China, May 15-19, 2017.

- AT&T, “CSI-RS design principle”, R1-1707748, Hangzhou, China, May 15-19, 2017.

- Spreadtrum, “On using sub-time unit in CSI-RS for beam management”, R1-1707783, Hangzhou, China, May 15-19, 2017.

- NEC, “On CSI-RS design for beam management”, R1-1707810, Hangzhou, China, May 15-19, 2017.

- MediaTek, “On CSI-RS design for potential extension to support fine time and frequency tracking”, R1-1707853, Hangzhou, China, May 15-19, 2017.

- Samsung, “Discussions on CSI-RS design for NR MIMO”, R1-1707970, Hangzhou, China, May 15-19, 2017.

- Samsung, “CSI-RS for beam management”, R1-1707971, Hangzhou, China, May 15-19, 2017.

- InterDigital, “On CSI-RS Design for Beam Management”, R1-1708340, Hangzhou, China, May 15-19, 2017.

- NTT DOCOMO, “Views on CSI-RS”, R1-1708458, Hangzhou, China, May 15-19, 2017.

- Qualcomm, “Discussion on CSI-RS design”, R1-1708594, Hangzhou, China, May 15-19, 2017.

- Ericsson, “On CSI-RS design”, R1-1708701, Hangzhou, China, May 15-19, 2017.

- Ericsson, “On CSI-RS design for beam management”, R1-1708702, Hangzhou, China, May 15-19, 2017.

- Nokia, Alcatel-Lucent Shanghai Bell, “On the CSI-RS configurations for NR CSI acquisition”, R1-1708922, Hangzhou, China, May 15-19, 2017.

- Nokia, Alcatel-Lucent Shanghai Bell, “On CSI-RS Design for DL Beam Management”, R1-1708923, Hangzhou, China, May 15-19, 2017.

- KT, “CSI-RS design for DL beam management”, R1-1709146, Hangzhou, China, May 15-19, 2017.