3GPP TSG RAN WG1 Meeting #89                                                              R1-1709289

Hangzhou, China 15th – 19th May 2017

Agenda Item:     7.1.4.1.2

Source:              ZTE

Title: 	             NR LDPC design

Document for:   Discussion and Decision

# Introduction

At the 3GPP TSG RAN1 #88bis meeting, the following agreement and conclusion have been achieved [1]:

Agreement:

The base graph design is selected from the following alternatives:

Alt 1: One base graph covering ~1/5 <= R <= ~8/9

Alt 1a: Two nested base graphs, where:

- Base graph #1

  - Covers info block size K:

Kmin1 <=K<= Kmax1, Kmin1 > Kmin, Kmax1 =Kmax

  - Covers code rate R: ~1/3 <= R <= ~8/9; FFS whether Rmin can be ~1/5

- Base graph #2

  - Nested within base graph #1

  - Covers info block size K:

Kmin2 <=K<= Kmax2, Kmin2 =Kmin, Kmax2 < Kmax, where 512<=Kmax2<=2560

  - Covers code rate R: ~1/5 <= R <= ~2/3

  - Kbmax =16 is the starting point; lower values in the range 10<=Kbmax<16 are encouraged if feasible.

  - The set of supported shift sizes is taken from the set of shift sizes supported by the base graph supporting Kmax

Alt 2: Two base graphs, where:

- Base graph #1

  - Covers info block size K:

Kmin1 <=K<= Kmax1, Kmin1 > Kmin, Kmax1 =Kmax

  - Covers code rate R: ~1/3 <= R <= ~8/9; FFS whether Rmin can be ~1/5

- Base graph #2

  - Not nested within base graph #1

  - Covers info block size K:

Kmin2 <=K<= Kmax2, Kmin2 =Kmin, Kmax2 < Kmax, where 512<=Kmax2<=2560

  - Covers code rate R: ~1/5 <= R <= ~2/3

  - Kbmax = 10 is the starting point; higher values in the range 10<Kbmax<=16 can also be considered if necessary.

  - The set of supported shift sizes is taken from the set of shift sizes supported by the base graph supporting Kmax

BLER Performance is the main criterion for selecting between Alts 1, 1a and 2 (since it is already assumed that complexity is not increased significantly by the addition of a second smaller base graph); decoding latency (e.g. evaluated by the number of edges) should also be considered as an important criterion.

In this contribution, some further considerations for LDPC codes design are presented with focus on uniform LDPC base matrix design.

# Low Density Parity Check (LDPC) Codes

A LDPC code is defined by a sparse parity check matrix, which can be mapped to a bipartite / tanner graph composed of check nodes and variable nodes, as shown in Figure 1.

Figure 1 Tanner Graph for LDPC Codes

## Basic description of QC-LDPC Codes

The parity check matrix of structured LDPC Codes is defined by a matrix H of size, which consists of blocks of circularly shifted identity matrices or zero matrices of size, of the form as following:

If integer, define; if integer, defineand P is a standard permutation matrix of the form:

The size of  H is, here. And  of size  has the form:

There are several basic definitions and concepts for structured LDPC codes. H is the expand matrix of, and  is the base parity check matrix (or base matrix) of H, and z is the lift size (or expand factor). As we can see, the information block size K = N-M and N is the codeword block size. Through changing the lift size (expand factor) z, a LDPC set of variable information length and certain code rate can be obtained. Therefore, a base matrix Hb and an lift size (expand factor) z together can be used to represent H.

## Decode algorithm for evaluation of LDPC codes

Log-BP algorithm with LUT

The message passing algorithm is a decoding technique in which messages are passed from node to node through the Tanner graph. The nodes act as independent processes, collecting incoming messages and producing outgoing messages. There is no global control over the timing or the content of the messages; instead, the bit and check nodes follow a common local rule: send a message as soon as all necessary incoming messages have been received. When the graph is cycle-free, the message passing algorithm is a recursive algorithm that always converges to the true a posteriori log-likelihood ratios after a finite number of messages have been passed. However, most (if not all) “good” codes have cycles in their Tanner graphs. When applied to codes with cycles, the message passing algorithm is no longer exact but approximate. Fortunately, even when the graph has cycles, the message passing algorithm performs remarkably well, and its complexity is extremely low.

The message-passing decoder for an irregular or regular LDPC code can be summarized concisely in terms of the index sets and, as follows.

Let denote an “upward” message form check-node to bit-node n during the l-th iteration, and let denote an estimate of the n-th LLR after  iterations. The message passing decoder is:

Pseudo-code Description of Log-Domain Decoding Algorithm

- Initialization The variables and are initialized to * , for all and * , for all

- Parity node updates----(Row Information update, look up table) Where

- Bit node updates--------(Column Information update)

- Verify parity checks

- Iteration termination

Where we have defined

and used the fact that . The function  is fairly well behaved; it may be implemented via look-up table. When finite precision has been considered, 32 bit entries LUT is enough for LUT Log-BP.

MIN* Log-BP or Box-Plus Log-BP

In another approach [4], the check node update can be computed in pairs of incoming messages in recursive form, e.g. Umn= g (xn−1, g (xn−2, . . . , g(x2, x1))). It is based on the fact that for two random variables U and V, with LLR values L(U) and L(V), The function of Eq. (4) is usually referred to as box-plus operator, i.e.

## Consideration on NR LDPC design

### Down-selection of LDPC candidates for eMBB

For NR eMBB，because all designs are the base graph of Kbmax =22, the complexity and throughput difference of various companies are very slight. Therefore, it is suggested that only performance is used for down selection.

Proposal 1: The down-selection of eMBB LDPC candidates should be mainly based on performance.

For NR eMBB, the mother code rate at the maximum code size will affect the memory size, meanwhile the memory size will affect chip area and soft buffer size obviously. Therefore,  it is preferred that the lowest mother code rate is 1/3 instead of 1/5 when information block size = Kmax.

Proposal 2: the lowest mother code is 1/3 instead of 1/5 when information block size = Kmax.

### The size of parity check base matrix and compact base graph concept

According to several contributions in the previous meeting, we prefer that   kbmax≤26 and Zmax≥320 and

the matrix with small kbmax (e.g. 6, 8, 10, 12, 16, 20), which can support a compact structure. Fortunately, RAN1 has agreed that Kbmax=22 and Zmax = 384 which can comply with our preference very well.

Meanwhile, many companies have the interests to support two base graphs, one is used for large CBS larger than 2K and another more compact basegraph is used for small CBS lower than 2K. Furthermore, Kbmax=22 is used for the first one, and Kbmax=10~16 is used for the second one.

According to LTE TBS table in TS36.213, it is observed that high code rate between 0.66 and 0.93 also have to be supported for small CBS of several hundreds to 2K at a few top MCS levels.  Furthermore, for LTE 64QAM TBS tables, LTE downlink 256QAM TBS tables and LTE uplink 256QAM TBS tables, the above observation can be found. Since it is expected that NR eMBB will cover a similar scenario as LTE, we believe NR TBS table design could maintain the same philosophy as LTE.  Therefore, we also should consider the performance at small CBS and high code rate, but the importance of such case is lower than the performance at small CBS and low code rate due to concurrency.

Observation 1: it is observed that high code rate between 0.66 and 0.93 also have to be supported for small CBS of  several hundred bits to 2K bits at a few top MCS levels for LTE.

Proposal 3: It is preferred that the performance of LDPC codes at small CBS and high code rate should be considered but the importance is lower than that  at small CBS and low code rate.

According to our experience to design compact matrices, compared with none-compact matrix with Kbmax=32, we have found that very compact matrix with Kbmax<=10 has at least 0.1 dB performance loss for rate 2/3 and at least 0.2dB performance loss for rate 3/4~8/9 at BLER=0.01. However, it is found that normal compact matrix with Kbmax=16 has comparable performance to none-compact matrix for all code sizes and all code rates. Therefore, if two base graphs are considered by RAN1, it is reasonable that Kbmax=16 instead of Kbmax =10 is selected for the small base graph.

Proposal 4: Kbmax=16 instead of Kbmax =10 should be selected for the small base graph when two base graphs are studied.

### Number of base graph

It is preferred that single base graph is used for NR LDPC. The reason is shown as follows: 1) single base graph has comparable performance to two base graphs.  2) single base graph  is simple and unified.  3) single base graph is suitable for row parallel decoder because multiple matrices lead to complex connections between CNU pins and LLR memory banks.  4) single base graph needs less ROM for storage.   5)  two base graphs  cannot bring extra benefit of power consumption.  For the detail of the first reason, at least three companies have shown us enough good performance at small CBS with Kbmax=32 base graph or Kbmax=16 base graph.

Proposal 5: It is preferred that single LDPC base graph is defined for NR eMBB.

In the previous meeting, the base graph design is selected from Alt.1, Alt.1a or Alt.2.  Actually Alt.1a is a special case of Alt.1 since shortening based on single base graph can be equal to two nested base graphs. Therefore, our preference to Alt.1 also includes the function of Alt.1a.

Observation 2:  For the selection of base graph, Alt.1a can be regarded as since shortening based on single base graph can be equal to two nested base graphs.

### Uniform Base Matrix

To generate a LDPC code set of a certain range of code rate and code size, a uniform base matrix with the lowest code rate in the range is defined. A sub-base matrix of corresponding number of rows and columns is extracted from the uniform base matrix to support different code rates in the range. The lift size can be variable to support different code block sizes. An example for code rates of Ri and Rj is shown in Figure 2. An example of different lift size (expand factor) , namely Zs and Zt , is also shown in Figure 2.

Figure 2 Uniform Base Matrix for different Code Rates (Rj<Ri) and different Code Block Sizes (Zs<Zt)

In order to obtain the base matrix of specific code size, the uniform base matrix has to be modified to generate a modified base matrix, which will really be used as parity check base matrix in the encoder/decoder of the LDPC code of specific code size. That is to say, for the LDPC codes of different code sizes and the same code rate, the positions of non-negative-one elements of their base matrices is the same, and the values of non-negative-one elements of their base matrices need to be changed.

For each non-negative-one elements of the uniform base matrix above, the value should be modified. Let represents the i-th row, j-th column element of modified base matrix,  represents the i-th row, j-th column element of the uniform base matrixgiven by us. Then

Eg.

is the largest lift size(expand factor), and z is the currently used lift size uniquely corresponding to the currently used code size.  denotes the operation that rounds the elements in it to the nearest integers towards minus infinity. The selection of the aforementioned function can be based on performance, how many binary bits to indicate the modified base graph Hbm, the calculation complexity etc.

Proposal 6:  It is preferred that uniform base graph design should be considered for flexibility.

  - Single and uniform base graph(s) Hb is defined for all code rates and all code sizes.

  - modified base graph Hbm for lift size z1 is derived from the corresponding  uniform base graph and its predefined z0 by using function f as:

    - Hbm (i,j) = f(Hb (i,j),z1,z0)

- E.g., f = flooring, modulo, offset, combination

- The selection of f  is based on performance, how many binary bits to indicate the modified base graph Hbm, the calculation complexity etc

  - Parity-check matrix(s) for z1 is derived from the corresponding modified base graph(s) by using lifting

    - H= g(Hbm,z1)     where g = lifting ;

  - Shortening and puncturing are used for  providing various code rates  and code sizes

### The first element in each column of Base Matrix

In the LDPC decoder, each element (non -1) in the base matrix corresponds to a cyclic shift. The value of element equals ‘0’, which means that there is no need for cyclic shift. For any column in base matrix, if the first non -1 element of each column is equal to ‘0’, information bits in the original order can be derived when the first two rows are updated, which means that code block CRC can be used to terminate decoding and correct information block can be derived once CRC check can pass.

Firstly, this assumption will not affect performance and be suitable for any design of any company. Secondly, this assumption can avoid using inverse cycle shift network which means that half  complexity of cycle shift network can reduced. Finally, this idea has appeared for array LDPC about ten years ago.

Proposal 7: It is preferred that the first non-negative-one element in each column is zero for base parity check matrix since inverse cycle shift network can be avoided and the complexity of cycle shift network can be reduced to half.

## Uniform LDPC Design

### LDPC Base Matrix of uniform base graph

Three base matrices based on the same and single base graph are described in the attachment. The base matrix A is used for information block size between 1000 and 2048 for code rates of 1/3~8/9; base matrix B is used for information block size of [2048: 8448] for code rates of 1/3~8/9; base matrix C is used for information block size of [40 : 1000) for code rates of 1/5~3/4.  For base matrix C, the performance is optimized for each TBS & code rate with the best kbmax’ as shown in the attachment.

### LDPC Encoding Process

There are three steps to encode an information block of size K bits into a codeword block of size N bits. In detail,  Step 1: Choosing corresponding life size and modified base matrix; Step 2: LDPC encoding; Step 3: bit selection. Note that, if the size of information bits K is larger than Kmax=8448, code block segmentation is required to divide one TB into multiple code blocks.

#### Choosing life size and modified base matrix

For information size of K bits, the coding expanding factor Z is chosen to be the first element in Z set larger or equal to , wherein . The base matrix A  with Zmax of 96 is selected if 1000≤K<2048, B with Zmax of 384 is selected if 2048≤K≤8448 and  C with Zmax of 256 is selected if 40≤K<1000. Then the elements of coding base matrix are modified by coding expanding factor Z as the following:

#### LDPC Encoding

padding bits are attached at the end of information block.  bits information block is encoded into codeword of bits using the modified base matrix and lift size Z calculated above.

#### Bit Selection

Permutation is used to reorder the codeword bits in the predefined order. This operation can make suitable puncturing bits be selected to attain the best performance. The permutation vector is defined as follows: PV=[2~nb , 0, 1], wherein nb is the number of base matrix columns. .

, k=0, 1, …, nb*Z-1, where is the original sequence and is the shifted sequence. Bit selection is performed as the following to obtain codeword.

wherein “null” denotes padding bit. The process of LDPC encoding and bit selection for the base matrix A and B has been illustrated in Figure 3.

Figure 3 the process of LDPC encoding and bit selection

### Truncated Matrix for decoder in LDPC Decoding Process

In order to implement more efficient decoding, a part of matrix instead of the whole base matrix is used for decoding.  If K and N is given, the part of the previous mb’ rows and the previous nb’ columns of Hbm is truncated for decoding, which can be depicted by Figure 4. Furthermore, mb’ and nb’ can be calculated by the following formula:

For truncated base matrix:

the number of rows：

the number of columns：.

Figure 4    Truncated matrix from Hbm for decoder

### Performance

The performance of BLER=0.01 for the proposed LDPC base matrix A & B in the attachment is shown in the Figure 5 compared with [5]. More evaluation results are shown in [7] where LDPC in this contribution show a better performance than that of [5].

Figure 5 performance comparison to [5]

Figure 6 Required SNR vs. information size at BLER 10-2 (R = 8/9)

# Conclusion

In this contribution, some considerations of LDPC coding schemes for the new RAT are presented. In summary, we have the following proposals and observations:

Observation 1: It is observed that high code rate between 0.66 and 0.93 also have to be supported for small CBS of several hundred bits  to 2K bits at a few top MCS levels for LTE.

Observation 2:  For the selection of base graph, category 1a can be regarded as a special case of category 1 since shortening based on single base graph can be equal to two nested base graphs.

Proposal 1: The down-selection of eMBB LDPC candidates should be mainly based on performance.

Proposal 2: The lowest mother code is 1/3 instead of 1/5 when information block size = Kmax.

Proposal 3: It is preferred that the performance of LDPC codes at small CBS and high code rate should be considered but the importance is lower than that  at small CBS and low code rate.

Proposal 4: Kbmax=16 instead of Kbmax =10 should be selected for the small base graph when two base graphs are studied.

Proposal 5: It is preferred that single LDPC base graph is defined for NR eMBB.

Proposal 6:  It is preferred that uniform base graph design should be considered for flexibility.

  - Single and uniform base graph(s) Hb is defined for all code rates and all code sizes.

  - modified base graph Hbm for lift size z1 is derived from the corresponding  uniform base graph and its predefined z0 by using function f as:

    - Hbm (i,j) = f(Hb (i,j),z1,z0)

- E.g., f = flooring, modulo, offset, combination

- The selection of f  is based on performance, how many binary bits to indicate the modified base graph Hbm, the calculation complexity etc

  - Parity-check matrix(s) for z1 is derived from the corresponding modified base graph(s) by using lifting

    - H= g(Hbm,z1)     where g = lifting ;

  - Shortening and puncturing are used for  providing various code rates  and code sizes

Proposal 7: It is preferred that the first non-negative-one element in each column is zero for base parity check matrix since inverse cycle shift network can be avoided and the complexity of cycle shift network can be reduced to half.

# References

- 3GPP Draft Report of 3GPP TSG RAN WG1 #88bis V0.1.0

- 3GPP R1-1701598 Further consideration on flexibility of LDPC codes for NR 88#

- 3GPP R1-166413 Consideration on channel coding,  RAN1 86#

- "Approximate-Min* constraint node updating for LDPC code decoding," in Proc. of IEEE Military Communications Conference, Oct. 2003, pp. 157-162, Christopher Jones

- R1-1709162, LDPC Code Design for eMBB, Samsung, #89, Hangzhou, China, 15-19 May 2017.

- R1-1709181, On design and performance of NR eMBB LDPC Code, Qualcomm, #89, Hangzhou, China, 15-19 May 2017

- R1-1709288, Performance evaluation of LDPC codes for eMBB, ZTE