3GPP TSG RAN WG1 89			R1-1709354

# Hangzhou, China, 15th - 19th May 2017

Agenda item:	7.1.2.1.6

Source: 	Xinwei

Title: 	Summary of Issues on DL PRB bundling

Document for:	Discussion

Introduction

Agreements:

- For PRB bundling of data channel

  - Support common design for PRB bundling for different scenarios, e.g. channel reciprocal or non-reciprocal, different Tx/Rx beamforming, etc:

    - Specify common PRB bundling size set for all scenarios

      - FFS: Different PRB bundling size sets for different BWs

    - Specify common indication procedure for all scenarios

  - The following PRB bundling sizes are studied:

    - Specified value(s) X:

      - FFS X

      - FFS whether more than one value is needed.

      - Strive for value aligned with resource allocation granularity

    - Contiguous allocated PRBs when at least Y contiguous PRBs are allocated. FFS: values of Y;

    - Scheduled BW dependent, FFS the relationship

    - Values equal or larger than scheduled BW

    - FFS different transmission schemes may be related to different subsets of PRB bundling configurations

    - Others are not precluded;

  - Support UE specific PRB bundling size indication:

    - FFS: RRC configured with a subset, DCI dynamically indicated

      - DCI overhead should be considered;

        - MAC CE can be considered if the number of subset elements are large, details FFS

      - FFS: the presence of DCI field related to PRB bundling is configured by RRC;

    - FFS: Implicit signaling to inform PRB bundling size can be considered

  - FFS UE feedback assisted PRB bundling size

Based on the contributions in this agenda, the following issues are mainly discussed.

- Possible values in the set of PRB bundling size;

- UE-specific PRB bundling size indication procedures;

- UE assisted PRB bundling size;

- Others;

About Values in the Set

About values in the sets

- Consecutive scheduled bandwidth;(Xinwei, HW, QC)

- RBG dependent (Xinwei, Samsung, HW, IDG, E///, Nokia)

  - X = RBG; (Xinwei, Samsung, HW)

  - X= 0.5*RBG; (Xinwei)

- X =1; (Xinwei, Samsung)

- X=2; (HW)

- X=4; (HW,QC)

- Bandwidth larger than the scheduled; (Samsung)

About UE-specific PRB bundling Indication

The following PRB bundling size indication procedure are proposed

- Dynamic:

  - Flexible configuration + MAC CE +DCI; (ZTE)

  - DCI with two bits overhead increase; (Xinwei)

  - DCI;(HW)

  - Dynamically indicate full bandwidth operation; (E///)

  - Both dynamic and semi-static indication should be supported; (Nokia)

- Implicit indication; (IDG)

About UE-assisted PRB bundling size determination

CATT and HW supports UE-assisted PRB bundling size determination and configuration.

- References

[1] R1-1708672	On DL PRB bundling	Ericsson

[2] R1-1708584	Discussion on PRB bundling for DL	Qualcomm Incorporated

[3] R1-1707084	Further Evaluation and Discussion on PRB Bundling Size	Beijing Xinwei Telecom Techn.

[4] R1-1707116	On PRB Bundling	ZTE

[5] R1-1707473	Discussion on PRB bundling for DL	CATT

[6] R1-1707947	PRB bundling for NR DMRS	Samsung

[7] R1-1708133	PRB bundling size for DL data precoding	Huawei, HiSilicon

[8] R1-1708332	PRG Size Indication in NR	InterDigital, Inc.

[9] R1-1708897	On details of PRB Bundling	Nokia, Alcatel-Lucent Shanghai Bell