3GPP TSG RAN WG1 Meeting #89	R1-1709411

Hangzhou, China, 15-19 May, 2017

Agenda Item:	6.2.5.3

Source:	Huawei, HiSilicon, Qualcomm

Title:	TP on summary of evaluation for DL 1024 QAM

Document for:	Discussion/Decision

- Proposal

Incorporate the following text proposal in the downlink 1024QAM TR.

-----------------------------------------------------TP starts------------------------------------------------------------

<Unchanged parts are omitted>

- 5.1	Evaluation results

Editor’s note: This sub-clause collects the evaluation results of 1024QAM. Simulation assumptions are captured in Annex A.

- 5.1.1 Crossover point

Crossover point for AWGN

|  | 0% TX 0% RX | 1% TX 1% RX | 2% TX 2% RX | 3% TX 3% RX |
| --- | --- | --- | --- | --- |
| HW (R1-1708196) | 29.2dB for 2T2R; 23.2dB for 2T8R | 29.4dB for 2T2R; 23.4dB for 2T8R | 30.4dB for 2T2R; 24.0dB for 2T8R | 33.0dB for 2T2R; 25.5dB for 2T8R |
| ZTE (R1-1707186) | 25.8dB for 2T2R TM3; |  |  |  |

Crossover point for TDL-10ns

|  | Tx/Rx EVM(0%/0%) | Tx/Rx EVM(1%/1%) | 1.5%, 1% | 2%, 1% | Tx/Rx EVM(2%/2%) | 2.5%, 1% | 2.5%, 2% | 3%, 0% | 3%, 3% |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| HW (R1-1708196) |  | 32.5dB for 2T2R, TM4; 23.3dB for 2T8R, TM4 |  |  | 35dB for 2T2R; 24dB for 2T8R |  |  |  |  |
| ZTE (R1-1707186) | 24.1dB for 2T8R, TM4 |  | 29.6dB for 2T8R, TM9. | 31dB for 2T8R, TM9 |  | 36dB for 2T8R, TM9 | 39dB for 2T8R, TM9 |  |  |
| Intel (R1-1707312) | 27dB for 2T2R, 1layer 37dB for 2T2R, 2layer |  |  |  |  |  |  | 30dB for 2T2R, 1layer 42dB for 2T2R, 2layer | 30dB for 2T2R, 1layer |
| QC (R1-1708793) | 38.4dB for 2T2R TDL-A; 28.67dB for 2T4R TDL-A; 29dB for 2T2R TDL-D; 23dB for 2T4R TDL-D; | 29.63dB for 2T4R TDL-A; 29.73dB for 2T2R TDL-D; 23.09dB for 2T4R TDL-D; |  |  | 32.22dB for 2T4R TDL-A; 32.22dB for 2T2R TDL-D; 23.36dB for 2T4R TDL-D; |  |  |  |  |

Crossover point for TDL-100ns

|  | Tx/Rx EVM(0%/0%) | Tx/Rx EVM(1%/1%) | Tx/Rx EVM(2%/2%) |
| --- | --- | --- | --- |
| HW (R1-1708196) |  | 34dB for 2T2R, TM4; 24dB for 2T8R, TM4 | 36dB for 2T2R, TM4; 25dB for 2T8R, TM4 |
| ZTE (R1-1707186) | 23.9dB for 2T4R, TM9 |  |  |
| QC (R1-1708793) | 36.67 for 2T2R TDL-A; 29dB for 2T4R TDL-A; 28.64 for 2T2R TDL-D; 22.58dB for 2T4R TDL-D; | 40.31dB for 2T2R TDL-A; 29.73dB for 2T4R TDL-A; 29.36 for 2T2R TDL-D; 23.09dB for 2T4R TDL-D; | 32.59dB for 2T4R TDL-A; 31.62 for 2T2R TDL-D; 23.91dB for 2T4R TDL-D; |

- 5.1.2 Throughput

Throughput for AWGN

|  | 0%, 0% | Tx/Rx EVM(1%/1%) | 1.5%, 1% | Tx/Rx EVM(2%/2%) | 3%, 2% | 3%, 3% |
| --- | --- | --- | --- | --- | --- | --- |
| HW (R1-1708196) | 2T2R, TM4: ~175Mbps (CR 0.745) /~168Mbps (CR 0.9) @30dB 2T8R, TM4: ~175Mbps (CR 0.745) /~168Mbps (CR 0.9) @24dB | 2T2R, TM4: ~175Mbps (CR 0.745) /~168Mbps (CR 0.9) @30dB 2T8R, TM4: ~175Mbps (CR 0.745) /~168Mbps (CR 0.9) @24dB |  | 2T2R, TM4: ~175Mbps (CR 0.745) /~168Mbps (CR 0.9) @31dB 2T8R, TM4: ~175Mbps (CR 0.745) /~168Mbps (CR 0.9) @25dB |  | 2T2R, TM4: ~175Mbps (CR 0.745) /~168Mbps (CR 0.9) @34dB 2T8R, TM4: ~175Mbps (CR 0.745) /~168Mbps (CR 0.9) @28dB |
| ZTE (R1-1707186) | 22.4% gain for 2T2R, TM3 @ 30dB, 35dB |  | 21.8% gain for 2T2R, TM3 @ 30dB, 35dB |  | 21.8% gain for 2T2R, TM3 @ 35dB |  |

Throughput for TDL-10ns

|  | 0%, 0% | Tx/Rx EVM(1%/1%) | 1.5%, 1% | Tx/Rx EVM(2%/2%) | 3%, 0% | 3%,3% |
| --- | --- | --- | --- | --- | --- | --- |
| HW (R1-1708196) |  | 2T2R, TM4: ~175Mbps (CR 0.745) /~168Mbps (CR 0.9) @44dB 2T8R, TM4: ~175Mbps (CR 0.745) /~168Mbps (CR 0.9) @25dB |  | 2T2R, TM4:~175Mbps (CR 0.745) /~168Mbps (CR 0.9) @46dB 2T8R, TM4:~175Mbps (CR 0.745) /~168Mbps (CR 0.9) @26dB |  |  |
| ZTE (R1-1707186) | -24.8%, 0.4% gain@30dB, 35dB for 2T2R, TM4; 22.4% gain for 2T8R, TM4 @ 30dB, 35dB |  | 4%, 23.7% gain for 2T8R, TM9 @ 30dB, 35dB |  |  |  |
| Intel (R1-1707312) | 9%, 20% gain for 2T2R @ 30dB, 35dB |  |  |  | 0, 9.1% gain for 2T2R@30dB, 35dB -12%, -6% gain for 2T2R, 1layer @ 30dB, 35dB | 0, 5.6% gain for 2T2R@30dB, 35dB -54%, -24% gain for 2T2R, 1layer @ 30dB, 35dB |
| QC (R1-1708793) | -8%, -4% gain for 2T2R, TDL-A @30dB, 35dB; 3%, 20% gain for 2T4R, TDL-A @30dB, 35dB; 2%, 8% gain for 2T2R, TDL-D @30dB, 35dB; 13%, 11% gain for 2T4R, TDL-D @30dB, 35dB; | -8%, -4% gain for 2T2R, TDL-A @30dB, 35dB; 1%, 14% gain for 2T4R, TDL-A @30dB, 35dB; 1%, 7% gain for 2T2R, TDL-D @30dB, 35dB; 13%, 12% gain for 2T4R, TDL-D @30dB, 35dB; |  | -8%, -6% gain for 2T2R, TDL-A @30dB, 35dB; -5%, 3% gain for 2T4R, TDL-A @30dB, 35dB; -1%, 3% gain for 2T2R, TDL-D @30dB, 35dB; 11%, 13% gain for 2T4R, TDL-D @30dB, 35dB; |  |  |

Throughput for TDL-100ns

|  | 0%, 0% | Tx/Rx EVM(1%/1%) | Tx/Rx EVM(2%/2%) |
| --- | --- | --- | --- |
| HW (R1-1708196) |  | 2T2R, TM4: ~175Mbps (CR 0.745) /~168Mbps (CR 0.9) @44dB 2T8R, TM4: ~175Mbps (CR 0.745) /~168Mbps (CR 0.9) @25dB | 2T2R, TM4: ~175Mbps (CR 0.745) /~168Mbps (CR 0.9) @46dB 2T8R, TM4: ~175Mbps (CR 0.745) /~168Mbps (CR 0.9) @26dB |
| ZTE (R1-1707186) | 7.7%, 17.4% gain for 2T2R, TM9 @ 30dB, 35dB; 22.1%, 22.4% gain for 2T4R, TM9 @ 30dB, 35dB |  |  |
| QC (R1-1708793) | -6%, -3% gain for 2T2R, TDL-A @30dB, 35dB; 4%, 22% gain for 2T4R, TDL-A @30dB, 35dB; 3%, 9% gain for 2T2R, TDL-D @30dB, 35dB; 13%, 11% gain for 2T4R, TDL-D @30dB, 35dB; | -6%, -3% gain for 2T2R, TDL-A @30dB, 35dB; 1%, 19.0% gain for 2T4R, TDL-A @30dB, 35dB; 1%, 8% gain for 2T2R, TDL-D @30dB, 35dB; 13%, 12% gain for 2T4R, TDL-D @30dB, 35dB; | -5%, -2% gain for 2T2R, TDL-A @30dB, 35dB; -5%, 4.0% gain for 2T4R, TDL-A @30dB, 35dB; -2%, 3% gain for 2T2R, TDL-D @30dB, 35dB; 11%, 13% gain for 2T4R, TDL-D @30dB, 35dB; |

<Unchanged parts are omitted>

- 5.3	Summary of findings

Editor’s note: This sub-clause summarizes the feasibility and performance benefit of 1024QAM for DL channels.

The following can be observed:

Observation:

  - With increased number of receiving antennas, the crossover SNR is decreased.

  - With increased TX/RX EVM, the crossover SNR is increased.

  - The gains provided by 1024QAM are higher in scenarios with LOS component.

  - The crossover SNR is lower for scenarios with LOS component.

  - For 2T2R under TDL-A/B channel, 1024QAM provides performance gain of 3%~10% at 40dB for TX EVM less than or equal to 2%.

  - For 2T4R under TDL-A/B channel, 1024QAM provides performance gain of 10%~22% at 35dB for TX EVM less than or equal to 2%.

  - For 2T8R under TDL-B/D channel, 1024QAM starts to provide performance gain at 24~28dB for TX EVM less than or equal to 2%.

  - For 2T2R under TDL-D/E channel with correlated LOS, 1024QAM provides performance gain of 0-11% at 30dB SNR for TX EVM less than or equal to 2%

  - For 2T4R under TDL-D/E channel with correlated LOS, 1024QAM provides performance gains of 11-16% at 30dB SNR for TX EVM less than or equal to 2%

  - For 2T2R under TDL-D/E channel with uncorrelated LOS, 1024QAM provides performance gain of 0-19% at 30dB SNR for Tx EVM less than or equal to 2%

  - For 2T4R under TDL-D/E channel with uncorrelated LOS, 1024QAM provides performance gain of 8-22% at 30dB SNR  for Tx EVM less than or equal to 2%

  - For 2T2R and 2T4R TDL-D/E channel with uncorrelated LOS, 1024QAM provides performance gains of 16-22% gain at 35dB SNR

  - For 2T8R TDL-D channel, 1024QAM provides performance gains of 12.8%~21% at 35dB SNR for TX EVM less than or equal to 2%

-----------------------------------------------------TP ends------------------------------------------------------------