3GPP TSG RAN WG1 Meeting #89	R1-1709670

Hangzhou, China, 15th – 19th May, 2017

Agenda Item:	7.1.3.3.7

Source:	Huawei, HiSilicon

Title:	Numerology for multiplexing of eMBB and URLLC

Document for:	Discussion and decision

# Introduction

It is agreed in RAN1 meetings that FDM and/or TDM of mixed numerologies is supported. The following are also agreed.

Agreements:

- From network perspective, multiplexing of transmissions with different latency and/or reliability requirements for eMBB/URLLC in DL is supported by

  - Using the same sub-carrier spacing with the same CP overhead

    - FFS: different CP overhead

  - Using different sub-carrier spacing

    - FFS: CP overhead

  - NR supports both approaches by specification

- NR should support dynamic resource sharing between different latency and/or reliability requirements for eMBB/URLLC in DL

Agreement:

- Possible use cases for the extended CP include

  - Multiplexing of eMBB and URLLC deployed below 6 GHz

    - SCS for eMBB 15(NCP)/30/60kHz, SCS for URLLC = 60 kHz

  - Transmission of URLLC with 60 kHz SCS

  - High speed scenarios for 30kHz and 60kHz

- Support extended CP at least for 60 kHz SCS

  - UE support for ECP may depend on UE type/capability

  - FFS how to configure UE using different CP overhead

  - FFS the length of ECP

- FFS extended  CP for other scenarios/numerologies

This tdoc discusses application of different numerologies for coexistence of eMBB and URLLC. In particular, how eMBB and URLLC traffic can coexist in resources configured for 60kHz.

# Mixed numerology for eMBB and URLLC multiplexing

NR may support diverse kinds of traffic in a common carrier with same or different numerology,e.g., eMBB and URLLC have different KPI requirements and URLLC requires much shorter latency than eMBB. To satisfy the URLLC latency, shorter transmission interval can be adopted by using larger SCS in a separate BW part than eMBB which may use smaller SCS such as 15kHz. If both eMBB and URLLC traffic are scheduled in shared time-frequency region, the BW parts configured for eMBB and URLLC UEs can be potentially overlapped.

## Overlapping BW parts

BW parts of different UEs can be configured in an overlapping manner. During resources assignment, gNB ensures orthogonality of resources allocation, cf. Figure 1.The overlapping part can be assigned dynamically. In an example, UE 1 monitors control more frequently than UE 2, in particular, monitoring period is two times longer for UE 2 than UE 1. In Figure 1a, in the first interval, UE 1 PDCCH is not received. NW assigns overlapping resources to UE 2. When in the second interval, UE 1 PDCCH is received, NW pre-empts part of the overlapping resource from UE 2 and assigns it to UE 1. In Figure 1b, it is assumed that pre-emption is not supported. As the overlapping part is not the entire BW, it would be beneficial for the preemption indication design to consider the overlapping BW parts.

Figure 1: Overlapping portion is assigned dynamically which may or may not cause pre-emption of resources of another transmission

Observation1: Configured BW parts for eMBB and URLLC UEs can overlap for both UL and DL.

## Coexistence region

In Figure 2, FDM of numerologies is exploited to configure resources for eMBB and URLLC transmission. In particular, slot-based transmission is adopted. URLLC slot spans 0.125ms based on 60kHz whereas eMBB transmission used 15kHz slot of 7 symbols. eMBB traffic can be scheduled with same granularity, e.g., slot, which will not require pre-emption. On the other hand, eMBB traffic can be scheduled over a longer interval, e.g., by aggregating multiple 0.125ms slots, which may experience pre-emption by URLLC traffic. For TDD, 60kHz based resources can be adopted cf. Figure 3.

Figure 2: BW part configured with different numerologies can be used for transmission of eMBB and URLLC traffic. BW part configured with 60kHz SCS is shown to contain both URLLC and eMBB traffic. BW part configured with 15kHz SCS is used for scheduling eMBB traffic only.

Figure 3: TDD structure with 60kHz SCS

In [1] and [2], it was discussed why 60kHz 7-symbol slot is preferred to meet latency and reliability requirement for URLLC transmission. Furthermore, 15kHz numerology resources may be used for initial access related information and critical system information. Scheduling URLLC traffic would have to avoid the resources containing critical information. If 15kHz numerology is used as default for sending those information, then URLLC scheduling will be constrained in the 15kHz resources. Hence, we propose that for data transmission, 60kHz resources should be chosen as default for URLLC transmission.

We further provide link level simulation results comparing the performance of 60kHz and 30kHz based duration containing same number of symbols. For evaluation purpose, we assume 2-symbol TTI for both 60kHz and 30kHz. The key simulation assumptions are listed in Table 1. From the results, it can be observed that in order to reach 10-5 BLER, the required SNR is -5dB for 60kHz 2-symbol and -2dB for 30kHz 2-symbol. For both cases, the URLLC packet size is 32 byte with 1/3 code rate. The reason that 60kHz outperforms 30kHz significantly is that more transmission opportunities in 1ms are available for a packet transmitted with 60kHz 2-symbol. For 60kHz, the maximum number of HARQ transmissions of a packet is 6, while for 30kHz, the maximum number of HARQ transmission is 3. Considering the fact that the URLLC KPI would also need to be met at cell edge where low SNR can be observed, it is preferable to configure 60kHz SCS for URLLC UE so that adequate transmission opportunities can be realized to meet URLLC KPI.

Observation 2: Higher SNR is required for 30kHz 2-symbol to meet URLLC KPI when compared with 60kHz 2-symbol which can be critical for operation of cell-edge URLLC UE.

Proposal 1: 60kHz SCS should be adopted as the default SCS for URLLC transmission.

Proposal 2: 15kHz should be adopted as default feature for initial access related information and critical system information in sub-6GHz.

Figure 4:	Performance comparison between 30kHz 2-symbol and 60kHz 2-symbol

Table 1	Link level simulation assumptions

| Bandwidth | 20MHz |
| --- | --- |
| Subcarrier space | 30kHz，60kHz |
| URLLC TTI | 2 symbol |
| Rank | 1 |
| Tx/Rx | 2X2 |
| Channel Model | TDL-C 300ns |
| Max HARQ | 30KHz: 3 60kHz: 6 |
| URLLC data rate | 1 packet/TTI |
| URLLC data size | QPSK 32byte 1/3 code rate |
| AMC | OFF |

## Scheduling eMBB traffic in Coexistence Region

If eMBB traffic is scheduled in coexistence region employing 60kHz, it may employ slot aggregation, cf. Figure 5. One or more sub-sequent slots in the aggregation can be pre-empted by URLLC traffic. Aggregated sub-sequent slot may or may not contain DMRS. The symbol containing DMRS in a slot may be protected from URLLC transmission. If the URLLC traffic arrives in the slot where the UE monitors PDCCH, slot aggregation may start from next slot.

Figure 5: eMBB traffic slot aggregated in coexistence region

## Coexistence with same/different CP overhead

ECP use with 60kHz has been agreed. eMBB and URLLC traffic can be scheduled with different CP overhead in different duration, cf. Figure 6.

Figure 6: Coexistence  with NCP an ECP at different durations.

In particular, if eMBB traffic is slot-based and/or scheduled by slot aggregation, URLLC traffic can pre-empt one eMBB slot where URLLC transmission may use a different CP overhead, cf. Figure 7. An ECP slot has one less symbol than NCP slot of same sub-carrier spacing. As seen in Figure 7, ECP and NCP transmission align at the slot boundary of the same sub-carrier spacing or at the slot boundary of the smaller sub-carrier spacing.

Figure 7:  eMBB slotted traffic can be pre-empted by URLLC traffic with same or a numerology with larger SCS with same or different CP overhead.

Proposal 3: FDM of ECP/NCP slot(s) of URLLC traffic and slot of eMBB traffic is supported, where URLLC traffic may employ same or larger SCS than eMBB.

# Conclusion

In this contribution, we provide our views on mixed numerology coexistence in a carrier. We have the following observations and proposals.

Observation1: Configured BW parts for eMBB and URLLC UEs can overlap for both UL and DL.

Observation 2: Higher SNR is required for 30kHz 2-symbol to meet URLLC KPI when compared with 60kHz 2-symbol which can be critical for operation of cell-edge URLLC UE.

Proposal 1: 60kHz SCS should be adopted as the default SCS for URLLC transmission.

Proposal 2: 15kHz should be adopted as default feature for initial access related information and critical system information in sub-6GHz.

Proposal 3: FDM of ECP/NCP slot(s) of URLLC traffic and slot of eMBB traffic is supported, where URLLC traffic may employ same or larger SCS than eMBB.

# References

[1] R1-1611219, “Overview of DL URLLC Support in NR”, Huawei, HiSilicon, RAN1 # 87, Nov 2016

[2] R1-1700023, “Discussion on mini-slot for URLLC”, Huawei, HiSilicon, RAN1 Adhoc, Jan 2017