3GPP TSG RAN WG1 Meeting #89	R1-1709858

Hangzhou, China, 15-19 May, 2017

Agenda Item:	6.2.5.1

Source:	Huawei, HiSilicon

Title:	TP on summary of evaluation for DL 1024 QAM

Document for:	Discussion/Decision

- Proposal

Incorporate the following text proposal in the downlink 1024QAM TR.

-----------------------------------------------------TP starts------------------------------------------------------------

<Unchanged parts are omitted>

- 5.1	Evaluation results

Editor’s note: This sub-clause collects the evaluation results of 1024QAM. Simulation assumptions are captured in Annex A.

- 5.1.1 Crossover point

The evaluation results of crossover SNR are summarized in Table 5.1.1-1 and 5.1.1-2 for AWGN channel and TDL channels, respectively.

Table 5.1.1-1 Crossover point for AWGN

| sources | scenarios | EVM |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0% TX 0% RX | 1% TX 1% RX | 1.5% TX 1% RX | 2% Tx 1% Rx | 2% TX 2% Rx | 2.5% TX 1% RX | 3% TX 2% RX | 3% TX 3% RX |
| Source 1 | TM3 2T2R, Fixed MCS, TBS=105528 (256QAM 0% TX and 0% RX EVM) | 25.8dB |  |  |  |  |  |  |  |
|  | TM3 2T2R, Fixed MCS, TBS=119816 (256QAM 0% TX and 0% RX EVM) | 28.6dB |  |  |  |  |  |  |  |
|  | TM3 2T2R, Fixed MCS, TBS=105528 (256QAM 3% TX and 0% RX EVM) | - |  | 23.8dB | 26.3dB | 26.4dB | 27dB | 27.8dB | 27.8dB |
|  | TM3 2T2R, Fixed MCS, TBS=119816 (256QAM 3% TX and 0% RX EVM) | 28.6 |  | 29.2dB | 29.7dB | 30.7dB | 30.9 | 32.8 | 33.6 |
| Source 3 | TM4 2T2R, Fixed MCS | 29.2dB | 29.4dB |  |  | 30.4dB |  |  | 33.0dB |
|  | TM4 2T48, Fixed MCS | 23.2dB | 23.4dB |  |  | 24.0dB |  |  | 25.5dB |
| Source 4 | TM3, 2T2R, Fixed MCS | 27dB | 28.5dB |  | 28.6dB |  |  |  |  |

Table 5.1.1-2 Crossover point for TDL Channels

| sources | Scenarios | EVM |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0% TX 0% RX | 1% TX 1% RX | 1.5% TX 1% RX | 2% TX 1.5% Rx | 2% TX 2% Rx | 2.5% TX 2% Rx | 3% Tx 0% Rx | 3% Tx 1.5% Rx | 3% Tx 3% Rx |
| Source 1 | TDL-D, 10ns, 2T8R, TM9, 1layer, Fixed MCS, TBS=119816 (256QAM 3% TX and 0% RX EVM) | 28dB |  | 29.7dB | 31dB |  | 35.8dB | 38.2dB | 39dB |  |
| Source 2 | TDL-A 10ns, TM9 2T2R, Fixed MCS | 35dB |  |  |  |  |  | 42dB | Not observed | Not observed |
| Source 3 | TDL-B 10ns, 2T2R TM4,, Fixed MCS |  | 32.5dB |  |  | 35dB |  |  |  |  |
|  | TDL-B 10ns, TM4 2T8R, Fixed MCS |  | 23.3dB |  |  | 24dB |  |  |  |  |
|  | TDL-D, 100ns, TM4 2T2R,, Fixed MCS |  | 34dB |  |  | 36dB |  |  |  |  |
|  | TDL-D, 100ns, TM4 2T8R,, Fixed MCS |  | 24dB |  |  | 25dB |  |  |  |  |
| Source 4 | TDL-A 100ns, 2T2R, TM2, OLLA | 35.25dB | 37.43dB |  | Not Observed | Not Observed |  | Not Observed | Not Observed |  |
|  | TDL-A 100ns, 2T4R, TM2, OLLA | 27.75dB | 29.53dB |  | 30.11dB | 30.97dB |  | 31.67dB | 33.43dB |  |
|  | TDL-A 10ns, 2T2R, TM2, OLLA | 32.0dB | 32.75dB |  | 34.75dB | 38.16dB |  | 34.0dB | 37.62dB |  |
|  | TDL-A 10ns, 2T4R, TM2, OLLA | 27.67dB | 27.93dB |  | 29.46dB | 30.2dB |  | 31.0dB | 32.45dB |  |
|  | TDL-B 100ns, 2T2R, TM2, OLLA | 35.5dB | 38.21dB |  | Not Observed | Not Observed |  | Not Observed | Not Observed |  |
|  | TDL-B 100ns, 2T4R, TM2, OLLA | 27.8dB | 28.5dB |  | 29.86dB | 30.66dB |  | 32.0dB | 33.92dB |  |
|  | TDL-B 10ns, 2T2R, TM2, OLLA | 34.5dB | 35.95dB |  | 39.01dB | Not Observed |  | 36.0dB | Not Observed |  |
|  | TDL-B 10ns, 2T4R, TM2, OLLA | 27.4dB | 27.93dB |  | 29.86dB | 30.66dB |  | 31.25dB | 32.81dB |  |
|  | TDL-D 100ns, 2T2R, TM2, OLLA | 28.64dB | 29.36dB |  | 30.63dB | 31.62dB |  | 32.0dB | 33.92dB |  |
|  | TDL-D 100ns, 2T4R, TM2, OLLA | 22.58dB | 23.09dB |  | 23.73dB | 23.91dB |  | 24.5dB | 24.79dB |  |
|  | TDL-D 10ns, 2T2R, TM2, OLLA | 29.0dB | 29.73dB |  | 31.11dB | 32.22dB |  | 32.0dB | 33.92dB |  |
|  | TDL-D 10ns, 2T4R, TM2, OLLA | 23.0dB | 23.09dB |  | 23.2dB | 23.36dB |  | 24.5dB | 24.79dB |  |
|  | TDL-E 100ns, 2T2R, TM2, OLLA | 28.03dB | 28.48dB |  | 29.99dB | 30.82dB |  | 31.47dB | 33.14dB |  |
|  | TDL-E 100ns, 2T4R, TM2, OLLA | 22.09dB | 22.32dB |  | 22.94dB | 23.09dB |  | 23.77dB | 24.01dB |  |
|  | TDL-E 10ns, 2T2R, TM2, OLLA | 28.5dB | 29.16dB |  | 30.72dB | 31.73dB |  | 32.0dB | 33.92dB |  |
|  | TDL-E 10ns, 2T4R, TM2, OLLA | 22.11dB | 22.35dB |  | 22.96dB | 23.11dB |  | 23.76dB | 24.0dB |  |
|  | TDL-D Orth. LOS 100ns, 2T2R, TM2, OLLA | 28.43dB | 29.14dB |  | 31.24dB | 32.4dB |  | 33.12dB | 35.84dB |  |
|  | TDL-D Orth. LOS 100ns, 2T4R, TM2, OLLA | 21.75dB | 25.35dB |  | 26.96dB | 27.36dB |  | 29.29dB | 30.21dB |  |
|  | TDL-D Orth. LOS 10ns, 2T2R, TM2, OLLA | 28.43dB | 29.14dB |  | 31.26dB | 32.42dB |  | 33.14dB | 35.88dB |  |
|  | TDL-D Orth. LOS 10ns, 2T4R, TM2, OLLA | 21.6dB | 25.14dB |  | 26.96dB | 27.36dB |  | 29.29dB | 30.21dB |  |
|  | TDL-E Orth. LOS 100ns, 2T2R, TM2, OLLA | 28.75dB | 29.45dB |  | 31.27dB | 32.44dB |  | 33.17dB | 35.93dB |  |
|  | TDL-E Orth. LOS 100ns, 2T4R, TM2, OLLA | 27.06dB | 27.34dB |  | 27.08dB | 27.49dB |  | 29.22dB | 30.13dB |  |
|  | TDL-E Orth. LOS 10ns, 2T2R, TM2, OLLA | 28.6dB | 29.44dB |  | 31.26dB | 32.42dB |  | 33.17dB | 35.93dB |  |
|  | TDL-E Orth. LOS 10ns, 2T4R, TM2, OLLA | 27.03dB | 27.36dB |  | 27.08dB | 27.49dB |  | 29.22dB | 30.13dB |  |

- 5.1.2 Throughput

The evaluation results of throughput gain for 1024QAM are summarized in Table 5.1.2-1 and 5.1.2-2 for AWGN channel and TDL channels, respectively.

Table 5.1.2-1 Throughput gain for AWGN channel

| Sources | Scenarios | EVM |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0% TX, 0% RX | 1% TX, 1% RX | 1.5% TX, 1% RX | 2% TX, 2% RX | 2.5% TX, 2% RX | 3% TX, 3%RX |
| Source 1 | Fixed MCS, TM3, 2T2R, TBS=119816 | 22.4% @30dB, 22.4% @30dB | 21.39% @30dB, 22.4% @35dB | 19.0% @30dB, 22.4% @35dB | -38.8% @30dB, 22.3% @35dB | 38.8%@30dB, 22.2% @35dB | -38.8% @30dB 22.9% @35dB |
|  | Fixed MCS, TM3, 2T2R, TBS=105528 | 7.8% @30dB, 7.8% @35dB | 7.8% @30dB, 7.8% @35dB | 7.8% @30dB, 7.8% @35dB | 7.8% @30dB, 7.8% @35dB | 7.8% @30dB, 7.8% @35dB | 7.7% @30dB, 7.8% @35dB |
| Source 3Note 1 | TM4, 2T2R, Fixed MCS | 4.2%@30dB | 4.2% @30dB |  | 4.2%@31dB |  | 4.2% @34dB |
|  | TM4, 2T8R, Fixed MCS | 4.2% @24dB | 4.2%@24dB |  | 4.2%@25dB |  | 4.2%@28dB |
| Source 4 | TM3, 2T2R, Fixed MCS | 7.8%@30dB 22.2%@35dB | 7.3%@30dB 21.6%@35dB |  | 6.2%@30dB 17.5%@30dB |  |  |

Note 1: The coding rate is 0.745 for 1024QAM, and 0.9 for 256QAM.

Table 5.1.2-2 Throughput gain for TDL channels

| sources | scenarios | EVM |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0% TX, 0% RX | 1% TX, 1% RX | 1.5% TX, 1% RX | 2% Tx, 1.5%Rx | 2% TX, 2% RX | 3%TX, 0%RX | 3% TX, 1.5% RX | 3% TX, 3%RX |
| Source 1 | TDL-D, 10ns, 2T8R, TM9, 1layer,Fixed MCS ( TBS=119816) | 13.7%@30dB, 22.4%@35dB | 0.2%@30dB, 20.4%@35dB | 12.5%@35dB |  |  |  |  |  |
|  | TDL-A, 10ns, 2T8R, TM4, Fixed MCS ( TBS=119816 ) | 22.4%@30dB, 22.4%@35dB |  |  |  |  |  |  |  |
|  | TDL-C, 100ns, 2T2R, 1layer,TM9, Fixed MCS ( TBS=105528 ) | 7.7%@30dB 7.8%@35dB |  |  |  |  |  |  |  |
|  | TDL-C, 100ns, 2T2R, 1layer,TM9, Fixed MCS ( TBS=119816) | -25.1%@30dB 17.4%@35dB |  |  |  |  |  |  |  |
|  | TDL-C, 100ns, 2T4R, 1layer,TM9, Fixed MCS ( TBS=105528) | 7.8%@30dB, 7.8%@35dB |  |  |  |  |  |  |  |
|  | TDL-C, 100ns, 2T4R, 1layer,TM9, Fixed MCS ( TBS=119816 ) | 22.1%@30dB, 22.4%@35dB |  |  |  |  |  |  |  |
| Source 2 | TDL-A 10ns, TM9 2T2R, Fixed MCS | 23%@45dB |  |  |  |  | 6%@50dB | 0%@50dB | -8%@50dB |
| Source 3Note1 | TDL-B, 10ns, 2T2R, TM4, Fixed MCS |  | 4.2%@44dB |  |  | 4.2% @46dB |  |  |  |
|  | TDL-B, 10ns, 2T8R, TM4, Fixed MCS |  | 4.2%@25dB |  |  | 4.2%@26dB |  |  |  |
|  | TDL-D, 100ns, 2T2R, TM4, Fixed MCS |  | 4.2%@44dB |  |  | 4.2% @46dB |  |  |  |
|  | TDL-D, 100ns, 2T8R, TM4, Fixed MCS |  | 4.2%@25dB |  |  | 4.2%@26dB |  |  |  |
| Source 4 | TDL-A 100ns, 2T2R, TM2, OLLA | -2.0% @30dB -1.0% @35dB | -1.0% @30dB -0.0% @35dB |  | -1.0% @30dB 0.0% @35dB | -1.0% @30dB -0.0% @35dB | -1.0% @30dB -1.0% @35dB | -1.0% @30dB -0.0% @35dB |  |
|  | TDL-A 100ns, 2T4R, TM2, OLLA | 9.0% @30dB 22.0% @35dB | 4.0% @30dB 21.0% @35dB |  | -0.0% @30dB 12.0% @35dB | -2.0% @30dB 7.0% @35dB | -3.0% @30dB 7.0% @35dB | -4.0% @30dB 2.0% @35dB |  |
|  | TDL-A 10ns, 2T2R, TM2, OLLA | -2.0% @30dB 1.0% @35dB | -2.0% @30dB 2.0% @35dB |  | -2.0% @30dB 0.0% @35dB | -2.0% @30dB -2.0% @35dB | -2.0% @30dB 1.0% @35dB | -2.0% @30dB -1.0% @35dB |  |
|  | TDL-A 10ns, 2T4R, TM2, OLLA | 8.0% @30dB 20.0% @35dB | 5.0% @30dB 17.0% @35dB |  | 1.0% @30dB 11.0% @35dB | -0.0% @30dB 7.0% @35dB | -1.0% @30dB 8.0% @35dB | -2.0% @30dB 3.0% @35dB |  |
|  | TDL-B 100ns, 2T2R, TM2, OLLA | -2.0% @30dB -1.0% @35dB | -1.0% @30dB -1.0% @35dB |  | -1.0% @30dB -1.0% @35dB | -0.0% @30dB -2.0% @35dB | -1.0% @30dB -1.0% @35dB | -1.0% @30dB -2.0% @35dB |  |
|  | TDL-B 100ns, 2T4R, TM2, OLLA | 10.0% @30dB 22.0% @35dB | 6.0% @30dB 22.0% @35dB |  | 0.0% @30dB 15.0% @35dB | -2.0% @30dB 8.0% @35dB | -4.0% @30dB 8.0% @35dB | -4.0% @30dB 1.0% @35dB |  |
|  | TDL-B 10ns, 2T2R, TM2, OLLA | -2.0% @30dB 1.0% @35dB | -2.0% @30dB -1.0% @35dB |  | -2.0% @30dB -2.0% @35dB | -2.0% @30dB -3.0% @35dB | -2.0% @30dB -1.0% @35dB | -2.0% @30dB -2.0% @35dB |  |
|  | TDL-B 10ns, 2T4R, TM2, OLLA | 8.0% @30dB 22.0% @35dB | 6.0% @30dB 20.0% @35dB |  | 0.0% @30dB 11.0% @35dB | -0.0% @30dB 7.0% @35dB | -1.0% @30dB 8.0% @35dB | -2.0% @30dB 3.0% @35dB |  |
|  | TDL-D 100ns, 2T2R, TM2, OLLA | 3.0% @30dB 9.0% @35dB | 1.0% @30dB 8.0% @35dB |  | -1.0% @30dB 5.0% @35dB | -2.0% @30dB 3.0% @35dB | -2.0% @30dB 4.0% @35dB | -2.0% @30dB 1.0% @35dB |  |
|  | TDL-D 100ns, 2T4R, TM2, OLLA | 13.0% @30dB 11.0% @35dB | 13.0% @30dB 12.0% @35dB |  | 12.0% @30dB 13.0% @35dB | 11.0% @30dB 13.0% @35dB | 8.0% @30dB 12.0% @35dB | 7.0% @30dB 12.0% @35dB |  |
|  | TDL-D 10ns, 2T2R, TM2, OLLA | 2.0% @30dB 8.0% @35dB | 1.0% @30dB 7.0% @35dB |  | -0.0% @30dB 5.0% @35dB | -1.0% @30dB 3.0% @35dB | -1.0% @30dB 4.0% @35dB | -2.0% @30dB 1.0% @35dB |  |
|  | TDL-D 10ns, 2T4R, TM2, OLLA | 13.0% @30dB 11.0% @35dB | 13.0% @30dB 12.0% @35dB |  | 12.0% @30dB 13.0% @35dB | 11.0% @30dB 13.0% @35dB | 8.0% @30dB 12.0% @35dB | 6.0% @30dB 12.0% @35dB |  |
|  | TDL-E 100ns, 2T2R, TM2, OLLA | 6.0% @30dB 14.0% @35dB | 4.0% @30dB 11.0% @35dB |  | 0.0% @30dB 8.0% @35dB | -2.0% @30dB 6.0% @35dB | -3.0% @30dB 8.0% @35dB | -4.0% @30dB 3.0% @35dB |  |
|  | TDL-E 100ns, 2T4R, TM2, OLLA | 17.0% @30dB 16.0% @35dB | 16.0% @30dB 17.0% @35dB |  | 15.0% @30dB 15.0% @35dB | 15.0% @30dB 15.0% @35dB | 12.0% @30dB 16.0% @35dB | 8.0% @30dB 15.0% @35dB |  |
|  | TDL-E 10ns, 2T2R, TM2, OLLA | 3.0% @30dB 13.0% @35dB | 2.0% @30dB 9.0% @35dB |  | -2.0% @30dB 6.0% @35dB | -6.0% @30dB 4.0% @35dB | -7.0% @30dB 7.0% @35dB | -12.0% @30dB 2.0% @35dB |  |
|  | TDL-E 10ns, 2T4R, TM2, OLLA | 17.0% @30dB 16.0% @35dB | 16.0% @30dB 16.0% @35dB |  | 16.0% @30dB 16.0% @35dB | 15.0% @30dB 16.0% @35dB | 13.0% @30dB 15.0% @35dB | 9.0% @30dB 14.0% @35dB |  |
|  | TDL-D Orth. LOS 100ns, 2T2R, TM2, OLLA | 10.0% @30dB 22.0% @35dB | 4.0% @30dB 22.0% @35dB |  | -2.0% @30dB 17.0% @35dB | -3.0% @30dB 8.0% @35dB | -3.0% @30dB 11.0% @35dB | -4.0% @30dB -1.0% @35dB |  |
|  | TDL-D Orth. LOS 100ns, 2T4R, TM2, OLLA | 22.0% @30dB 22.0% @35dB | 21.0% @30dB 22.0% @35dB |  | 8.0% @30dB 22.0% @35dB | 6.0% @30dB 21.0% @35dB | 3.0% @30dB 20.0% @35dB | -1.0% @30dB 7.0% @35dB |  |
|  | TDL-D Orth. LOS 10ns, 2T2R, TM2, OLLA | 9.0% @30dB 22.0% @35dB | 4.0% @30dB 22.0% @35dB |  | -2.0% @30dB 17.0% @35dB | -3.0% @30dB 8.0% @35dB | -3.0% @30dB 10.0% @35dB | -4.0% @30dB -1.0% @35dB |  |
|  | TDL-D Orth. LOS 10ns, 2T4R, TM2, OLLA | 22.0% @30dB 22.0% @35dB | 21.0% @30dB 22.0% @35dB |  | 8.0% @30dB 22.0% @35dB | 6.0% @30dB 21.0% @35dB | 3.0% @30dB 20.0% @35dB | -1.0% @30dB 7.0% @35dB |  |
|  | TDL-E Orth. LOS 100ns, 2T2R, TM2, OLLA | 19.0% @30dB 22.0% @35dB | 8.0% @30dB 22.0% @35dB |  | -2.0% @30dB 21.0% @35dB | -2.0% @30dB 14.0% @35dB | -2.0% @30dB 19.0% @35dB | -2.0% @30dB -1.0% @35dB |  |
|  | TDL-E Orth. LOS 100ns, 2T4R, TM2, OLLA | 22.0% @30dB 22.0% @35dB | 21.0% @30dB 22.0% @35dB |  | 7.0% @30dB 22.0% @35dB | 5.0% @30dB 22.0% @35dB | 4.0% @30dB 20.0% @35dB | -1.0% @30dB 6.0% @35dB |  |
|  | TDL-E Orth. LOS 10ns, 2T2R, TM2, OLLA | 19.0% @30dB 22.0% @35dB | 8.0% @30dB 22.0% @35dB |  | -2.0% @30dB 21.0% @35dB | -2.0% @30dB 15.0% @35dB | -2.0% @30dB 19.0% @35dB | -2.0% @30dB -1.0% @35dB |  |
|  | TDL-E Orth. LOS 10ns, 2T4R, TM2, OLLA | 22.0% @30dB 22.0% @35dB | 21.0% @30dB 22.0% @35dB |  | 7.0% @30dB 22.0% @35dB | 5.0% @30dB 21.0% @35dB | 4.0% @30dB 20.0% @35dB | -1.0% @30dB 6.0% @35dB |  |

Note 1: The coding rate is 0.745 for 1024QAM, and 0.9 for 256QAM.

-----------------------------------------------------TP ends------------------------------------------------------------