3GPP TSG RAN WG1 Meeting #89                                         R1-1709868

Hangzhou, China, 15th  – 19th May 2017

Agenda Item:	7.1.1.2.1

Source: 	Ericsson

Title: 	Summary of email discussion [89-15] Details for NR-PBCH evaluation

Document for:	   Discussion

# Background

The following email discussion was initiated at RAN1#89:

Agreements: - Down select from following alternatives based on further evaluation/analysis in the next meeting - Alt. 1: NR-PBCH coded bits are mapped across REs in N PBCH symbols, where N is the number of PBCH symbols in a NR-SS block - Alt. 2: NR-PBCH coded bits are mapped across REs in a PBCH symbol, the NR-PBCH symbol is copied to N-1 NR-PBCH symbol in a NR-SS block - Other Alternatives are not precluded - Note that all proponents need to provide their own proposal until 26th May - For evaluation purposes, followings should be considered - Channel coding - Rate matching - Accuracy of CFO estimation - DMRS RE mapping - Channel estimation performance - NR-PBCH one to four shot(s) performance within 80 msec - Complexity of NR-PSS/SSS/NR-PBCH decoding and mobility measurement - Reliability of time index - All proponents need to provide followings until 2nd June to achieve further evaluation/analysis – Asbjorn (Ericsson) - NR-PBCH RE mapping - SS-block composition - SS-block time index indication - SFN indication - DMRS RE mapping - PBCH payload size - PBCH channel coding scheme (all proponents need to follow the latest agreements/WAs in channel coding session) - Note that all decisions of channel coding scheme should be done in channel coding session/agenda - Receiver algorithms

This is the summary of the discussion

# Proposals for NR-PBCH RE mapping

Fill in the details for the alternative NR-PBCH mapping.

| vivo | We propose a revised repetition solution based on Alt2, namely Alt3 below: - Alt. 3: NR-PBCH coded bits are mapped across REs in N PBCH symbols within the bandwidth of PSS/SSS, and the NR-PBCH band within the bandwidth of PSS/SSS is copied to other NR-PBCH band in a NR-SS block. With Alt.3, the minimum bandwidth required to support NR operation can be potentially reduced from both network and terminal perspective. |
| --- | --- |

# Proposed design for NR-PBCH

Fill in the details for the proposed design for NR-PBCH.

| Company 1 (example) | NR-PBCH RE mapping <Alternative from section 2> SS-block composition <e.g. PSS-PBCH-SSS-PBCH> SS-block time index indication <Implicit/explicit> SFN indication <Implicit/explicit> DMRS sequence and RE mapping <Even/uneven mapping, density etc. > PBCH payload size <Number of bits> PBCH channel coding scheme <Details of the channel coding scheme> Receiver algorithms <Details of the receiver algorithms > |
| --- | --- |
| CATT | NR-PBCH RE mapping Our preference is “Alt. 1: NR-PBCH coded bits are mapped across REs in N PBCH symbols, where N is the number of PBCH symbols in a NR-SS block”. NR-PBCH coded bits mapped across REs of two OFDM symbols would allow to use the Polar code base code rate 1/12 for NR-PBCH encoding comparing to that of code rate 1/6 for Alt. 2. It has been shown that the performance gain of 0.2 dB for Alt 1 base code rate 1/12 comparing to Alt 2 with code rate 1/6 with repetition. Frequency first mapping with frequency domain interleaving SS-block composition PSS-PBCH-PBCH-SSS SS-block time index indication Explicit in the PBCH payload SFN indication Full length of SFN in the PBCH payload DMRS sequence and RE mapping Zadoff-Chu sequence with density 1/3 with distance of 3 REs among DMRS PBCH payload size 70 bits PBCH channel coding scheme Hash-Polar or CC-Polar with 16-bit CRC and 6-bit hash bits Receiver algorithms Hash Polar decoder |
| Sony | NR-PBCH RE mapping Alt. 1: NR-PBCH coded bits are mapped across REs in N PBCH symbols, where N is the number of PBCH symbols in a NR-SS block. Constellation: QPSK Mapping: Frequency dimension then time SS-block composition PSS-SSS-PBCH-PBCH but we are also OK with PSS-PBCH-SSS-PBCH – pending evidence on which of the two is best. We will assess both in simulation. SS-block time index indication Implicit – carry a Gold Sequence on PBCH-DMRS indicative of the SS-block time index. SFN indication Part of SFN in the PBCH payload DMRS sequence and RE mapping DMRS Sequence: Gold sequence chosen according to SS block index and scrambled by Cell ID RE Mapping: - For block composition PSS-SSS-PBCH-PBCH: DMRS density - On non-SSS overlapping RBs: 1:3 - On overlapping RBs - Symbol 1: none - Symbol 2: 1:6 Number of DMRS: 129 - For block composition PSS-PBCH-SSS-PBCH: DMRS density - On non-SSS overlapping RBs: 1:3 - On overlapping RBs - Symbol 1: none - Symbol 2: none Number of DMRS: 108 In addition, DMRS RE is shifted in frequency domain according to cell ID. PBCH payload size Number of REs remaining for DMRS payload in each case: - For block composition PSS-SSS-PBCH-PBCH: 288 – 129 = 159 - For block composition PSS-PBCH-SSS-PBCH: 288 – 108 = 180 Rate match to whatever payload size to the available resources. Actual payload size between 40 – 72 bits. PBCH channel coding scheme Polar code when the details of the code are available. We will use TBCC with 16-bit CRC for simulation now. Receiver algorithms Channel estimation: MMSE Viterbi decoder (for TBCC) |
| vivo | NR-PBCH RE mapping Alt. 3 SS-block composition PSS-SSS-PBCH-PBCH SS-block time index indication Implicit indication by PBCH DMRS Phase Rotation* or combined with part explicit indication in PBCH payload. *: PBCH DMRS Phase Rotation is similar to PUCCH format 2b in LTE Depend on the maximum number (L) of SS blocks within SS burst set: - For frequency range up to 3 GHz (L=4): Implicit indication by PBCH DMRS Phase Rotation - For frequency range from 3 GHz to 6 GHz (L=8): Implicit indication by PBCH DMRS Phase Rotation - For frequency range from 6 GHz to 52.6 GHz (L=64): 3bits implicit indication by PBCH DMRS Phase Rotation combined with 3bits explicit indication in PBCH payload. SFN indication Part of SFN is explicitly indicated in PBCH payload, e.g. HSBs of SFN. Implicit indication for LSBs of SFN to support PBCH combining within PBCH TTI. DMRS sequence and RE mapping PN sequence with 1/3 density, DMRS sequence is mapped on subcarriers with equal interval PBCH payload size 64bits PBCH channel coding scheme Polar code Receiver algorithms MMSE2D for channel estimation; Joint detection of 2 copies for Alt. 3; SCL decoding. |
| Nokia | NR-PBCH RE mapping Alt. 1: NR-PBCH coded bits are mapped across REs in N PBCH symbols, where N is the number of PBCH symbols in a NR-SS block Frequency first mapping SS-block composition PSS-PBCH-SSS-PBCH SS-block time index indication We consider explicit indication as a part of the PBCH payload given payload and performance (one-shot) targets are hold/met. If explicit does not provide sufficient performance (one-shot) and/or there is need to reduce the payload, we should consider providing SS block time indices e.g. through DMRS sequence initialization. SFN indication To support combining across the SS burst sets (in the 80ms TTI), we propose to use PBCH scrambling (similar to LTE) to deliver the 3 LSBs of the SFN. DMRS sequence and RE mapping Pseudo-random sequence initialized at least based on PCID. DMRS sequence is mapped on subcarriers with equal interval using 4/12 density. PBCH payload size No more than 72 bits (including CRC), 64 bits maximum preferred. Meeting one-shot demodulation performance target may become an issue with payloads in the higher end of the payload range [48, 72] bits. PBCH channel coding scheme Construction: Polar coding with 16 CRC bits appended. Sequence: Polarization weight (PW) scheme Rate matching: bit reversal shortening and repetition proposed in R1-1708835 Receiver algorithms Channel estimation: realistic concatenated freq. and time 1 D wiener filter, SNR estimated. Decoder algorithm: CRC assisted successive cancellation list -8 decoder. |
| ITL | NR-PBCH RE mapping Our preference is Alt 1. Constellation: QPSK Mapping order: Frequency first then time SS-block composition <e.g. PSS-PBCH-SSS-PBCH> SS-block time index indication <Implicit/explicit> SFN indication <Implicit/explicit> DMRS sequence and RE mapping <Even/uneven mapping, density etc. > PBCH payload size <Number of bits> PBCH channel coding scheme <Details of the channel coding scheme> Receiver algorithms <Details of the receiver algorithms > |
| MediaTek | NR-PBCH RE mapping Alt. 1: NR-PBCH coded bits are mapped across REs in N PBCH symbols, where N is the number of PBCH symbols in a NR-SS block QPSK, and frequency first mapping SS-block composition PBCH-PSS-SSS-PBCH SS-block time index indication Explicit in the PBCH payload Up to 7 bits. SFN indication Full length of SFN (10bits) in the PBCH payload DMRS sequence and RE mapping Gold sequence for DMRS; Even mapping of DMRS; 2/12, 3/12 and 4/12 densities are under evaluation for different channels. PBCH payload size Info bits: 38-bit static data + (3-bit varying SFN + 7-bit varying time index) CRC: 19-bits => Total: 67 bits PBCH channel coding scheme Baseline CRC assisted Polar (with 16 + 6 CRC) Receiver algorithms SCL-list8 decoding |
| Motorola Mobility, Lenovo | NR-PBCH RE mapping Coded bits corresponding to at least SS block time index are mapped to REs within the PSS/SSS bandwidth in N PBCH symbols, where N is the number of PBCH symbols in a NR-SS block. Coded bits corresponding to at least remaining NR-PBCH contents are mapped across remaining REs in the N PBCH symbols. Whether single codeword or two codewords for NR-PBCH is FFS. UE can operate with bandwidth same as the PSS/SSS bandwidth for determining SS block time index during inter-frequency neighbour cell measurement. SS-block composition PSS-SSS-PBCH-PBCH PSS is close to SSS for coherent detection of SSS. SS-block time index indication Explicit 6 bits for a SS-block index within a SS burst set are included in PBCH payload. Half-frame timing information is indicated via SSS sequences. SFN indication Explicit 7 bits are included in PBCH payload. Radio frame timing within 80ms PBCH TTI is indicated implicitly via scrambling sequences to support combining across the SS burst sets and interference randomization. DMRS sequence and RE mapping DMRS sequence is mapped evenly on every 6th subcarriers over the entire PBCH bandwidth, and PBCH DMRS in the second PBCH symbol is staggered by 3 subcarriers. For DMRS sequence, the pseudo-random sequence generator is initialized based on physical cell ID (not dependent on SS block time index). PBCH payload size Approximately 60 bits including CRC PBCH channel coding scheme Polar code (Details on code construction is FFS) Receiver algorithms UE can use SSS and PBCH DM RS for fine frequency tracking (48 CFO estimate samples per SS block). |
| Samsung | NR-PBCH RE mapping Alt. 1 with frequency first mapping SS-block composition PSS-SSS-PBCH-PBCH SS-block time index indication 1st preference: DMRS to indicate SS block indices in order to avoid additional latency and power consumption of reading PBCH payload. 2nd preference: DMRS to indicate partial SS block indices and offload remaining SS block indices in explicit bits of PBCH payload, if this design can satisfy the reliability and latency issues addressed by LS R2-1706160. SFN indication 3-bit LSB of SFN implicitly indicated by using scrambling and 7-bit MSB of SFN explicitly indicated in MIB DMRS sequence and RE mapping Even mapping on subcarriers with equal interval Gold-sequence-based DMRS PBCH payload size FFS in the range of 48~72 PBCH channel coding scheme CA-polar with 19-bit CRC (detailed design up to coding session) (TBCC would be used for simulation comparison of timing indication schemes only) Receiver algorithms CA-polar decoder (detailed design up to coding session) |
| Huawei, HiSilicon | NR-PBCH RE mapping Alt. 1: NR-PBCH coded bits are mapped across REs in N PBCH symbols, where N is the number of PBCH symbols in a NR-SS block. SS-block composition Option 1: The mapping order of SS blocks is PSS-SSS-PBCH-PBCH. SS-block time index indication Methods based on explicit and/or implicit indications are studied and proposed. For the case of combined explicit and implicit indications method: - Implicit 2-bit is used for SS block indication within an SS burst. Cyclic shift of Polar codewords implicit technique is used. (Details of implicit indication schemes can be found in R1-1708158.) - Explicit 4-bit is used for SS burst indication within an SS burst set. Depending on the SCS and the total SS blocks that need to be indicated, not all the explicit 4 bits need to be used. Some explicit bits may be reserved. SFN indication 8 bits MSB of SFN explicitly indicated in MIB with 7 bits for explicit SFN with 80ms granularity and 1 bit for 10ms granularity. The SS burst set with 20ms granularity is indicated implicitly using 4 scrambling sequences. For the half frame (5ms) indication, the used indications from the SS Burst Set periodicities are used. An additional 1 bit for explicit SFN is used to indicate the half frame timing. DMRS sequence and RE mapping DMRS sequence is Gold sequence based on PCID. Different sequence and resource mapping per cell is assumed. For the DMRS RE mapping, Option 2 is proposed whereby DMRS sequence is mapped on subcarriers with unequal interval (e.g., less or no mapping within NR-SSS transmission bandwidth). The DMRS RE density is FFS. PBCH payload size PBCH payload size: FFS in the range of 48~72 PBCH channel coding scheme PC-CA polar (R1-1706969) with the assistant of parity check bits and CRC bits is adopted. Receiver algorithms MMSE with channel filtering and Polar list decoding as described in R1-1706969. |
| Qualcomm | NR-PBCH RE mapping Alt. 2: NR-PBCH coded bits are mapped across REs in a PBCH symbol, the NR-PBCH symbol is copied to N-1 NR-PBCH symbol in a NR-SS block Frequency first mapping SS-block composition PSS-PBCH-SSS-PBCH SS-block time index indication Explicit as PBCH payload SFN indication Every bit except the third and second LSB of SFN (s2, s1) are carried explicitly as PBCH payload. For clarification, s2 and s1 are the bits that indicate the 0ms, 20ms, 40ms, 60ms boundary within 80ms s2, s1 are carried by PBCH data scrambling, FFS (part of) s2 and s1 can be carried by PBCH DMRS DMRS sequence and RE mapping Gold sequence For the PBCH part not overlapping with PSS/SSS: Density 1/4 For the PBCH part overlapping with PSS/SSS: DMRS density can be sparser PBCH payload size 48 PBCH channel coding scheme polar code (FRANK with block puncturing ) Receiver algorithms CRC-Aided Successive Cancellation List decoding (CA-SCL) with list size = 8 |
| ZTE | NR-PBCH RE mapping Alt. 1. SS-block composition PSS-PBCH-SSS-PBCH SS-block time index indication A combination of PBCH-DMRS, PBCH-implicit and/or PBCH-explicit. Solution depends on the maximum number of SS blocks per SS burst set (4, 8 or 64) and on the number of indices that can be carried on PBCH-DMRS. SFN indication A combination of PBCH-explicit, PBCH-DMRS and/or PBCH-implicit. Solution depends on periodicity of SS burst set, maximum number of SS blocks per SS burst set (4, 8 or 64) and on the number of indices that can be carried on PBCH-DMRS. DMRS sequence and RE mapping Uneven mapping with no/reduced DMRS within SSS bandwidth. 1/4 DMRS density outside SSS bandwidth. PBCH payload size 48 bits including CRC PBCH channel coding scheme Polar coding, details to be decided in channel coding session. Receiver algorithms Polar decoding algorithm (e.g. SCL-decoding) |
| InterDigital | NR-PBCH RE mapping Alt. 2: NR-PBCH coded bits are mapped across REs in a PBCH symbol, the NR-PBCH symbol is copied to N-1 NR-PBCH symbol in a NR-SS block. Constellation: QPSK Frequency first mapping SS-block composition PSS-SSS-PBCH-PBCH. PSS- PBCH- SSS-PBCH can also be considered. SS-block time index indication Combined explicit and implicit indication SFN indication Hybrid indication (combined implicit and explicit indication) DMRS sequence and RE mapping Gold (or m) sequence DMRS Density 1/3 or 1/4 preferred Uneven mapping preferred PBCH payload size [48, 72] bits including CRC PBCH channel coding scheme PC/CA Polar Codes. Code word of Length 512. Receiver algorithms - CFO Estimation using repeated symbol. - 2D-MMSE Channel Estimation. - MMSE detection of data at two receive antennas - Joint constellation demapping and polar decoding |
| Sierra Wireless | NR-PBCH RE mapping Alt. 1: NR-PBCH coded bits are mapped across REs in N PBCH symbols, where N is the number of PBCH symbols in a NR-SS block SS-block composition PSS-SSS-PBCH-PBCH SS-block time index indication Explicit indication by NR-PBCH payload SFN indication MSB part of the SFN is explicit (in the NR-PBCH payload), and LSB part of the SFN is implicit. DMRS sequence and RE mapping DMRS sequence is mapped on subcarriers with unequal interval: No DMRS mapped within the NR-SSS transmission bandwidth, and equal interval mapping outside the NR-SSS bandwidth. Zadoff-Chu sequence for DMRS. PBCH payload size <Number of bits> PBCH channel coding scheme <Details of the channel coding scheme> Receiver algorithms <Details of the receiver algorithms > |
| NTT DOCOMO | NR-PBCH RE mapping Alt. 1: NR-PBCH coded bits are mapped across REs in N PBCH symbols, where N is the number of PBCH symbols in a NR-SS block. Frequency first mapping and QPSK are applied. SS-block composition In case that number of PBCH symbols is two within a SS block, Option 2: The mapping order of SS blocks is PSS-PBCH-SSS-PBCH. For 6-52.6 GHz frequency range, number of PBCH symbols can be three within a SS block if (at least part of) SS block index is explicitly indicated in NR-PBCH and short delay for PBCH reading is really necessary as argued in RAN2 LS (R2-1706160). The mapping order of SS blocks in such case is PSS-PBCH-SSS-PBCH-PBCH. SS-block time index indication For below 6 GHz frequency range, SS block index is implicitly indicated by using PBCH-DMRS sequence. For 6-52.6 GHz frequency range, a part of SS block index is implicitly indicated by using PBCH-DMRS sequence, and remaining part of SS block index is explicitly indicated in NR-PBCH. SFN indication 3 bits of SFN (within 80 ms) are implicitly indicated, and remaining 7 bits of SFN are explicitly indicated in NR-PBCH. Additional explicit 1 bit in NR-PBCH is used for half radio frame timing indication. DMRS sequence and RE mapping Option 1: DMRS sequence is mapped on subcarriers with equal interval. DMRS RE density is 3 RE per PRB in each PBCH symbol. At least for evaluation purpose, LTE pseudo random sequence generator based on Gold sequence is assumed for DMRS sequence generation. PBCH payload size FFS in the range of [48-64]. Different payload size of NR-PBCH for below-6 GHz frequency range and for 6-52.6 GHz frequency range can be considered. PBCH channel coding scheme DSPC-Polar codes with 16-bits CRC and 3-bit distributed simple parity check bits Receiver algorithms MMSE 2D channel estimation DSPC-Polar codes with list decoding |
| Intel | NR-PBCH RE mapping Alt. 1: NR-PBCH coded bits are mapped across REs in N PBCH symbols, where N is the number of PBCH symbols in a NR-SS block. SS-block composition PSS-PBCH-SSS-PBCH SS-block time index indication Implicit mapping of SS block time index further complicates PBCH decoding, and therefore is not preferred. Selection between DM-RS based SS block time index indication and Explicit indication in PBCH payload will depend on further evaluations. (Intel plans to evaluate and provide input by RAN1 NR 2017 June Ad-hoc meeting). SFN indication Similarly to SS block time index indication, Intel plans to evaluate and provide input by RAN1 NR 2017 June Ad-hoc meeting. At the moment, FFS. DMRS sequence and RE mapping Uniform mapping of DM-RS REs across the PBCH. Intel is evaluating the exact density of the DM-RS and we will provide input by RAN1 NR 2017 June Ad-hoc meeting. DMRS sequence should be aligned with DMRS sequence of NR PDSCH. The exact sequence details are FFS. PBCH payload size The actual payload size will depend on RAN2 response and details of the CORESET configuration parameters for RSMI. Our guess and estimate puts PBCH payload size in the ranges of 48 ~ 64 bits. PBCH channel coding scheme As per agreement, Polar coding construction as used for downlink control channel with Nmax = 512. Note the following is the agreement from RAN1 #89bis Agreement: - Polar coding is adopted for NR-PBCH - Using same polar code construction as for the control channel - Nmax = 512 - Working assumption that the data, including time index if carried by NR-PBCH excluding DMRS, is transmitted explicitly - Can be revisited if significant benefit is shown from partial implicit transmission of time index if allowed by the polar code design Receiver algorithms List 8 SC decoding of Polar code. |
| LG Electronics | NR-PBCH RE mapping Alt. 1: Constellation: QPSK Frequency first mapping SS-block composition PBCH-PSS-SSS-PBCH SS-block time index indication Alt1. PBCH DMRS based SS block time index indication Alt2. Combination of PBCH DMRS based indication and Explicit indication in PBCH payload SFN indication Implicit 3bits with explicit 7bits DMRS sequence and RE mapping Gold sequence (Cell-ID, SS Block index) Even mapping of DMRS REs across the PBCH 1/3 density PBCH payload size 40~64bits PBCH channel coding scheme Polar coding Receiver algorithms |
| Ericsson | NR-PBCH RE mapping Alt 2. Note that when being transmitted, different cell-specific scrambling is applied on the different NR-PBCH symbols in a NR-SS block. SS-block composition PSS-PBCH-SSS-PBCH SS-block time index indication Explicit + implicit. The exact number of bits for each depends on evaluation. The implicit bits are delivered by the phase relationship between the two NR-PBCH symbols. SFN indication Explicit including half radio frame. DMRS sequence and RE mapping A pseudo-random cell-specific sequence allocated in the even spaced DMRS REs with 4 REs/OS/PRB PBCH payload size 50 bits PBCH channel coding scheme Polar code Receiver algorithms Realistic channel estimation |
| Sequans | SS-block time index indication Implicit indication by PBCH with time-related scrambling sequences. The following figure depicts explicit (option a) versus implicit (options b & c): We further propose to scramble only the most reliable data bits with the time related scrambling sequence to gain less FAR increase due to multiple CRC checks and more reliable soft combining. Options b & c are utilizing a longer time-stamp sequence (with longer Hamming distance between different timing sequences) scrambled with the more reliable bits to mitigate the chance of detecting a wrong SS time index and thus increase its reliability. Receiver algorithms Realistic channel estimation. Possible schemes for the decoding process giving the implicit indication: Decoding of a single block: Soft combination among multiple blocks: Where Z is the encoded scrambling sequence S before the encoder. |