3GPP TSG RAN WG1 Meeting #89	R1-1707076

Hangzhou, P.R. China 15th – 19th May 2017

Source:	Ericsson

Title:	Performance Comparison of Rate Matching Schemes for Polar Codes

Agenda Item:	7.1.4.2.1.2

Document for:	Discussion and Decision

# Introduction

In RAN1#88 meeting, an agreement was reached to adopt Polar codes for both UL and DL control channels in NR (except for very small block lengths where repetition/simplex/Reed-Mueller block codes are used). Rate matching methods are needed to attain high block length granularity for NR control channels.

This contribution compares the performance of several different puncturing-based rate matching methods. All methods considered here satisfy the nest puncturing property and the reciprocal property described in [1]. Each method consists of a Type- I and a Type-II variant (see [1]), each corresponding to one of the two ends of the code block that the first punctured bit belongs.

# Puncturing Schemes

Let M denote the block length (after puncturing), and let N=2log2(M) deonte the block length of mother Polar code.  Also let p=N-M be the number of punctured bits.

The following puncturing methods are evaluated in this contribution:

- “Natural – Type I”:

  - Puncture code bits with indices {0,1,2,⋯,N-M-1}, i.e. the first (N-M) bit positions.

- “Natural – Type II”:

  - Puncture code bits with indices M,M+1,⋯,N-1, i.e. the last (N-M) bit positions.

- “Bit-reversed – Type I”:

  - Puncture code bits with indices given by bit reversing {0,1,2,⋯,N-M-1}.

- “Bit-reversed – Type II”:

  - Puncture code bits with indices given by bit reversing M,M+1,⋯,N-1, as described in [2].

- “Split-natural – Type I”:

  - Puncture the first N/4 bits naturally from bit index 0, and additional bits if needed are punctured alternately from N/4+1 and N/2+1, as described in [3].  More precisely, the set of punctured bit indices BpN with p punctured bits is given by:

BpN={0,1,2,⋯,p-1}for 0≤p≤N40,1,2,⋯,N4+p-N42-1∪N2,N2+1, ⋯,N2+p-N42-1for  N4<p≤3N4

- “Split-natural – Type II”:

  - Puncture the last N/4 bits in reverse order from bit index N and additional bits if needed are punctured alternately in reverse order from 3N/4 and N/2, complementary puncturing patterns of “Split-natural – Type I” generated as described in Section 2.1.3 of [1]. More precisely, the set of punctured bit indices BpN with p punctured bits is given by:

BpN={N-p,N-p+1,⋯,N-1}for 0≤p≤N43N4-p-N42,⋯,N-1∪N2-p-N42,⋯,N2-1for  N4<p≤3N4

- “Reliability- Type I”:

  - Use the bit ordering for selecting the frozen bit positions for puncturing [1] ,i.e. puncture the indices corresponding to the least reliable bit-channel positions first.

- “Reliability- Type II”:

  - Use the bit ordering for selecting the information bit positions for puncturing [1], i.e. puncture the indices corresponding to the most reliable bit-channel positions first.

# Simulations

## Settings

In this section, we present numerical results on the performance of different puncturing schemes.  Simulations were performed on AWGN channels with noise variances derived from the specified Es/No.  A standard Successive Cancellation List (SCL) decoder is used for all simulations based on the following parameters:

- CA-Polar with nCRC=16+3 bit CRC.

- The decoder uses the 19 CRC bits to select the best code-word from the final list.

- List sizes, L = 8.

- Norminal code rates, R∈{1/6, 1/3, 1/2, 2/3}, where R=K/M, where K is the number of data bits excluding CRC, which is round to the nearest integer if necessary.

- Decoder code rate:  RDEC=(K+nCRC)/M

- CRC polynomials is given by [4]:

D19+D16+D14+D13+D12+D10+D8+D7+D4+D4+1.

For Polar code construction, the Q-sequence method described in [1] is used for generating the information and frozen bit positions.

## Results

Figure 1– Figure 24 compare the performance of various puncturing schemes at different sample block lengths at normial code rates of 1/6, 1/3, 1/2, and 2/3.  Most of the puncturing methods perform similarly in most of the code block lengths except when M∈N2,3N4, where N=2log2M .  At those block lengths, since relatively large amount of puncturing is applied, performance difference among different methods becomes more significant.

In general, Type-I puncturing performs better than Type-II puncturing at low code rates while Type-II puncturing performs better at high code rates.  The breakeven point is around R=1/3.

- Type-I puncturing performs better than Type-II puncturing at low code rates while Type-II puncturing performs better at high code rates.  The breakeven point is around R=1/3.

Type-II puncturing typically provides more stable performance while Type-II puncturing of some methods break down at block lengths close to and slightly above N/2.  The BLER curves of Natural Type-I puncturing and Bit-Reversed Type-I puncturing can become much flatter at block lengths approaching N/2, causing more than 2 dB degradation at 1% BLER compared to other methods, as shown in Figure 13 (M=267), Figure 15 (M=282), and Figure 19 (M=330).

Among all 4 puncturing methods, Split-Natural and Reliabilty-based puncturing methods generally provide more stable performance than Natrual and Bit-Reversed methods. The best performance can be achieved if Type-I puncturing is used at low code rates while Type-II puncturing is used at high code rates, with either Split-Natural or Reliabilty-based method.  Using a single puncturing method facilitate a circular or linear buffer. implementation.

- Split-Natural and Reliabilty-based puncturing generally provide more stable performance (with smaller loss compared to the best scheme) than Natural and Bit-Reversed Puncturing.

- The performance of Natural Type-I puncturing and Bit-Reversed Type-I puncturing can break down when block length M approaches N/2 from larger M values.

- Adopt either Split-Natural or Reliabilty-based puncturing method for NR control channels with Type I and Type II puncturing applied at low rate and high rate regions, respectively.

Figure 1. Performance of Type-I Puncturing Methods with block length M=72

Figure 2. Performance of Type-II Puncturing Methods with block length M=72

Figure 3. Performance of Type-I Puncturing Methods with block length M=80

Figure 4. Performance of Type-II Puncturing Methods with block length M=80

Figure 5. Performance of Type-I Puncturing Methods with block length M=96

Figure 6. Performance of Type-II Puncturing Methods with block length M=96

Figure 7. Performance of Type-I Puncturing Methods with block length M=112

Figure 8. Performance of Type-II Puncturing Methods with block length M=112

Figure 9. Performance of Type-I Puncturing Methods with block length M=160

Figure 10. Performance of Type-II Puncturing Methods with block length M=160

Figure 11. Performance of Type-I Puncturing Methods with block length M=192

Figure 12. Performance of Type-II Puncturing Methods with block length M=192

Figure 13. Performance of Type-I Puncturing Methods with block length M=267

Figure 14. Performance of Type-II Puncturing Methods with block length M=267

Figure 15. Performance of Type-I Puncturing Methods with block length M=282

Figure 16. Performance of Type-II Puncturing Methods with block length M=282

Figure 17. Performance of Type-I Puncturing Methods with block length M=320

Figure 18. Performance of Type-II Puncturing Methods with block length M=320

Figure 19. Performance of Type-I Puncturing Methods with block length M=330

Figure 20. Performance of Type-II Puncturing Methods with block length M=330

Figure 21. Performance of Type-I Puncturing Methods with block length M=384

Figure 22. Performance of Type-II Puncturing Methods with block length M=384

Figure 23. Performance of Type-I Puncturing Methods with block length M=448

Figure 24. Performance of Type-II Puncturing Methods with block length M=448

# Conclusions

In this contribution we made the following observations:

- Type-I puncturing performs better than Type-II puncturing at low code rates while Type-II puncturing performs better at high code rates.  The break-even point is around R=1/3.

- Split-Natural and Reliability-based puncturing generally provide more stable performance (with smaller loss compared to the best scheme) than Natural and Bit-Reversed Puncturing.

- The performance of Natural Type-I puncturing and Bit-Reversed Type-I puncturing can break down when block length M approaches N/2 from larger M values.

- Adopt either Split-Natural or Reliability-based puncturing method for NR control channels with Type I and Type II puncturing applied at low rate and high rate region respectively.

# References

R1-1707075, “Rate Matching Schemes for Polar Codes,” Ericsson, Hangzhou, P.R. China 15th – 19th May 2017.

R1-1611254, “Details of the Polar code design”, Huawei, HiSilicon, Reno, USA, November 10th – 14th, 2016

R1-167533, “Examination of NR Coding Candidate for Low-Rate Applications”, MediaTek, Gothenburg, Sweden, August 22nd-26th, 2016

Philip Koopman, “CRC Hamming Weight Data, https://users.ece.cmu.edu/~koopman/crc/hw_data.html.

ETSI TS 136 212 v12.2.0 (2014-10) LTE Release 12, http://www.etsi.org/deliver/etsi_ts/136200_136299/136212/12.02.00_60/ts_136212v120200p.pdf