3GPP TSG RAN WG1 Meeting #89                                                                           R1-1707124

Hangzhou, China, 15th - 19th May 2017

# Source: 	ZTE

Title:	Details of CSI acquisition

Agenda Item:	7.1.2.3.2

Document for:     Discussion and Decision

- Introduction

In RAN1#88bis, the following agreement is made on interference measurement schemes [1].

# Agreements:

- For interference measurement, down selection from options will be conducted.

  - NZP CSI-RS based

    - Opt. A1: Estimation on NZP CSI-RS for channel estimation (by subtracting CSI-RS from Rx signal)

    - Opt. A2: Emulation on NZP CSI-RS which is represented by multiplied value of channel and precoding matrix

  - DM-RS based

    - Opt. B1: Estimation on DM-RS for own data demodulation (by subtracting DM-RS from Rx signal)

    - Opt. B2: Estimation on DM-RS for other UEs

- Criteria for design and down selection are as follows.

  - Required RS densities

  - UE processing latency

  - Support of self-contained CSI reporting (if supported) at least depends on the location of IMR.

- FFS: Whether the emulation is performed at TRP side or UE side

- FFS: RAN1 specification impact, if any, on the options above

In this contribution, we discuss some specific schemes on channel and interference measurement.

- Discussion on channel and interference measurement

2.1 Interference measurement

Channel and interference measurement for CSI feedback has been discussed in RAN1 for quite long time. The state-of-art channel measurement is based on NZP CSI-RS. On the other hand, several approaches have been discussed in NR on interference measurement. One approach is the legacy IM approach used in LTE, i.e., IM based on ZP CSI-RS. However, some concerns on ZP CSI-RS based IM have been pointed out in [2]-[3], especially for MU-CSI. Specifically, as the real co-scheduled MU paired users may be different with the MU assumption UE makes for CSI calculation, inaccurate MU interference calculation occurs.

For NZP CSI-RS based, the following two options have been identified.

Opt A1: Estimation on NZP CSI-RS for channel estimation (by subtracting CSI-RS from Rx signal)

In this option, NZP CSI-RS is still used for desired channel estimation. After performing channel estimation, interference can be measured by subtracting CSI-RS with estimated channel from Rx signal. In this option, NZP CSI-RS is also included in IMR. However, UE’s behavior, e.g., subtracting CSI-RS with desired channel, is not necessary to be specified. Thus this option has very low spec impact. However, to enhance MU interference measurement, the potentially paired UEs should transmit CSI-RS in the same resource. If interference is significant in the received signal, the estimation for the desired channel performs badly. Then interference measurement is not accurate either.

Opt A2: Emulation on NZP CSI-RS which is represented by multiplied value of channel and precoding matrix

In this option, UE is configured with multiple MU hypotheses by the network. To reduce CSI-RS overhead for multiple hypotheses, the paired UEs in each MU hypotheses share a configured NZP CSI-RS resource pool for interference measurement. gNB indicates the particular resource used for IM upon triggering. Then gNB transmits the NZP CSI-RS for all the paired UEs in one shared resource. In addition, for the interference caused by the cells outside the coordinating set, it can be measured by a single ZP CSI-RS resource configured for all the UEs in the set.

As NZP CSI-RS can also be used for channel measurement, the resource pool of IM also contains the NZP CSI-RS resource for channel measurement for better interference measurement and further resource sharing. gNB indicates the paired UEs which resources in the resource pool are used for channel measurement or interference measurement. UE acquires the CSI for its desired channel based on the channel measurement resource, and then subtract the CSI-RS power w.r.t. the desired channel in the interference measurement resource to obtain the interference power.  Explicit interference channel can be also estimated from NZP CSI-RS.  In addition, it is often required to obtain CSI with different channel and interference hypothesis for CoMP scenario. Sometimes the role between channel and interference can be swapped e.g. in case of DPS/DPB.  In such case, overhead of IMR can be greatly reduced by sharing the NZP-CSI-RS pool for channel and interference measurement.   Hence, to increase the RS efficiency for channel and interference measurement, IM and channel measurement should share the same pool including both NZP and ZP CSI-RS resources.

Proposal 1: NR should support Opt A2 for NZP CSI-RS based IMR.

Proposal 2: NR should support aperiodic interference measurement based on sharing NZP and ZP CSI-RS resource pool among UEs.

Different Rx beam may lead to different levels of interference. Since the RS resources for interference measurement and for channel measurement may be separate, e.g. NZP-CSI-RS and IMR, gNB may need to know or control the receive beam which is used for interference measurement. Or else, if receive beams used for interference measurement and for channel are not known in gNB, there may be an unexpected mismatch of interference experienced during the transmission and the interference measured in CSI which leads to poor link adaptation.

Proposal 3:   Signalling which assists UE to determine receive beam used for interference measurement is supported.

2.2 Channel measurement

In LTE, several schemes have been specified for channel measurement and CSI report such as codebook based PMI feedback, CRI based beam information feedback and so on. In NR, some of these schemes need to be enhanced or refined in order to solve their own issues or meet the new design target. In this subsection, enhancements of two potential measurement schemes are discussed.

CSR based PMI feedback

In LTE, codebook based CSI measurement and feedback is built with a set of configured parameters, e.g., codebook dimensions, oversampling factors, codebook config and so on. Except codebook dimensions, other parameters describes how W1 beam group is constructed from the grid of beams. Moreover, the constructed beam group can only be uniformly distributed in the spatial domain. As a matter of fact, this function can be implemented with one parameter, codebook subset restriction (CSR). Moreover, as Type I feedback is more suitable for non-rich scattering scenarios, only a few clusters exist in the wireless channel. Then only a small angular range of the entire space needs to be oversampled with large factor. Based on a baseline grid of beams, CSR can be used to indicate which angular range can be oversampled with larger factor, and which angular range can be quantized with more coarse beams. Hence configuration based on CSR can achieve similar performance as the current parameterized codebook in LTE with lower overhead. Additionally, with CSR, flexible codebook configuration can be more forward compatible.

Based on the above discussion, CSR is the essential parameter for codebook configuration. Then the flexibility and signaling overhead for CSR are important metrics in the design of CSI settings. Fig.3 gives a good example of CSR indication with a multi-layer structure. In our companion contribution [4], we give a more specific description on the design of CSR.

Fig. 3 Flexible CSR configuration

Proposal 4: CSR based CSI feedback should be supported for NR CSI measurement and report.

Beam information feedback based on port/resource measurement restriction

In CSI report involving BFed CSI-RS, such as Class B feedback and hybrid CSI feedback, port-level or resource-level beam information measurement is required. The most straightforward way to select beams is based on UE measurement on all the CSI-RS resources or ports. Specifically, gNB transmits BFed CSI-RS in one or more CSI-RS resources. Subsequently, UE measures channel quality of the configured resources and feedback the selected beams based on CRI or Class B codebook. However, the following issues can be identified if only this direct measurement and report approach is supported.

- Unfavorable for further CSI acquisition: In the hybrid CSI framework with Class B K>1 and Class B K=1, beam selection results in 1st level have a significant impact on the CSI acquired in the 2rd level since the gNB configures the precoding and RS resource for the 2rd level CSI based on the beam selection results. On the other hand, some CSI feedback schemes have particular requirement for the selected beams. For example, linear combination based feedback mechanism is a potential technique to give a better characterization than the legacy implicit feedback mechanism. Moreover, as pointed out in several contributions [5][6], linear combination feedback with orthogonal beams selected performs much better than the one with non-orthogonal beams selected. To employ the linear combination based CSI feedback mechanism, orthogonal ports should be configured for UE in the 2nd level CSI feedback. Hence UE’s report on beam selection in 1st level should contain the information of orthogonal beams at least. However, if the UE just feeds back the beams with channel measurement directly, the gNB may have trouble to find orthogonal beams based on the feedback.

- UE’s incorrect decision on beam selection: Due to the multipath channel propagation feature and beam width, in the UE side, the observed best beams are adjacent and correspond to the same Tx beam in some cases. Hence UE may make an incorrect decision if the feedback is only based on channel measurement.

The above issues appear in beam selection of both Class B feedback and hybrid CSI. One approach to solve this issue is to constrain UE’s behavior on beam measurement. Port/resource restriction on beam measurement is an efficient approach to set this constraint. For example, port/resource restriction can be applied to constraint the UE to measure and report particular resources or ports associated with orthogonal beam group or other beams with particular requirements. On the other hand, port/resource restriction can be applied to constrain the UE only report one beam in a group of adjacent beams, e.g., QCLed ports.

Proposal 5: NR should support beam information feedback based on configured port/resource restriction.

- Conclusions

In this contribution, we give some specific design on channel and interference measurements. Based on the above discussion, we have the following proposals.

Proposal 1: NR should support Opt A2 for NZP CSI-RS based IMR.

Proposal 2: NR should support aperiodic interference measurement based on sharing NZP and ZP CSI-RS resource pool among UEs.

Proposal 3:  Signalling which assists UE to determine receive beam used for interference measurement is supported.

Proposal 4 CSR based CSI feedback should be supported for NR CSI measurement and report.

Proposal 5: NR should support beam information feedback based on configured port/resource restriction.

- References

[1] Chairman’s notes, 3GPP RAN1 RAN1#88bis

[2] R1-1608683, Discussion on interference measurement for MU-CSI, ZTE, ZTE Microelectronics

[3] R1-1608598, Interference measurement enhancement in LTE Rel-14, Huawei, HiSilicon

[4] R1-1707126, On Type I CSI feedback details, ZTE

[5] R1-167645, W1 design for implicit advanced CSI reporting, Ericsson

[6] R1-1611419, Linear combination based CSI report for NR MIMO, ZTE