3GPP TSG RAN WG1 Meeting #89			R1-1707222

Hangzhou, P. R. China, 15th – 19th May 2017

Source:		SoftBank

Title:	Requirements and traffic models for aerials

Agenda Item:	6.2.8.2

Document for:	Discussion and Decision

Introduction

In RAN1#88bis, RAN1 has agreed the following simulation assumption related to traffic model:

Agreement:

- For data based traffic, the following traffic model is used for both terrestrial UEs and aerial UEs for evaluations:

  - FTP model 3 for UL/DL

    - Resource utilization values of 20% and 50%

- For command and control based traffic, the traffic models for aerial UEs are FFS

- FFS: from system perspective, whether a mix of data based traffic and command and control based traffic models for terrestrial and aerial UEs needs to be considered or not for evaluations

Also, operators are requested to input the requirements on aerials to accelerate the discussions. This contribution shares our views on the FFS part above, i.e. how to apply the traffic models to aerial/terrestrial UEs, In addition, the related requirements are proposed taking into account the current implementation of drones.

Requirement for traffics

We propose the following requirements for drone type traffics. Please note that the value is calculated considering the current implementations of drones for both consumer and enterprise use. Also, some additional room for feature extendibility is taken into account.

Proposal 1:

Additonal requirements for drone traffics:

|  | Command-and-control based traffic |  | Data based traffic |
| --- | --- | --- | --- |
| Use case | Light control (e.g. telemetry) | Heavy control (e.g. real time piloting) | Buffered video streaming etc. |
| UL/DL Data rate | 5 kbps | 50 kbps | 10Mbps ~ |
| Latency | 50 ms | 50 ms | N/A (depend on application) |
| Typical packet size (application layer) | 250 Bytes | 250 Bytes | N/A (Follow FTP Model 3 = 0.5 Mbytes ?) |
| Average duration of packet arrival | 500 ms | 50 ms | N/A (Tuned to meet RU=20% and 50%) |

Traffic Models

Models for command and control based traffic

Though the actual traffic characteristics depend on the application, protocol and use case, we should consider simplifying the simulations to meet the completion date of this SI. RAN1 discuss and choose one option from the following options. Note that the actual value (X, Y, Z) is defined after the traffic requirements are fixed.

Proposal 2:

Choose one option for command and control based traffic model for the evaluation purpose:

  - Option 1: constant bit rate

    - A UE continuously generatets small packet (X Byte) every Y ms

  - Option 2: FTP Model 3

    - Appliy smaller packet size (X Byte) and higher arrival rate (Z) than data based traffic

Mixed use of traffic models by a UE

Strictly speaking, aerial UEs may generate both command-and-control and data based traffic during the same time period depending on the use case. However for the simplicity, we can accept not to mix the traffic type from a UE.

Proposal 3:

RAN1 evaluations assume that a UE generates either command-and-control based traffic or data based traffic.

Control based traffic from terrestrial UEs

We are not interested in a scenario where terrestrial UEs generate command-and-control based traffic for drones because the use case is unclear.

Proposal 4:

RAN1 evaluations assume that a terrestrial UE generates data based traffic only.

Mixed use of traffic models by different UEs

Given the proposal 2, 3 and 4 above, we have three remaining possible combinations for RAN1 evaluations:

(1) Both aerial and terrestrial UEs apply data based traffic

(2) Terrestrial UEs apply data based traffic while aerial UEs apply command-and-control based traffic

(3) Terrestrial UEs apply data based traffic while aerial UEs apply either command-and-control based traffic or data based traffic (i.e. mixed use)

From our perspective, all the traffic modes are important and interesting (more importantly, this is realistic!), but (3) can be skipped to simplify the evaluations and analyses. Thus, we propose the following:

Proposal 5:

RAN1 to agree the following to traffic modes combinations for the evaluations:

  - Traffic mode 1

    - Terrestrial UEs:	data based traffic

    - Aerial UEs: 	data based traffic

  - Traffic mode 2

    - Terrestrial UEs:	data based traffic

    - Aerial UEs: 	command-and-control based traffic

Scheudler assumption for QoS handling

If traffic mode 2 in proposal 5 is agreed, we would encounter a new question whether or not eNB can/should handle these two traffic types differently using QoS. More concretely, command and control based traffic should be prioritized over data based traffic because it has a delay requirement. Even though the necessity of QoS is quite obvious from a service perspective, there still be a problems for RAN1 evaluations:

From the evaluation effort point of view, simulator implementation of QoS may not be so easy for some companies. In addition, it would be difficult to align the QoS scheduler policy among companies, resulting in the misalignment of simulation results.

At least until Rel-14, a QoS class (QCI) for drone command-and-control has not been defined in 3GPP. So, we are not sure whether eNBs can be aware of / differenciate such a traffic in reality. Thus technical feasibility of QoS scheduling itself should be carefully investigated.

Given the analysis above, we propose the following

Proposal 6:

For Traffic mode 2, RAN1 should discuss a QoS handling issue and chose one solution:

  - Solution 1. The eNB scheduler supports QoS between command-and-control based fraffic, and data based traffic. Thus control-and-command is prioritized to satify the (delay) requirements.

  - Solution 2. The eNB scheduler does not support QoS, i.e. command-and-control and data based traffic is equally handled by the eNB scheduler.

  - Solution 3. RAN1 does not define/recommend anything about scheduler policy, but companies are requested to disclose information about QoS policy with the simulation results.

Conclusion

Our proposals in this contribution are summarized as follows:

Proposal 1:

Additonal requirements for drone traffics:

|  | Command-and-control based traffic |  | Data based traffic |
| --- | --- | --- | --- |
| Use case | Light control (e.g. telemetry) | Heavy control (e.g. real time piloting) | Buffered video streaming etc. |
| UL/DL Data rate | 5 kbps | 50 kbps | 10Mbps ~ |
| Latency | 50 ms | 50 ms | N/A (depend on application) |
| Typical packet size (application layer) | 250 Bytes | 250 Bytes | N/A (Follow FTP Model 3 = 0.5 Mbytes ?) |
| Average duration of packet arrival | 500 ms | 50 ms | N/A (Tuned to meet RU=20% and 50%) |

Proposal 2:

Choose one option for command and control based traffic model for the evaluation purpose:

  - Option 1: constant bit rate

    - A UE continuously generatets small packet (X Byte) every Y ms

  - Option 2: FTP Model 3

    - Appliy smaller packet size (X Byte) and higher arrival rate (Z) than data based traffic

Proposal 3:

RAN1 evaluations assume that a UE generates either command-and-control based traffic or data based traffic.

Proposal 4:

RAN1 evaluations assume that a terrestrial UE generates data based traffic only.

Proposal 5:

RAN1 to agree the following to traffic modes combinations for the evaluations:

  - Traffic mode 1

    - Terrestrial UEs:	data based traffic

    - Aerial UEs: 	data based traffic

  - Traffic mode 2

    - Terrestrial UEs:	data based traffic

    - Aerial UEs: 	command-and-control based traffic

Proposal 6:

For Traffic mode 2, RAN1 should discuss a QoS handling issue and chose one solution:

  - Solution 1. The eNB scheduler supports QoS between command-and-control based fraffic, and data based traffic. Thus control-and-command is prioritized to satify the (delay) requirements.

  - Solution 2. The eNB scheduler does not support QoS, i.e. command-and-control and data based traffic is equally handled by the eNB scheduler.

  - Solution 3. RAN1 does not define/recommend anything about scheduler policy, but companies are requested to disclose information about QoS policy with the simulation results.