3GPP TSG RAN WG1 Meeting #89	     R1-1707258

Hangzhou, P.R. China 15th – 19th May 2017

Agenda Item:	7.1.3.3.2

Source: 	Fujitsu

Title:                     Discussions on HARQ for grant-free transmission

Document for:	Discussion/Decision

# Introduction

The general framework of grant-free UL URLLC has been established as follows.

Agreements:

- For UL transmission without grant,

  - The resource configuration includes at least the following

    - Time and frequency resources, FFS: including resources for repetitions, implicitly or explicitly

    - Modulation and coding scheme(s), possibly including RV, implicitly or explicitly

    - Reference signal parameters

      - FFS: Details

    - FFS: The number of repetitions K

      - FFS: Whether multiple number of K can be configured to one UE

    - FFS other parameters

  - FFS: A UE may continue repetitions for a TB until one of the following conditions is met

    - An ACK is successfully received from gNB

    - The number of repetitions for the TB reaches K

In this contribution, we share some more discussions on HARQ related issues. More specifically, they include UE behavior under BS misdetection, potential hopping mechanisms over retransmissions/repetitions, and support of multiple HARQ processes.

# Discussion

For URLLC, the target for user plane latency should be 0.5ms for UL, and 0.5ms for DL, and the reliability requirement for one transmission of a packet is 10-5 for 32 bytes with a user plane latency of 1ms [1]. For the target of low latency, grant-free transmission is beneficial in the sense that the latency related to the transmission of SR/BSR and UL grant can be removed. As for the high reliability, it has been agreed to support HARQ retransmissions and repetitions with the same or different RVs. Compared with grant-based transmission, HARQ for grant-free transmission should be designed with particular considerations. For example, there could be detection errors at the BS and physical resource collisions among UEs for grant-free transmission. These will lead to reception errors, and some of these errors can be corrected by HARQ retransmissions/repetitions within the required latency. In the following, we share some considerations on HARQ related issues for grant-free URLLC.

## UE behavior under BS misdetection

In grant-free transmission, the BS needs to blindly detect which UEs are performing transmission. As RS parameters could be at least configured semi-statically, it is assumed that there is a one-to-one mapping between DMRS properties (e.g. DMRS sequence) and UE ID in this contribution. However, misdetection is still quite possible, i.e., the BS completely fails to detect a transmission. In this case, the BS will not feedback any ACK/NACK at least for the UE initial transmission. From the UE side, the UE has performed transmission but not received any ACK or NACK. Due to lack of information on the previous transmission, the BS cannot perform HARQ IR combining anyway. Therefore, it would be reasonable to let the UE only repeat the previous transmission instead of transmitting with a new RV.

Proposal 1: For grant-free transmission, the UE will repeat the previous transmission when neither ACK nor NACK is received.

## Hopping over retransmissions/repetitions

Collisions are inevitable for grant-free transmission when physical resources are shared among UEs. Since collisions are likely to result in reception errors, they should be avoided or reduced with best effort. As exemplified in Figure 1, even if the initial transmissions collide, the retransmission/repetition collisions in frequency and/or time domain can be avoided.

a) ReTx collision avoided in frequency domain      b) ReTx collision avoided in frequency and time domain

Figure 1. Illustration of HARQ retransmissions/repetitions

To achieve the more desired results in Figure 1, one way is explicitly configuring UE the retransmissions/repetitions by BS signaling. The configuration information can be transmitted together with NACK, e.g., contained within one PDCCH. The benefit is that collisions are more controllable due to centralized scheduling. The drawback could be the additional signaling overhead and the possible cost to keep the signaling also highly reliable. Another way could be making the UE transmissions follow certain hopping patterns. The resources for retransmissions/repetitions can be determined in a pseudo-random manner. The same can also apply to transmissions without ACK/NACK, e.g., consecutive transmissions [2]. The principle of pseudo-randomness has been used in LTE. For example, the PDCCH search space is determined pseudo-randomly by using Yk which is a recursive function related to RNTI and subframe index k. Similar methods can be used to obtain the retransmission/repetition hopping patterns. For URLLC, preferably different UEs will have different DM-RS sequences and/or UE IDs. Therefore, DM-RS ID and/or UE ID can be used to generate Yk. If a transmission occurs at the time and frequency resource k, then the corresponding calculated Yk can be used to determine the next retransmission/repetition resource for example using: kr=Yk mod R. Here kr is an index within the R available resources for the retransmission/repetition. Constraints may be put onto R to make the whole transmission complete within the required latency. For retransmission/repetition hopping, signaling overhead can be saved but collisions cannot be completely avoided. Tradeoff exists between these two methods.

Proposal 2: The time/frequency index of the first detected grant-free transmission and the UE ID can be used to determine hopping patterns for any retransmissions/repetitions.

## Support of multiple HARQ processes

In LTE FDD, a maximum 8 HARQ processes can be supported. The HARQ process number is used to identify a particular TB. The HARQ process number is explicitly indicated for DL HARQ, and can be implicitly derived for UL HARQ. To support multiple HARQ processes in grant-free transmission, the UE needs to identify which TB is being scheduled for retransmission or acknowledged when receiving a UL grant [3] [4]. Here the UL grant is used to schedule a retransmission for a certain TB, or used to acknowledge a correctly received TB and/or schedule a new TB transmission. In the following, we only discuss the case where the timing between PUSCH and ACK/NACK is not fixed. The following characteristics make it difficult to reuse LTE solutions.

- The HARQ process number cannot be derived from the timing as in LTE UL HARQ due to flexible HARQ timing.

- The HARQ process number cannot be assigned by the BS from the beginning since there is no UL grant for the initial transmission.

- Letting the BS and the UE count and number the TBs independently will cause ambiguity if the BS misses a certain TB from the UE.

To this end, new mechanisms are needed to associate a UL grant with its targeted TB. One method could be indicating the resource index of the TB in the UL grant. For example in Figure 2, a resource grid (red box) is formed w.r.t. the UL grant receiving instant, and a TB can be located by a resource index within this grid. By indicating the resource index in the UL grant, both the UE and the BS will have a common understanding on the referred TB. The size of the resource grid may depend on the maximum interval between a TB transmission and the corresponding UL grant. The frequency dimension is used to index a TB mainly from the consideration that a new TB may arrive and be transmitted during the repetitions of another TB as shown in Figure 3. If this is not allowed, then the resource index can be represented by the time dimension alone.

Figure 2. Illustration of resource index indication (w/o repetition)

Figure 3. Illustration of resource index indication (w/ repetition)

Another method could be associating different HARQ processes with different DM-RS signatures. As aforementioned, a one-to-one mapping between UE IDs and DMRS sequences is assumed. Furthermore, a complex symbol can be used to modulate the DM-RS sequence to indicate a HARQ process. For example, symbols from a QPSK constellation can indicate up to 4 HARQ processes. The BS can blindly detect the existence of the DM-RS sequence and the carried symbol. Then both UE activity and HARQ process number can be identified by the BS. In the UL grant, the BS will notify the UE the HARQ process number.

Based on above analyses, we give the following proposal.

Proposal 3: To identify which TB is associated with a particular UL grant, following alternatives can be considered

- Alt. 1: The resource index of the TB is signaled by the UL grant.

- Alt. 2: HARQ process number is signaled by the UL grant, and different HARQ processes are associated with different DM-RS signatures.

# Conclusions

In this contribution, we share our views on grant-free transmission for UL URLLC. It has been agreed to support grant-free transmission and retransmissions/repetitions for UL URLLC. Still studies on how to support the HARQ are needed for grant-free transmission. Based on the analyses, we give the following proposals.

Proposal 1: For grant-free transmission, the UE will repeat the previous transmission when neither ACK nor NACK is received.

Proposal 2: The time/frequency index of the first detected grant-free transmission and the UE ID can be used to determine hopping patterns for any retransmissions/repetitions.

Proposal 3: To identify which TB is associated with a particular UL grant, following alternatives can be considered

- Alt. 1: The resource index of the TB is signaled by the UL grant.

- Alt. 2: HARQ process number is signaled by the UL grant, and different HARQ processes are associated with different DM-RS signatures

# References

- 3GPP TR 38.913 V14.0.0, “Study on Scenarios and Requirements for Next Generation Access Technologies”.

- R1-1701665, “UL Grant-free transmission”, Huawei, HiSilicon, RAN1#88.

- R1-1704762, “Grant-free UL transmission scheduling and HARQ aspects”, Intel, RAN1#88b.

- R1-1704428, “HARQ for URLLC UL grant-free transmission”, ZTE, ZTE Microelectronics, RAN1#88b.