3GPP TSG RAN WG1 Meeting #89                                                         	         R1-1707299

Hangzhou, P.R. China 15th – 19th May 2017

Agenda Item:          6.2.2.3

Source: 		Intel Corporation

Title:	Considerations for autonomous uplink access on LAA SCells

Document for: 	Discussion/Decision

# Motivation

A new work item on ‘enhancements to LTE operation in unlicensed spectrum’ was approved in TSG RAN Meeting #75 [1]. The objective of the new WI from RAN1 perspective is twofold:

- Specify support for multiple starting and ending positions in a subframe for UL and DL on SCell with Frame structure type 3.

- (Starting in RAN1#90): Study, and specify if needed, support for autonomous uplink access with Frame Structure type 3 considering solutions from the L2 latency reduction work item

This contribution to further motivate the second objective and to suggest several design aspects that needs to be considered to enable autonomous uplink access before the start of the work in RAN1#90.

It was identified that LAA UL can experience significant performance degradation as documented in our previous contributions [1-2]. The unbalance in LAA DL and UL performance is recaptured in the figure below.

Figure 1. The unbalance in LAA DL and UL performance

One of the main reasons of the performance degradation is due to the “quadruple” contention for UEs to access the UL. That is, 1) UE to send SR, 2) LBT performed at the eNB before sending UL grant (especially in the case of self-carrier scheduling), 3) UE scheduling (internal contention amongst UEs associated with the same eNB) and 4) LBT performed only by the scheduled UE. In addition, one particular limitation imposed on the LTE system is the four subframe processing delay between UL grant and PUSCH transmission.

The non-scheduled UL transmission has advantage over scheduled UL transmission in the following aspects and can be helpful to improve the LAA UL performance. 1) If a UE succeeds the LBT, then it can start transmitting immediately as Wi-Fi. Thus, no more multiple contention imposed on the UL access. 2) The UL autonomous transmission does not rely on the UL grant. 3) It will naturally well-coexist with Wi-Fi as the UE behaviour is not different from Wi-Fi stations.

# Necessary considerations to enable UL autonomous transmission

## Overall operation

In this section, necessary enhancements to enable non-scheduled operation will be discussed including UL LBT, UL subframe design, scheduling, link adaptation and the detection of UL burst at the eNB.  Figure 1 illustrates the potential non-scheduled operation. The UE that intends to transmit to the anchored eNB performs channel sensing without the need for UL grant from the eNB. After completing the LBT, the UE then starts its transmission immediately.

Figure 2: An illustration of UL autonomous transmission

## UL LBT

In order to maintain fair co-existence with incumbent systems and with other LAA networks, the non-scheduled UEs should perform full category 4 LBT as the Release 13 DL LAA. This will ensure natural coexistence with neighbour Wi-Fi/LAA networks as it is shown in the performance evaluation results in Section 3.

## UL subframe design

The non-scheduled UE may sense the channel and perform UL transmission at any time. Similar to the design consideration for Release 13 LAA, a partial subframe for PUSCH transmission can be considered to minimize the implementation impact. As another alternative, PUSCH can be transmitted without the alignment with PCell subframe boundary (aka floating subframe). The UE may transmit the PUSCH transmission based on the legacy 1 ms subframe design after the completion of LBT.

## PUSCH detection at eNB

For the UE-autonomous transmission, the eNB needs to detect the presence of the UL burst. The detection can be done via:

- Preamble signal (or initial signal)

- DMRS

In Section 3, the DMRS-based presence detection performance is provided.

## Link adaptation

Link adaptation and MCS selection at the UE can be performed based on the CQI feedback from the eNB, or using channel reciprocity. Some information such as UE C-RNTI and the chosen MCS needs to be signalled to the eNB to help decoding. The following options for signalling can be considered.

- Using newly defined preamble signal: The preamble signal can not only be used for detection but also contain information such as UE C-RNTI, MCS, and the length of UL burst, etc.

- Using newly defined control channel for non-scheduled access: The newly defined control channel can be used to indicate the above listed information.

# Performance Evaluation

In this section, we compare the performance of UL autonomous transmission to that of the original scheduled operation. In addition, we provide the link-level presence detection performance based on DMRS.

## Comparison between scheduled and non-scheduled UL access

The simulation assumptions comply with 3GPP evaluation methodology [4]. Figure 3 provides the UL UPT when Wi-Fi coexists with LAA without and with UL autonomous access. The traffic ratio for DL:UL was assumed 50%:50% and simulated for indoor scenario.

Figure 3: Improved LAA UL performance with UL autonomous transmission

Observation 1:

- UL autonomous transmission can help to balance the UL performance between LAA and Wi-Fi.

- The LAA with UL autonomous access well coexists with Wi-Fi.

## DMRS based UL transmission presence detection

We present the evaluation results for the DMRS based PUSCH presence detection as follows. The simulation assumption is given in Table 1 in appendix. Figure 4 illustrates the misdetection probability at the given false-alarm probability. From Figure 3, we can observe that the SNR required to achieve detection probability higher than 99% and 99.9% are -14dB and -12.3dB, respectively, when targeting false-alarm probability is 1%. When the false-alarm probability is 0.1%, the SNR required to achieve detection probability higher than 99% and 99.9% are -13.5dB and -11.4dB, respectively. Thus, we can conclude that the DMRS based detection can provide enough performance.

1)  Pfa = 1%                                                                         2)  Pfa = 0.1%

Figure 4: DMRS-based presence detection for non-scheduled UL access

Observation 2:  The DMRS based detection can provide sufficiently good performance for non-scheduled PUSCH detection.

# Conclusion

In this contribution, the second objective of the new FeLAA WI, i.e., UL autonomous transmission, was motivated and several design aspects that needs to be considered were suggested. The following observations were made through the performance evaluation.

Observation 1:

- UL autonomous transmission can help to balance the UL performance between LAA and Wi-Fi.

- The LAA with UL autonomous access well coexists with Wi-Fi.

non-scheduled (grant-less) UL access for eLAA was introduced. Below we summarize our observations and proposals:

Observation 1:

- The LAA UL performance is significantly improved by non-scheduled access.

- LAA with non-scheduled UL access well coexists with Wi-Fi.

Observation 2:  The DMRS based detection can provide sufficiently good performance for non-scheduled PUSCH detection.

# References

[1] RP-170848, New Work Item on Enhancements to LTE operation in unlicensed spectrum, Nokia, Ericsson, Intel, Qualcomm, 3GPP TSG RAN Meeting #75, March, 2017

[2] R1-160425, “On the UL scheduling starvation issue”, 3GPP TSG RAN WG1 #84, Intel Corporation, Feb., 2016

[3] R1-160424, “On the Remaining details for UL LBT”, 3GPP TSG RAN WG1 #84, Intel Corporation, Feb., 2016

[4] TR 36.889

# Appendix

Table 1: Simulation Assumptions for DMRS Based Detection

|  | Comments |  |
| --- | --- | --- |
| Simulation setup | System bandwidth | 20 MHz |
|  | Number of transmitter antennas | 1 |
|  | Number of receiver antennas | 2 |
|  | Channel model | EPA-5Hz |
|  | Number of PRBs for PUSCH transmission | 100 PRBs |
|  | Number of UEs that can simultaneously transmit | At most 1 UE |
|  | Number of UEs anchored at eNB | 12 |
|  | Targeting false-alarm probability | 1%, 0.1% |
| Key performance metrics | False-alarm probability (Pfa) | The probability that no PUSCH is transmitted in the subframe, but eNB detects the presence of the signal. |
|  | Misdetection probability (Pmd) | The probability that the UE transmits, but eNB detects that the UE of interest does not transmit. |