3GPP TSG-RAN WG1 #89   													                             R1-1707376

Hangzhou, P. R. China, 15th - 19th May 2017

Source: 	Intel Corporation

Title:	Evaluations of distributed NR-PDCCH transmission

Agenda item:	    7.1.3.1.1

Document for:	Discussion and Decision

# Introduction

In RAN1 #88b [1], the following agreements on transmission schemes and NR-PDCCH structure have been agreed.

Working assumption: - One-port transmit diversity scheme with REG bundling per CCE is used for NR-PDCCH - FFS the bundling size - FFS: REG bundling is also for localized mapping in time and/or frequency-domain - Companies are encouraged to provide evaluation results for 10 MHz and 20 MHz for larger aggregation levels and 5 MHz and 10 MHz for smaller aggregation levels Agreement: NR-PDCCH can be mapped contiguously or non-contiguously in frequency with localized or distributed mapping of REGs to a CCE (in the physical domain) Agreements: - A CCE may be mapped to REGs with interleaved or non-interleaved REG indices within a CORESET - Definition of a REG bundle: The UE may assume that the same precoder is used for the REGs in a REG bundle and that the REGs in a REG bundle are contiguous in frequency and/or time - REG bundling per CCE is supported for NR-PDCCH - FFS: Whether this applies to common search space - FFS: Whether all REGs have DMRS or not - FFS: Whether wideband precoding is supported and the definition of a REG bundle if it is supported - FFS: whether REG bundle size is different for mapping of NR-PDCCH with or without interleaved mapping of CCE to REGs - FFS on REG bundle size - FFS whether REG bundle size is configurable Working assumption: - A NR-CCE is defined as 6 REGs - Candidate bundle sizes for distributed REG-to-CCE mapping: 2 or 3 REGs if NR-CCE is defined as 6 REGs

This contribution presents our evaluation results on distributed NR-PDCCH transmission scheme, to study performance impacts of different REG bundling (REGB) schemes for different possible CORESET configurations. The general design aspects on REGB and distributed NR-PDCCH/NR-CCE resource mapping are discussed in [1]. Moreover, the DMRS design for NR-PDCCH and transmission schemes for multi-beam operation are presented in [3] and [4], respectively.

# Discussion

According to the agreements from RAN1#88b, a CCE may be mapped to REGs with interleaved or non-interleaved REG indices within a CORESET. Moreover, REG bundle (REGB) as a terminology is officially defined so that UE may assume same precoding used for all REGs in a REGB, which are continuously mapped in frequency and/or time. Furthermore REGB per CCE is supported. Given that one NR-CCE is comprised of 6 REGs, the current work assumptions on candidate REGB sizes for distributed REG-to-CCE mapping are two and three REGs per REGB. Based on these agreements, distributed NR-PDCCH can be constructed by using CCEs being mapped to REGs with interleaved REG indices within the CORESET. Since REGs in a REGB can be continuous mapped in frequency and/or time, at least two REGB options depending on the domain in which the bundling is performed can be supported in the specification.

As summarized in the above agreements table, REGB size is FFS. In this contribution, we evaluate different REGB sizes, i.e., 2 and 3 REGs with different bundling directions, i.e. bundling in frequency or time, for different CORESET configurations comprised of different numbers of OSs and RBs in frequency. Specifically, we have considered three CORESET configurations, i.e., CORESETs of 1, 2 and 3 OSs. For each CORESET configuration, different REGB designs are evaluated. For the sake of simplicity, we define the BxT(F)-T(F)F, where x is 2 or 3, as the REGB configuration which consists of x consecutive REGs in time (frequency) domain and is numbered in time (frequency) first order. For all the distributed NR-PDCCH, per-REGB precoder cycling (PC) is employed to achieve maximum possible transmit diversity. Detailed simulation parameters are described in the table in Appendix. The evaluated REGB options for each considered CORESET configuration are summarized in the following table 1.

Table 1. REGB options for evaluations

| CORESET of 1 OS and 48 RBs in frequency | B2F-FF; B3F-FF |
| --- | --- |
| CORESET of 2 OSs and 24 RBs in frequency | B2T-FF, B2F-FF, B2F-TF, B3F-TF |
| CORESET of 3 Oss and 16 (18) RBs in frequency | B3T-FF, B3F-FF, B2F-TF, B3F-TF |

## 2.1 CORESET of one OS

In case of CORESET of 1 OS, REGs can be bundled only in frequency domain. We compare two REGB options, i.e., B2F-FF and B3F-FF as illustrated in Fig. 1, corresponding to REGB of 2 and 3 REGs. Given CCE size of 6 REGs, to accommodate an AL8 NR-PDCCH candidate, we consider a CORESET of 48 consecutive RBs in frequency domain.

Figure 1. CORESET of 1 OS and 48 RBs, Two REGB options: B2F-FF and B3F-FF. Each block represents one REG, and consecutive REGs in same color defines a REGB. The indices of REGs indicate the index of CCE.

Four ALs, namely AL1, 2, 4 and 8, are simulated for four different channel models, i.e., TDL-A(C) of DS 30ns and 300ns. The simulation results are given in Fig. 2.

| - TDL-A, DS 30ns | - TDL-A, DS 300ns |
| --- | --- |
| - TDL-C, DS 30ns | - TDL-C, DS 300ns |

Figure 2. B2F-FF vs B3F-FF, DCI: 60b, CORESET of 1 OS and 48 RBs.

Based the above simulation results in Fig. 2, we have the following observations.

Observation 1: In case of CORESET of 1 OS and 48 RBs, REGBs of 2 and 3 REGs exhibit similar performance in most cases. REGB of 3 REGs shows slightly better performance than REGB of 2 REGs for high AL NR-PDCCH in low SNR, while the REGB of 2 REGs is slightly better for AL1 NRPDCCH in high SNR.

## 2.2 CORESET of two OSs

In case of CORESET of 2 OSs and 24 RBs, we compare four REGB options, namely B2T-FF, B2F-FF, B2F-TF and B3F-TF as illustrated in Fig. 3. It should be noted there are other possible REGB option, i.e., B3F-FF, however as discussed in [1], it would lead to unwanted localized NR-PDCCH transmission. Hence it is not simulated.

Figure 3. CORESET of 2 OSs and 24 RBs, Four REGB options: B2T-FF, B2F-TF, B2F-FF and B3F-TF. Each block represents one REG, and consecutive REGs in same color defines a REGB. The indices of REGs indicate the index of CCE.

The simulation results are given in Figs 4 – 6.

| - TDL-A, DS 30ns | (b) TDL-A, DS 300ns |
| --- | --- |
| (c) TDL-C, DS 30ns | (d) TDL-C, DS 300ns |

Figure 4 B2F-TF vs B2F-FF, DCI: 60b, CORESET of 2 OSs and 24 RBs.

| - TDL-A, DS 30ns | - TDL-A, DS 300ns |
| --- | --- |
| - TDL-C, DS 30ns | - TDL-C, DS 300ns |

Figure 5 B2F-TF vs B3F-TF, DCI: 60b, CORESET of 2 OSs and 24 RBs.

| - TDL-A, DS 30ns | - TDL-A, DS 300ns |
| --- | --- |
| (c) TDL-C, DS 30ns | (d) TDL-C, DS 300ns |

Figure 6. B2T-FF vs B2F-TF, DCI: 60b, CORESET of 2 OSs and 24 RBs.

Based on the above simulation results in Figs. 4-6, we have the following observations.

Observation 2: In case of CORESET of 2 OSs and 24 RBs, while all four REGB options show quite similar performance in most cases, REGB option of B3F-TF shows slightly better performance than other 3 REGB options for AL8 NR-PDCCH in low SNR region.

## 2.3 CORESET of three OSs

In case of CORESET of 3 OSs and 16 or 18 RBs, we compare four REGB options, namely B3T-FF, B3F-FF, B3F-TF and B2F-TF as illustrated in Fig. 7. It should be noted there are other possible REGB option, i.e., B2F-FF, however as discussed in [1], it would lead to unwanted localized NR-PDCCH transmission. Hence it is not simulated. As illustrated in Fig. 7, it should be also noted that for REGB options of bundling 3 consecutive REGs in frequency, it would require number of RBs in CORESET to be a multiple of 3, e.g., 18 RBs, this would further lead to some unnumbered/unused REGs (shadowed REGs in Fig. 7) in CORESET.

Figure 7. CORESET of 3 OSs and 16 (18) RBs, Four REGB options: B3T-FF, B3F-FF, B3F-TF and B2F-TF. Each block represents one REG, and consecutive REGs in same color defines a REGB. The indices of REGs indicate the index of CCE.

The simulation results are given in Figs 8 – 10.

| - TDL-A, DS 30ns | (b) TDL-A, DS 300ns |
| --- | --- |
| (c) TDL-C, DS 30ns | (d) TDL-C, DS 300ns |

Figure 8 B2F-TF vs B3F-FF, DCI: 60b, CORESET of 3 OSs and 16 (18) RBs.

| - TDL-A, DS 30ns | - TDL-A, DS 300ns |
| --- | --- |
| - TDL-C, DS 30ns | - TDL-C, DS 300ns |

Figure 9 B3F-FF vs B3F-TF, DCI: 60b, CORESET of 3 OSs and 16 (18) RBs.

| - TDL-A, DS 30ns | - TDL-A, DS 300ns |
| --- | --- |
| (c) TDL-C, DS 30ns | (d) TDL-C, DS 300ns |

Figure 10. B3T-FF vs B2F-TF, DCI: 60b, CORESET of 3 OSs and 16 (18) RBs.

From the above simulation results in Figs. 8-10, we have the following observations.

Observation 3: In case of CORESET of 3 OSs and 16 or 18 RBs, while all four REGB options show quite similar performance in most cases, REGB option of B3T-FF shows slightly better performance than other 3 REGB options for AL8 NR-PDCCH in low SNR region.

# Conclusions

Our observations based on evaluation results in this contribution are summarized as follows.

Observation 1: In case of CORESET of 1 OS and 48 RBs, REGBs of 2 and 3 REGs exhibit similar performance in most cases. REGB of 3 REGs shows slightly better performance than REGB of 2 REGs for high AL NR-PDCCH in low SNR, while the REGB of 2 REGs is slightly better for AL1 NRPDCCH in high SNR.

Observation 2: In case of CORESET of 2 OSs and 24 RBs, while all four REGB options show quite similar performance in most cases, REGB option of B3F-TF shows slightly better performance than other 3 REGB options for AL8 NR-PDCCH in low SNR region.

Observation 3: In case of CORESET of 3 OSs and 16 or 18 RBs, while all four REGB options show quite similar performance in most cases, REGB option of B3T-FF shows slightly better performance than other 3 REGB options for AL8 NR-PDCCH in low SNR region.

# References

- Chairman’s notes, RAN1 #88b, 3GPP TSG RAN WG1 Meeting, April 2017.

- R1-1707375, NR PDCCH structure and transmissions, Intel Corporation

- R1-1707377, On DMRS design for NR-PDCCH, Intel Corporation

- R1-1707378, NR-PDCCH transmission schemes and multi-beam operation, Intel Corporation

# Appendix

Table 1. Link level simulation parameters

| Parameters | Value |
| --- | --- |
| Channel model | TDL-A and TDL-C, Delay scaling: 30 and 300ns |
| UE speed [kmph] | 3 |
| Channel coding | TBCC |
| Number of gNB transmit antennas | 2 |
| Number of UE receive antennas | 2 |
| Numerology [KHz] | 15 |
| System bandwidth [MHz] | 20 |
| DCI payload size [Bits] | 60 |
| CORESET PRB set for distributed transmission | [1:48] for CORESET of 1 OS; [1:24] for CORESET of 2 OSs; [1:16] or [1:18] for CORESET of 3 OSs. |
| Number of control symbols | 1, 2 and 3 |
| Number of REGs/CCE | 6 |
| REG bundling | Bundling of 2 and 3 REGs in frequency or time domain. |
| REG-To-CCE | Interleaved REGB indices which is numbered in time-first and freq-first order. |
| Aggregation levels | 1, 2, 4 and 8 |
| Transmission scheme | per-REGB precoder cycling. |
| Channel estimation | MMSE, averaging over REGB. |
| UE receiver | MMSE |