3GPP TSG RAN WG1 #89  	R1-1708046

# Hangzhou, China, 15th – 19th May 2017

Agenda item:	7.1.4.1.2

Source: 	Samsung

Title: 	LDPC Code Design for eMBB

Document for:	Discussion and Decision

Introduction

Related to LDPC code design, the following working assumption was made in the RAN1 #88bis meeting [1].

Working Assumption: - The largest info block size supported by LDPC encoder Kmax and the largest shift size Zmax defined is {8448, 384} => Kbmax = 22 - To be confirmed automatically at RAN1#89 if no significant implementation or performance issues are identified. - The base graph supporting Kmax should support the following set of shift sizes Z, where Z= a×2j: Z; a. 2; 3; 5; 7; 9; 11; 13; 15. j; 0; 2; 3; 5; 7; 9; 11; 13; 15. 1; 4; 6; 10; 14; 18; 22; 26; 30. 2; 8; 12; 20; 28; 36; 44; 52; 60. 3; 16; 24; 40; 56; 72; 88; 104; 120. 4; 32; 48; 80; 112; 144; 176; 208; 240. 5; 64; 96; 160; 224; 288; 352. 6; 128; 192; 320. 7; 256; 384. - FFS by RAN1#89 whether some values can removed from the above table. - FFS by RAN1#89 whether some of {272, 304, 336, 368} can be added to the above table.

In addition, the followings are agreed in the RAN1 #88bis meeting [1].

Agreement: The base graph design is selected from the following alternatives: Alt 1: One base graph covering ~1/5 <= R <= ~8/9 Alt 1a: Two nested base graphs, where: - Base graph #1 - Covers info block size K: Kmin1 <=K<= Kmax1, Kmin1 > Kmin, Kmax1 =Kmax - Covers code rate R: ~1/3 <= R <= ~8/9; FFS whether Rmin can be ~1/5 - Base graph #2 - Nested within base graph #1 - Covers info block size K: Kmin2 <=K<= Kmax2, Kmin2 =Kmin, Kmax2 < Kmax, where 512<=Kmax2<=2560 - Covers code rate R: ~1/5 <= R <= ~2/3 - Kbmax =16 is the starting point; lower values in the range 10<=Kbmax<16 are encouraged if feasible. - The set of supported shift sizes is taken from the set of shift sizes supported by the base graph supporting Kmax Alt 2: Two base graphs, where: - Base graph #1 - Covers info block size K: Kmin1 <=K<= Kmax1, Kmin1 > Kmin, Kmax1 =Kmax - Covers code rate R: ~1/3 <= R <= ~8/9; FFS whether Rmin can be ~1/5 - Base graph #2 - Not nested within base graph #1 - Covers info block size K: Kmin2 <=K<= Kmax2, Kmin2 =Kmin, Kmax2 < Kmax, where 512<=Kmax2<=2560 - Covers code rate R: ~1/5 <= R <= ~2/3 - Kbmax = 10 is the starting point; higher values in the range 10<Kbmax<=16 can also be considered if necessary. - The set of supported shift sizes is taken from the set of shift sizes supported by the base graph supporting Kmax BLER Performance is the main criterion for selecting between Alts 1, 1a and 2 (since it is already assumed that complexity is not increased significantly by the addition of a second smaller base graph); decoding latency (e.g. evaluated by the number of edges) should also be considered as an important criterion.

In this contribution, we propose LDPC codes designed with the agreed parameters based on Alt 2. The evaluation results show that the proposed LDPC codes have a good performance according to the change of information block sizes and code rates.

Performance Evaluation of Proposed QC LDPC Codes

  - Evaluation Results for Our Proposed QC LDPC Codes

In this section, the performance evaluation results of the updated QC LDPC code are provided. The parameters of QC LDPC codes and the simulation assumption are presented in Table 1-1, 1-2, and Table 2, respectively, and the other details are given in the excel sheets attached separately. Note that only one shift value matrix is used for each base graph.

Table 1-1:  LDPC Parameters for Base Graph #1

| Number of information columns in base graph (Kb) | 22 |
| --- | --- |
| Supported shift sizes (Z) | 2:1:7, 8:2:14, 16:4:28, 32:8:56, 64:16:112, 128:32:224, 256:64:384 |
| Supported information sizes (K) | 40 ~ 8448 |
| Minimum code rate of base graph (Rmin) | 1/5 |
| Number of punctured information bits | 2Z |

Table 1-2:  LDPC Parameters for Base Graph #2

| Number of information columns in base graph (Kb) | 10 |
| --- | --- |
| Supported shift sizes (Z) | 4:1:7, 8:2:14, 16:4:28, 32:8:56, 64:16:112, 128:32:256 |
| Supported information sizes (K) | 40 ~ 2560 |
| Minimum code rate of base graph (Rmin) | 1/5 |
| Number of punctured information bits | 2Z |

Table 2:  Simulation Assumptions

| Channel | AWGN |
| --- | --- |
| Modulation | QPSK |
| Coding Scheme | LDPC |
| Code rate | 1/5, 1/3, 2/5, 1/2, 2/3, 3/4, 5/6, 8/9 |
| Decoding algorithm | Sum-product algorithm with flooding scheduling (iter=50) |
| Info. block length (K) | 40:8:512, 512:16:1024, 1024:32:2048, 2048:64:6144, 6144:128:8448 |

Z value calculation

Z value is selected as the minimum value from supported Z values such that Z*Kb >= K for given information size K and number of information column blocks of base graph Kb.

Lifting method based on single shift value matrix

A Lifting and shortening method is applied to adjust information block length. Using the lifting method, each shift values for adjusted shift sizes are easily calculated by the specified formula. For example, we can obtain the shift value matrix EZH=Pij(Z) for the parity-check matrix HZ from the exponent matrix EH=Vij for the parity-check matrix H as follows:

EZH=Pij(Z)    s.t.   Pij(Z)=Vij,Vij<0f(Vij, Z),Vij≥0.

Here, EZH is the parity-check matrix consisting of Z×Z circulant permutation matrices and/or zero matrices for given integer Z and f(Vij, Z) is an integer function of Vij and Z.

Figure 1. Lifting technique for length compatibility

We propose the lifting function fVij, Z as follows:

fVij, Z=VijVij<0mod(Vij,2k)Vij≥0  and  2k≤Z<2k+1

where mod(Vij,A) means a modulo operation Vij mod A. Note that for 2k≤Z<2k+1, the shift value matrices have exactly the same integer entries

Shortening

After lifting, (kbZ-K) known bits are attached to the last part of information bits for shortening and they are not transmitted, where kb is the number of information column blocks.

Operations between two base graphs

There are some cases of information lengths and code rates which can be covered by two base graphs. We need to describe which base graph covers those cases explicitly. Regarding overlapped cases between two base graphs, we propose to select a base graph according to performance. In this contribution, base graph #2 covers 40<=K<700 and 1/5<=R<=2/3 while base graph #1 covers the other cases for initial transmission.

Evaluation results

In Figures 2 and 3, we can see that the proposed QC LDPC code provides a good and stable CBLER (code block error rate) performance up to 10-4 for all information block sizes and code rates. Detailed numbers for cross-check are provided in the separated excel sheets.

Figure 2. Required SNR for CBLER 10-2

Figure 3. Required SNR for CBLER 10-4

Observation 1: The proposed QC LDPC codes support a good and stable performance at down to BLER 10-4 for considered information block sizes and code rates.

Proposal 1: Two base graphs should be selected for eMBB data transmission.

Proposal 2: Boundary between two base graphs should be determined based on BLER performance.

Proposal 3: The proposed QC LDPC codes should be selected.

Observations and Proposals

In this contribution, we present the following observations and proposals:

Observation 1: The proposed QC LDPC codes support a good and stable performance at down to BLER 10-4 for considered information block sizes and code rates.

Proposal 1: Two base graphs should be selected for eMBB data transmission.

Proposal 2: Boundary between two base graphs should be determined based on BLER performance.

Proposal 3: The proposed QC LDPC codes should be selected.

- References

- Chairman’s note, 3GPP TSG RAN WG1 #88bis, Spokane, WA, 3-7 April 2017.