3GPP TSG RAN WG1 #89	R1-1708053

# Hangzhou, China, 15th – 19th May 2017

Agenda item:	7.1.4.3

Source: 	Samsung

Title: 	Implicit data transmission over PBCH using LDPC code

Document for:	Discussion and Proposal

Introduction

In RAN1#88bis, we had a conclusion for initial access and mobility:

Conclusions:

- For NR-PBCH,

- Following remaining issues need to be finalized in the next meeting

  - Transmission scheme

    - Including antenna port relationship to NR-PSS and NR-SSS

  - DMRS design

  - Time index indication signaling

  - Content and corresponding payload

    - Estimate

  - Channel coding

  - Mapping of NR-PBCH to SS blocks within NR-PBCH TTI

- Followings remaining issues need to be finalized by Nov. meeting

  - Delivery of remaining system information

  - Note that all RRC related aspects need to be finalized by Oct. meeting

To transmit data successfully, a conventional channel coding scheme encodes the data to generate parity bits. For our convenience, such a conventional method is referred to as an explicit encoding method. However, in this contribution, we introduce an implicit encoding method to transmit data based on quasi-cyclic (QC) LDPC codes. We will show that the implicit method is very simple and provides an efficient soft combining technique for multiple received blocks corresponding to LDPC codewords. Furthermore, since it doesn’t need a blind decoding for the QC LDPC code, but only for CRC code, the decoding complexity is very low.

SS Block or Timing Index Signal Based on QC Property

First, we revisit a study on important characteristics of QC LDPC code and then

Quasi-cyclic LDPC code

Let H be the mbZ×nbZ matrix given by

H=Pe11Pe21Pe12Pe22⋯⋯Pe1nbPe2nb⋮Pemb1⋮Pemb2⋱⋯⋮Pembnb

where eij∈{-1, 0, 1, 2, …, Z-1} are exponent indices of permutation matrices, nb and mb are the numbers of column and row blocks, respectively. Peij is just the circulant permutation matrix which shifts the identity matrix I(=P0) to the right by eij times for any integer eij, 0≤eij<Z. For simple notation, we denote the Z×Z zero matrix O by P-1. When H has full rank, we can assign (nb-mb)Z information bits to some nb-mb column blocks. (For our convenience, we call these kb=nb-mb column blocks information column blocks). Then the code with H is referred to as a QC LDPC code.

Quasi-Cyclic Property

For a vector c of length nZ, we define the following operator Ti():

c=c0,c1,c2,…,cn   ⟹ Tic≜Pic0,Pic1,Pic2,…,Picn

where cl=(cl×Z, cl×Z+1,…, cl×Z+Z-1), l=0, 1, …, n.

Example: Z=4, c= c0,c1,c2, c3, c0=(1,0,0,1), c1=(0,0,0,1), c2=(0,1,0,1), c3=(1,1,1,0)

c= T0(c)=1,0,0,1 ⋮ 0,0,0,1 ⋮ 0,1,0,1 ⋮ 1,1,1,0

T1c=0,0,1,1 ⋮ 0,0,1,0 ⋮ 1,0,1,0 ⋮ 1,1,0,1

T2c=0,1,1,0 ⋮ 0,1,0,0 ⋮ 0,1,0,1 ⋮ 1,0,1,1

T3c=1,1,0,0 ⋮ 1,0,0,0 ⋮ 1,0,1,0 ⋮ 0,1,1,1

(Quasi-Cyclic Property) For the QC-LDPC code C with parity-check matrix H, any codeword c∈C satisfies the following property

c=c0,c1,c2,…,cnb-1∈C     ⟹     Tic∈C

for all i, 0≤i<Z.

We can easily check the above QC property for QC LDPC codes as follows:

H⋅ TicT=H⋅Pic0Pic1⋮Picnb-1=H⋅PiOOPi⋯O⋯O⋮⋮OO⋱⋮⋯Pinb×nb⋅cT

=PiOOPi⋯O⋯O⋮⋮OO⋱⋮⋯Pimb×mb⋅H⋅cT

c∈C    ⟹    H⋅cT=0    ⟹   H⋅ TicT=0     ⟹   Tic∈C

Implicit data transmission to carry SS block or time index

We assume that NR-PBCH data u is first CRC-encoded as s=u, CRC and then s is LDPC-encoded by using H as c=s,p=u, CRC,p where p is the QC LDPC parity bit vector, as shown in Figure 1.

To transmit SS block index i, we send Tic instead of c at transmitter in Figure 1. Since Tic is also a legitimate codeword for QC LDPC code, the LDPC decoding can be performed without any pre-process for received signal r at receiver. In other words, we don’t need any blind decoding for QC LDPC code. However, the vector s=u, CRC' decoded by QC LDPC decoder does not pass the CRC-check unless the received signal is reverse-shifted as T-ir. Consequently, we need 1 LDPC decoding and a blind CRC decoding to recover both the NR-PBCH data and the SS block or time index i. Since the complexity for a blind CRC decoding is negligible compared to that of LDPC decoding, the total decoding complexity for implicit data transmission based on QC LDPC codes is significantly small.

Figure 1: Transmission of SS block index by quasi-cyclic shift

Observation 1: QC LDPC codes can provide an efficient decoding method to transmit SS block or time index implicitly, specifically LDPC blind decoding can be reduced.

Efficient combining of received blocks

We assume that NR-PBCH data u is CRC-encoded and LDPC-encoded as c=u, CRC,p and transmit Tx1c and Tx2c for SS block indices x1 and x2 (x2>x1), as shown in Figure 2. In addition, assume that the receiver failed to decode both Tx1c and Tx2c due to bad channel condition and therefore, a combining of two received signals corresponding to Tx1c and Tx2c is required. Here, the receiver (or decoder) doesn’t know the values of x1 and x2, but their difference Δx=x2-x1.

Figure 2: Implicit Transmission for different SS block indices

Let r(1) and r(2) be the received LLR vectors corresponding to Tx1c and Tx2c, respectively. By definition of operator Ti, we can find Tx1c=Tx2-Δxc=T-ΔxTx2c. Therefore, as shown in Figure 3, we can combine the received vectors as follows:

rcombined=r(1)+T-Δxr(2).

Based on the combined LLR vector rcombined, we can perform LDPC and CRC decoding similar to the decoding described in subsection 2.3. Since 1 LDPC decoding and a blind CRC decoding are carried out, the decoding complexity is still small. Finally, we can obtain the data and the value of x1 from the decoding based on rcombined. Furthermore, we can determine x2=x1+Δx.

Figure 3: Example of combining of received signals

Observation 2: Implicit data transmission using QC LDPC codes can provide an efficient combining method for multiple received blocks.

Performance Evaluation

In this section, the performance evaluation results (BLER) of LDPC and Polar codes are provided for a fixed code-length 864 and K=64, 80. Simulation assumptions are summarized in the following table. Details of evaluated polar codes are presented in [2], [3]. The evaluated QC LDPC codes are specified by the parity-check matrices given in the excel sheet of [4].

| Channel | AWGN |
| --- | --- |
| Modulation | QPSK |
| SS block index | 0, 1, 2, …, 15 (4 bits) |

| K | M | LDPC (OMS, Offset=0.25) |  | PC-CA Polar (L=8) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | CRC | K+CRC | CRC | K+CRC | PC-frozen (Reliable) | PC-frozen (Unreliable) | Mother N |
| 64 | 864 | 8 | 72 | 18 | 82 | 4 | 2 | 512 |
| 80 | 864 | 8 | 88 | 18 | 98 | 4 | 2 | 512 |

In Figures 4 and 5, the performance of LDPC and polar codes is comparable for LDPC decoding iteration = 50. However, while the required number of polar decoding with explicit/implicit data transmission, proposed in [5], is up to 16 for a blind decoding, that of LDPC decoding is 1 and therefore, the decoding complexity is not fair. If we allow more decoding iterations, e.g., 100 or 400, we can obtain a better performance for LDPC codes with still relatively lower decoding complexity. In other words, for fair decoding complexity, QC LDPC codes with implicit data transmission can perform better than polar codes with explicit/implicit data transmission.

Figure 4: Performance of LDPC and Polar codes (K = 64)

Figure 5: Performance of LDPC and Polar codes (K = 80)

Observation 3: For fair decoding complexity, QC LDPC codes with implicit data transmission can perform better than polar codes with explicit/implicit data transmission.

Proposal 1: QC LDPC coding is the most suitable for NR-PBCH in terms of decoding complexity, latency and coding performance. Therefore, QC LDPC codes should be adopted for NR-PBCH.

Observations and Proposals

In this contribution, we present the following observations and proposal:

Observation 1: QC LDPC codes can provide an efficient decoding method to transmit SS block or time index implicitly, specifically LDPC blind decoding can be reduced.

Observation 2: Implicit data transmission using QC LDPC codes can provide an efficient combining method for multiple received blocks.

Observation 3: For fair decoding complexity, QC LDPC codes with implicit data transmission can perform better than polar codes with explicit/implicit data transmission.

Proposal 1: QC LDPC coding is the most suitable for NR-PBCH in terms of decoding complexity, latency and coding performance. Therefore, QC LDPC codes should be adopted for NR-PBCH.

- References

- "Chairman’s note," 3GPP TSG RAN WG1 #88bis, Spokane, WA, 3-7 April 2017.

- R1-1708049, Samsung, "Comparison of CA polar codes and PC-CA polar codes," 3GPP TSG RAN WG1 #89, Hangzhou, China, 15-19, May 2017

- R1-1704247, Huawei, "Polar coding design for control channel," 3GPP TSG RAN WG1 #88bis, Spokane, WA, 3-7, April 2017

- R1-1708052, Samsung, "Channel Coding for PBCH, " 3GPP TSG RAN WG1 #89, Hangzhou

- R1-1704249, Huawei, "Channel Coding for PBCH," 3GPP TSG RAN WG1 #88bis, Spokane, WA, 3-7, April 2017