3GPP TSG-RAN WG1 #89	R1-1708291

Hangzhou, China, 15th – 19th May, 2017

Source:	Ericsson

Title:	On channel models for aerial UEs

Agenda Item:	6.2.8.1

Document for:	Discussion and Decision

# Introduction

In RAN1#88b, it was agreed that three scenarios will be supported for aerial studies, i.e. UMi AV, UMa AV and RMa AV [1]. The agreements are attached below. The scenarios correspond to the UMi/UMa/RMa scenarios defined in TR38.901 with addition of aerial UEs.  One of the remaining undecided items is the channel modelling for aerial UEs.

Agreement:

| Parameters |  | UMi AV | UMa AV | RMa AV |
| --- | --- | --- | --- | --- |
| Cell layout |  | Hexagonal grid, 19 micro sites, 3 sectors per site (ISD = 200m); FFS: if 37 micro sites, 3 sectors per site are needed | Hexagonal grid, 19 macro sites, 3 sectors per site (ISD = 500m); FFS: if 37 macro sites, 3 sectors per site are needed | Hexagonal grid, 19 macro sites, 3 sectors per site (ISD = 1732m; optionally ISD = 5000m); FFS: if 37 macro sites, 3 sectors per site are needed |
| BS antenna height |  | 10m | 25m | 35m |
| Carrier frequency |  | 2GHz | 2 GHz | 700 MHz; optionally 800 MHz |
| Total BS Tx power |  | 41/44 dBm for 10/20MHz | 46/49 dBm for 10/20MHz | 46/49 dBm for 10/20MHz |
| BS antenna configuration |  | 2Tx/2Rx cross polarized; Optionally (M, N, P) = (8, 4, 2) according to TR36.873 with 32 Tx ports and 32 Rx; other antenna configurations are not precluded | 2Tx/2Rx cross polarized; Optionally 8Tx/8Rx cross polarized; Optionally (M, N, P) = (8, 4, 2) according to TR36.873 with 32 Tx ports and 32 Rx; other antenna configurations are not precluded | 2Tx/2Rx cross polarized; other antenna configurations are not precluded |
| BS antenna pattern |  | FFS | FFS | FFS |
| BS antenna element pattern |  | For FD-MIMO, according to TR38.901 | For FD-MIMO, according to TR38.901 | For FD-MIMO, according to TR38.901 |
| BS antenna element gain + connector loss |  | 8dBi | 8dBi | 8dBi |
| UT location | Outdoor terrestrial/indoor terrestrial/aerial | Outdoor terrestrial and indoor terrestrial (same as UMi/UMa in TR38.901), and aerial UTs |  | Outdoor terrestrial and indoor terrestrial (same as RMa in TR38.901), and aerial UTs |
|  | LOS/NLOS (terrestrial) | LOS and NLOS |  |  |
|  | LOS/NLOS (aerial) | FFS | FFS | FFS |
|  | Height hUT (terrestrial) | For non FD-MIMO: FFS; For FD-MIMO: Same as UMi in TR 38.901 | For non FD-MIMO: FFS; For FD-MIMO: Same as UMa in TR38.901 | Same as RMa in TR38.901 |
|  | Height hUT (aerial) | Uniformly distributed between 1.5 m and [150]m; Optionally: fixed height values in the range between 1.5m and [150]m | Uniformly distributed between 1.5 m and [150]m; Optionally: fixed height values in the range between 1.5m and [150]m | Uniformly distributed between 1.5 m and [150]m; Optionally: fixed height values in the range between 1.5m and [150]m |

- Note 1: The above scenarios are for evaluation purposes.

- Note 2: The base station height will be revisited in case of evaluations on positioning enhancements.

In this contribution, we discuss pathloss models and fast fading models for aerial UEs. In a companion paper [2], the details of LOS probabilities of aerial UEs under the three scenarios are discussed.

# Pathloss Models for Aerial UEs

It was agreed in RAN1#88bis that aerial UE heights of 1.5m to [150] m will be supported.  Given this height range, both LOS and NLOS channel models are needed for all three scenarios considered. The LOS probability as a function of aerial UE height and 2D distance away from the base station is further discussed in the companion paper [2]. In the following, we discuss pathloss models for LOS and NLOS under RMa AV, UMa AV and UMi AV.

## LOS pathloss model

### RMa AV

The RMa LOS pathloss model defined in TR38.901 is valid for UE height below 10m and 2D distance up to 10km. Thus pathloss model for aerial UEs with heights above 10m is needed. While LOS path loss is typically close to or lower bounded by the free space path loss at shorter distances, at longer ranges the curvature of the earth causes an increased loss due to diffraction. Our measurement data collected in a helicopter measurement campaign show that EPM73 pathloss model provides a lower bound for the measurement data as shown in Figure 1(a) and Figure 1(b) for different UE heights, where the base station height is about 50m and the surrounding clutter ((trees, buildings, etc) is about 25m. So the effective base station height of about 25m is used in the EPM73 model. The UE height is about 50m above ground in Figure 1(a) and about 30m in Figure 1(b).  The effective UE heights of 25m and 5m were used in the EPM73 model.   Pathloss with the 3GPP RMa channel is also shown in Figure 1 with a simple extension of the ranges of UE height and 2D distance in the existing RMa LOS and NLOS models, where building height h=25m was used to reflect the surrounding clutter height.  A breakpoint around 10km can be clearly seen and the simple extension of the RMa LOS model does not fit well to the measured data.  For UE height of 50m as in Figure 1(a), the breakpoint distance is beyond 100km. Even for UE height of 25m in Figure 1(b), the breakpoint distance is also greater than 50km.

For aerial UEs at heights above 10m and at 2D distances beyond 10km, pathloss provided by a simple extension of the RMa model does not fit well to the measured data.

One way is to modify the breakpoint distance of RMa for UE heights above 10m to make it close  to  EPM73.  One possible modification is given below, in which the breakpoint distance is reduced for UE heights above 10m.

dBPRMA-AV =2πhbshutfcc,hut≤10m2πhbs(9+hut10)fcchut>10m 									Eq.(1)

In addition, some modification of the RMa model is needed to match the measured data.  One possible modification is summarized in Table 1. The corresponding pathloss with the modifications is shown in Figure 2. It can be seen that the RMa LOS pathloss model with the modified breakpoint distance and the modification in Table 1 fits the measured data much better. The modified model provides a nice lower bound for UE heights at both 25m and 50m.

Table 1: Proposed RAM-AV LOS pathloss model for UE heights above 10m.

| Scenario | LOS/NLOS | Pathloss [dB], fc is in GHz and d is in meters |  |
| --- | --- | --- | --- |
| RMa-AV | LOS |  | hUT>10m |

Consider to use the RMa-AV LOS pathloss model in Table 1 with the modified breakpoint distance described in Eq.(1) for UE heights above 10m.

- BS and UE effictive height =25m, f=1.8Ghz

(b) Effective heights: 25 m BS and 5m UE:, f=1.8Ghz

Figure 1: RMa LOS pathloss model vs. measured data

(b)  Modified RMa LOS model, BS height=25m,

UE effective height 5m,f=1.8GHz

- Modified RMa LOS model, BS effective height=25m,

UE effective height =25m, f=1.8GHz

Figure 2: Modified RMa LOS pathloss model vs. measured data

### UMa AV

For UMa, the LOS pathloss model defined in TR38.901 is valid for UE heights below 23m.  For aerial UEs at heights above 23m, the breakpoint distance defined in TR38.901 for UMa needs to be modified to account for the environment heights. In this case, it is reasonable to choose the environment height from a discrete uniform distribution over {1m, 3m, 6m, 9m, … ,23m}, i.e.

d'bp= 4 (hbs-he) (hut-he) fc/c  										Eq.(2)

where 
he=as defined in 38.901 for UMa,hut≤23muniform1,3,6,9,…,21,23 hut>23m  								Eq.(3)

Figure 3(a) shows the UMa LOS pathloss as a function of UE height and 2D distance, while Figure 3(b) also shows the pathloss with UE heights above 23m after the breakpoint modification. The curves represent the mean pathlosses.  It can be seen that for UEs above 10m, the breakpoint is beyond 5km.  Considering 500m ISD and three tiers, pathloss for 2D distances up to 5km should be enough for system evaluation purpose.

- UMa

- UMa AV

Figure 3: LOS pathloss for UMa-AV

Figure 4 compares the 3GPP UMa pathloss model with the helicopter measurement data with base station at the roof top, with UE height of 50m in Figure 4(a) and UE height of 30m in Figure 4(b). It can be seen that the UMa LOS model provides a close lower bound for the measured pathloss. Breakpoint is not clearly visible within 5km range though.

(a) UMa model vs. measured data, roof top BS, UE height 50m.

(b) UMa model vs. measured data, roof top BS, UE height 30m.

Figure 4: 3GPP UMa LOS pathloss model vs. measured data

With a slight modification of the breakpoint distance for UEs above 22.5m, the UMa LOS pathloss model can be extended for UMa -AV.

Consider to reuse the UMa LOS pathloss model of TR38.901 for UMa-AV with a slight modification of the breakpoint distance for UE heights above 22.5m.

### UMi AV

Figure 5 shows the LOS pathloss and breakpoint distance of UMi.  Again, the breakpoint distance is beyond 5km for UE heights above 15m. In other words, the pathloss is almost the same for UE at different heights above 15m.  Therefore, the UMi pathloss model can be easily extended for UMi-AV.

Figure 5: 3GPP UMi LOS pathloss and breakpoint distance as a function of UE height.

The breakpoint distance for UE heights above 15m is beyond 5km under UMi.

Consider to extend the UMi LOS pathloss model for UMi-AV.

## NLOS pathloss model

### RMa-AV

For RMa-AV, it can be seen in Figure 1 that the existing RMa NLOS model of TR38.901 works reasonablly well. It provides a upper bound for the measured pathloss data. Therefore, the existing RMa NLOS pathloss model can be extended to UE heights above 10m and 2D distance beyond 10km.

The existing RMa NLOS pathloss model of TR38.901 fits the measured data well.

Reuse the RMa NLOS pathloss model of TR38.901 for RMa-AV.

### UMa-AV

For UMa-AV, it can be seen in Figure 4 that the 3GPP UMa NLOS pathloss model in TR38.901 does not provide a upper bound pathloss for the measured data. Note that there is an additional optional 3GPP UMa NLOS pathloss model in TR38.901, which does not have explicit UE height restriction. The corresponding pathloss vs. the measured data is shown in Figure 6. It can be seen that the optional UMa NLOS pathloss model in TR38.901 provies a upper bound for the measured data.

The optional UMa NLOS pathloss model fits the measured data well.

Consider to use the existing optional UMa NLOS pathloss model of TR38.901 for UMa-AV.

(b) Optional UMa NLOS model vs. measured data,

roof top BS, UE height 30m.

- Optional UMa NLOS model vs. measured data,

roof top BS, UE height 50m.

Figure 6: Optional UMa NLOS pathloss model vs. measured data.

### UMi-AV

For UMi-AV, we are processing measurement data and will update this section once the processing is finished. Note that there is an additional optional 3GPP UMi NLOS pathloss model in TR38.901, which does not have UT height restriction. This model could serve as a starting point.

## Shadow fading

It is expected that the aerial UEs will experience much less shadowing compared to ground-level UEs since the space around the aerial UEs will be free from other objects. In LOS conditions, little or no shadowing is expected, while in NLOS there may be some more local signal strength variations due to terrain height or obstacle height variations. Therefore, for UE heights above 10m in RMa AV and above 22.5m in UMa AV/UMi AV, the following is tentatively proposed: 0 dB (LOS). For NLOS, we could tentatively use 4 dB for UE heights above 10m in RMa AV.

For UE heights above 10m in RMa AV and above 22.5m in UMa AV/UMi AV, consider to use 0dB shadowing for LOS.

For UE heights above 10m in RMa AV, consider to use 4dB shadowing for NLOS.

Table 2 is a summary of our overall proposals on pathloss models for aerial studies.

Table 2: Proposed pathloss models for aerial studies.

| Scenario | Pathloss [dB], fc is in GHz and distance is in meters | Shadow fading std [dB] | Applicability range of antenna height |
| --- | --- | --- | --- |
| RMa AV LOS (Aerial UE) | Same as RMa LOS defined in TR38.901 | Same as in TR38.901 |  |
|  | with modified breakpoint distance dRMa-AVBP defined in Eq.(1) | 0 |  |
| RMa-AV NLOS (Aerial UE) | Same as RMA NLOS defined in TR38.901 | Same as in TR38.901 |  |
|  | Same as RMA NLOS defined in TR38.901 | 4 |  |
| UMa LOS (Aerial UE) | Same as UMA LOS defined in TR38.901 | Same as in TR38.901 |  |
|  | Same as UMA LOS defined in TR38.901, with modified breakpoint distance d'BP as in Eq.(2) and Eq.(3) | 0 |  |
| UMa NLOS (Aerial UE) | Same as optional UMA NLOS defined in TR38.901 | Same as in TR38.901 |  |
| UMi LOS, (Aerial UE) | Same as UMi LOS defined in TR38.901 | Same as in 38.901 |  |
|  | Same as UMi LOS defined in TR38.901 | 0 |  |
| UMi NLOS (Aerial UE) | TO BE UPDATED |  |  |

Consider to use the pathloss models summarized in Table 2 for aerial studies.

# Fast Fading Model

For aerial vehicles, there is no surrounding objects at heights above surrounding buildings. It is therefore expected that there will be very little or no local scattering around the aerial UE, and thus the multi-cluster models in TR38.901 may no longer be valid for aerial UEs. In LOS conditions and when the base station is elevated above the local cluster such as in the RMa-AV and UMa-AV scenarios, some weak multipath may result from scattering and reflections from terrain and buildings along or below the propagation path. This multipath may become relatively stronger in NLOS conditions where the direct path is attenuated by obstructions or the terrain. However, the UMi-AV scenario can actually be linked to a “reverse” UMa scenario in that the base station is below the average rooftop height while the UE is well above it. Considering these observations, it is proposed to model the RMa-AV and UMa-AV scenarios at UE heights above 10 or 22.5 m, respectively, with simpler CDL-like channel models that account for the low amount of multipath, while the UMi-AV scenario for UE heights above 22.5 m is proposed to be modelled by the regular UMa channel but with the BS and UE spreads interchanged.

We propose to use the LOS CDL-D model defined in TR38.901 for the purpose when aerial UEs are above 22.5m in UMa AV and above 10m in RMa AV. While the existing cluster models are used for aerial UEs otherwise.

The fast fading for LOS aerial UEs above 22.5m in UMa AV and above 10m in RMa AV can be generated as follows:

- Follow steps 1- 3 in section 7.5 of TR38.901 (or section 7.3 of TR36.873) for UE dropping, LOS/NLOS assignment and pathloss calculation;

- Continue with steps 1- 4 in section 7.7.1 of TR38.901 with parameters defined for CDL-D for channel coefficient generation where the LOS AOD/AOA/ZOD/ZOA are used in the CDL-D models

The angle values of CDL-D model are further scaled according to section 7.7.5.1 of TR38.901 with the actual LOS AOA, LOS AOD, LOS ZOA and LOS ZOD of a dropped aerial UE as the desired mean AOA, AOD, ZOA and ZOD, respectively. Considering the expected lack of local scattering around the aerial UE and the corresponding lack of multipath, the desired angular spreads to which the CDL-D should be scaled are proposed in Table 3.

Table 3: Desired angular spreads

| Parameter | ASA | ASD |  | ZSA | ZSD | K-factor |
| --- | --- | --- | --- | --- | --- | --- |
| Unit | º | º |  | º | º | dB |
| UMa LOS | 0.5 | 0.5 |  | 0.1 | 0.1 | 20 |
| UMa NLOS | 1 | 1 |  | 0.3 | 0.3 | 10 |
| RMa LOS | 0.2 | 0.2 |  | 0.1 | 0.1 | 20 |
| RMa NLOS | 0.5 | 0.5 |  | 0.2 | 0.2 | 10 |

Note: the respective cluster spreads should be scaled accordingly to values that are a fraction 1/5 of the corresponding angular spreads. Further, an XPD of 20 dB should be used in LOS and 10 dB in NLOS.

The delay spread values of CDL-D can be scaled according to section 7.7.3 of TR38.901 with desired delay spread value DSdesired  given in Table 4.

Table 4: desired delay spread values for different scenarios

| Scenario |  |
| --- | --- |
| RMa -AV LOS | 10 ns |
| RMa-AV NLOS | 30 ns |
| UMa-AV LOS | 10 ns |
| UMa-AV NLOS | 30 ns |

The K-factor of the CDL-D model can be scaled to the desired K-factor according to section 7.7.6 of TR38.901.

Consider to adopt the CDL-D model with additional scaling on angular spread, delay spread as well as K-factor as specified in Table 3 and Table 4 for fast fading modeling aerial UEs above 22.5m under UMa-AV and above 10m under RMa-AV.

For aerial UEs below 22.5 m in UMa-AV or below 10m in RMa-AV, the existing multi-cluster fast fading models in TR38.901 are used.

For aerial UEs above 22.5 m in UMi-AV, consider reusing the existing multi-cluster fast fading model for UMa but with the angular spreads at base station and UE interchanged. While for aerial UEs below 22.5m, the existing multi-cluster fast fading models in TR38.901 are used.

# Conclusions

In this contribution, we have discussed channel modeling for aerial UEs.  We made the following observations:

Observation 1	For aerial UEs at heights above 10m and at 2D distances beyond 10km, pathloss provided by a simple extension of the RMa model does not fit well to the measured data.

Observation 2	With a slight modification of the breakpoint distance for UEs above 22.5m, the UMa LOS pathloss model can be extended for UMa -AV.

Observation 3	The breakpoint distance for UE heights above 15m is beyond 5km under UMi.

Observation 4	The existing RMa NLOS pathloss model of TR38.901 fits the measured data well.

Observation 5	The optional UMa NLOS pathloss model fits the measured data well.

Based on the discussion in this contribution we propose the following:

Proposal 1	Consider to use the RMa-AV LOS pathloss model in Table 1 with the modified breakpoint distance described in Eq.(1) for UE heights above 10m.

Proposal 2	Consider to reuse the UMa LOS pathloss model of TR38.901 for UMa-AV with a slight modification of the breakpoint distance for UE heights above 22.5m.

Proposal 3	Consider to extend the UMi LOS pathloss model for UMi-AV.

Proposal 4	Reuse the RMa NLOS pathloss model of TR38.901 for RMa-AV.

Proposal 5	Consider to use the existing optional UMa NLOS pathloss model of TR38.901 for UMa-AV.

Proposal 6	For UE heights above 10m in RMa AV and above 22.5m in UMa AV/UMi AV, consider to use 0dB shadowing for LOS.

Proposal 7	For UE heights above 10m in RMa AV, consider to use 4dB shadowing for NLOS.

Proposal 8	Consider to use the pathloss models summarized in Table 2 for aerial studies.

Proposal 9	Consider to adopt the CDL-D model with additional scaling on angular spread, delay spread as well as K-factor as specified in Table 3 and Table 4 for fast fading modeling aerial UEs above 22.5m under UMa-AV and above 10m under RMa-AV.

Proposal 10	For aerial UEs below 22.5 m in UMa-AV or below 10m in RMa-AV, the existing multi-cluster fast fading models in TR38.901 are used.

Proposal 11	For aerial UEs above 22.5 m in UMi-AV, consider reusing the existing multi-cluster fast fading model for UMa but with the angular spreads at base station and UE interchanged. While for aerial UEs below 22.5m, the existing multi-cluster fast fading models in TR38.901 are used.

# References

Chairman’s note, RAN1#88bis

R1-1708292, “On line of sight probability for aerial UEs”, Ericsson, RAN1#89

M.N Lustgarten and James A. Madison, “An Empirical Propagation Model (EPM-73)”, IEEE Trans on Electromagnetic Compatibility, Vol.EMC-19, No.3, August 1977.