3GPP TSG RAN WG1 Meeting #89                                     R1-1708322

Hangzhou, P.R. China, 15th – 19th May 2017

Agenda Item:	7.1.1.1.1

Source:	InterDigital Inc.

Title:	On Design of Synchronization Signal Sequences in NR

Document for:	Discussion and Decision

# Introduction

In RAN1 #88bis Meeting the synchronization signals for NR design were agreed for the following [1]:

Agreements:

- Support about 1000 hypotheses provided by NR-PSS/SSS to represent NR physical cell ID for NR-SS design.

  - FFS: indication of radio frame boundary by NR-SSS

- In both single beam and multi-beam scenario, support only time division multiplexing of PSS, SSS and PBCH.

Agreements:

- The number of antenna ports of NR-SSS is 1

Agreements:

- Working assumption: Number of PSS sequences: 3

  - PSS sequence details:

    - Frequency domain-based pure BPSK M sequence (fixing the time/freq. offset ambiguity)

      - 1 polynomial:  Decimal 145 (i.e. g(x) = x7 + x4 + 1)

      - In freq. domain 3 cyclic shifts (0, 43, 86) to get the 3 PSS signals

      - Initial poly shift register value: 1110110

    - FFS modified ZC: 2 ZC sequences concatenation or interleaving in time or freq., 4 ZC sequences concatenation in time

- Number of SSS signals: 1000 post-scrambling

- PSS sequence length: 127 for frequency domain-based pure BPSK M sequence

  - Note that PSS will be mapped to consecutive 127 subcarriers

- SSS sequence length: 127

- Subcarrier spacings for PSS/SSS for difference freq. ranges: 15kHz/30kHz for below 6 GHz, and 120kHz/240kHz for above 6 GHz

  - Note: RAN1 assumes that RAN4 will decide it depending on frequency ranges

- SSS sequence details: Long M-sequence with scrambling

- SYNC frequency raster: RAN1 assumes that RAN4 will decide it

- SS burst set periodicity default value for initial cell selection: 20/20 msec

  - Note that RAN1 assumes that RAN4 will investigate requirements

- Time index indication: PBCH conditioned that mobility and HO related requirements can be met

  - Note: RAN1 assumes that RAN2 will check against to RAN2 requirements

- PBCH BW: 288 subcarriers, 2 OFDM symbols (additional symbols if MIB size larger than assumed)

- PBCH phase reference: DMRS

- PBCH TTI: 80 msec

NR system operation is expected to support very high frequencies (e.g. > 6 GHz), for example, operation may initially be expected up to 52 GHz and later up to 70GHz or higher. For these high carrier frequencies, it is expected that single and multi-beam operation may be required due to the large path and penetration losses at these frequencies. It has been agreed that both multi-beam, and single-beam, based approaches need to be studied for NR [2].

In this contribution we further discussed and evaluated synchronization signal sequence design for NR-PSS and NR-SSS including M-sequence parameters and scrambling as well as the mappings from Cell IDs to NR-SS sequences.

# Synchronization Signal Sequence

In this section, we consider the NR-SS sequence design. At least 1000 hypotheses should be provided by NR-PSS/SSS to represent NR physical cell ID for NR-SS design. It may be further investigated whether NR-PSS/SSS can be used to indicate information other than the NR physical cell ID. It may be possible to use NR-SSS to indicate information such as radio frame boundary, however further investigation is needed. Since use of the NR-SSS to indicate other information may increase the number of hypotheses for NR-SSS detection which will result in increased detection complexity, therefore the use of NR-SSS to indicate additional information may need to be further studied. The trade-offs between benefits of obtaining other information from NR-SSS and increased detection complexity of NR-SSS should be carefully investigated.

## NR-PSS Sequence

It was agreed as a working assumption that the number of NR-PSS sequences is three. Frequency domain-based pure BPSK M sequence is assumed for NR-PSS sequence. This was proposed to mitigate the detection ambiguity due to the time and/or frequency offsets.

A single polynomial was agreed for generation of NR-PSS sequence. Such polynomial is based on decimal 145, and the following polynomial was assumed,

gx=x7+x4+1

In frequency domain, three cyclic shifts 0, 43 and 86 are used to generate the three NR-PSS signals. The initial polynomial shift register value can be set to 1110110.

It was for further study that modified Zadoff-Chu (ZC) sequences should be used or not. Modified ZC sequences may include two ZC sequences concatenation or interleaving in either time and/or frequency. Furthermore, four ZC sequences concatenation in time may also be considered.

NR-PSS sequence length was further agreed to be 127 for frequency domain-based pure BPSK M sequence which will be mapped to consecutive 127 subcarriers accordingly.

## NR-SSS Sequence

It was agreed that NR-SSS sequence should assume long M-sequence with scrambling. NR-SSS sequence length was also agreed to be 127 which is the same as the length of NR-PSS sequence.

The NR-SSS sequence denoted as d0, …, dN-1 may be a length-N binary sequence which is scrambled with a scrambling sequence provided by the primary synchronization signal (i.e., NR-PSS). NR-SSS sequence may be expressed as

dn=zjm0nvm1n ,         n=0, 1, ,…, N-1.

Generation of the sequence zjm0(n):

The sequence zjm0(n) may be defined as a m0 cyclic shift of the m-sequence zj(n)  based on the equation below

zjm0n=zj(n+m0 mod N)

where N is 127.

The m-sequence zj(n) may be generated by the following

zjl=1-2xjl,

where j=0, 1, 2. The value of xl is either 0 or 1.

x1l could be implemented by a shift register x(l+7)=xl+4+xlmod2, x2l could be implemented by x(l+7)=xl+3+xlmod2, and x3l could be implemented by x(l+7)=xl+1+xlmod2;

Initial conditions could be set to x0=0, x1=0, x2=0, x3=0, x4=0, x5=0, x6=1

Generation of the sequence v(m1)(n):

The scrambling sequence v(m1)(n) is used to scramble secondary synchronization signal (i.e., NR-SSS). The scrambling sequence v(m1)(n) depends on the primary synchronization signal (i.e., NR-PSS).

The sequence v(m1)(n) may be defined as a m1 cyclic shift of the m-sequence v(n)  according to the equation below:

vm1n=v(n+m1 mod N)

where N is 127.

vn can be generated by the following;

vl=1-2x(l)

The value of xl is either 0 or 1.

xl could be implemented by a shift register xl+7=xl+3+xl+2+xl+1+xl mod 2

The initial conditions can be set to x0=0, x1=0, x2=0, x3=0, x4=0, x5=0, x6=1

## NR Physical Cell ID Mappings

The physical-layer cell identity group NID(1) can be defined or mapped by the following equation:

NID(1)=jN+m0

for j=0, 1, 2 and 0≤m0≤N-1

NID(2) denotes the physical-layer cell identity within the physical-layer cell identity group which is denoted as NID(1). The physical cell ID can be mapped by the following

NPCI=3NID(1)+NID(2)

Therefore the physical cell ID can be mapped by the parameters j, m0 and m1 based on the equation below:

NPCI=3(jN+m0)+m1

for 0≤m0≤126 and 0≤m1≤2

Proposal 1: Long M sequence with scrambling can be considered for generation of NR-SSS sequence.

# Synchronization Signal Bandwidth

The SS transmission bandwidth depends on subcarrier spacing, and is proportional to both SS sequence length and subcarrier spacing. For example, when a length 127 SS sequence with small subcarrier spacing (e.g., 120 KHz) is used, we may need as low as 17.28 MHz for SS transmission bandwidth. When a length 127 SS sequence with large subcarrier spacing (e.g., 240K Hz) is used, we may need as high as 34.56 MHz for SS transmission bandwidth. It is similar for lower frequency category below 6GHz. Example frequency range category, subcarrier spacing and corresponding bandwidth is summarized in Table 1.

Table 1. Example frequency range category, subcarrier spacing and corresponding bandwidth

|  | Below 6 GHz |  | Above 6 GHz |  |
| --- | --- | --- | --- | --- |
| Minimum Carrier Bandwidth | 5 MHz | 10 MHz | 40 MHz | 80 MHz |
| SS Transmission Bandwidth | 2.16 MHz | 4.32 MHz | 17.28 MHz | 34.56 MHz |
| Subcarrier Spacing | 15KHz | 30KHz | 120KHz | 240KHz |

Proposal 2: Maximum NR SS bandwidth of 34.56 MHz and 4.32 MHz may be considered for frequency range above and below 6 GHz respectively.

# Synchronization Signal Evaluation

For initial access evaluation, we can consider the performance metrics below:

- PAPR

- Joint PSS/SSS misdetection rate

- Residual timing offset and carrier frequency offset

- Cross-correlation performance of NR-SS sequences

The evaluation results are provided as follows. Additional results are provided in Appendix.

PAPR

Figure 1 shows PAPR for long M sequence for NR-SSS.

- Figure 1. PAPR for long M sequence

Joint NR-PSS/SSS misdetection rate

Figure 2 shows misdetection rate for joint NR-PSS/SSS detection. Long M sequence is used to generate NR-SSS sequence.

- Figure 2. Joint NR-PSS/SSS Misdetection Rate

# Conclusions

In this contribution, we consider the SS sequence design and related observations for initial access for single and multi-beam initial access. We have the following proposals:

Proposal 1: Long M sequence with scrambling can be considered for generation of NR-SSS sequence.

Proposal 2: Maximum NR SS bandwidth of 34.56 MHz and 4.32 MHz may be considered for frequency range above and below 6 GHz respectively.

# References

- Chairman’s Notes, 3GPP TSG-RAN WG1 #88bis, 3rd – 7th April 2017, Spokane, USA

- Chairman’s Notes, 3GPP TSG-RAN WG1 #86, 22nd – 26th August 2016, Gothenburg, Sweden

# Appendix: Evaluation Assumptions And Results

Based on the evaluation assumptions described in 3GPP TR38.802, link-level evaluation assumptions is provided in Table A1.

- Table A1. Link-level Evaluation Assumptions

| Parameter | Value |  |
| --- | --- | --- |
|  | For below 6GHz | For above 6GHz |
| Carrier Frequency | 4GHz | 30GHz |
| Default subcarrier spacing | 15kHz, 30kHz | 120kHz, 240kHz |
| Channel Model | Table A.1.5-1 in [2] Note: Zero interfering TRP should be assumed for above 6GHz. |  |
| Antenna Configuration |  |  |
| UE speed |  |  |
| Number of interfering TRPs |  |  |
| Target received baseband SNR | -6dB Note: Proponents provide results over a range of SNR including -6 dB. |  |
| NR-PSS/SSS detection | Note: Good one-shot detection probability should be taken into account for reporting joint PSS/SSS misdetection rate, the residual timing error and frequency error. Note: For reporting joint PSS/SSS detection latency, accumulation among SS blocks in different SS burst sets is not precluded. |  |
| Frequency Offset | - For initial acquisition TRP: uniform distribution +/- 0.05 ppm UE: uniform distribution +/- 5, 10, 20 ppm (each company to choose one) - Non-initial acquisition TRP: uniform distribution +/- 0.05 ppm UE: uniform distribution +/- 0.1 ppm Note: The following target requirements should be taken into account in NR-PSS/SSS design - Robustness against initial frequency offset up to 5 ppm - 10 ppm as optional requirement |  |
| Timing arrival difference from interfering TRPs | - Synchronous scenario: Uniformly distributed from -3/N us to +3/N us, where N denotes scaling factor (N= Default subcarrier spacing (kHz)/15(kHz)). - Asynchronous scenario: Uniformly distributed from -1 ms to 1 ms |  |
| BS Tx power for NR-SS | Note: Companies describe whether it is assumed that the TX power is kept constant when comparing different sequence proposals. |  |

Joint NR-PSS/SSS misdetection rate

Figure 2 shows misdetection rate for joint NR-PSS/SSS detection. Long M sequence is used to generate NR-SSS sequence.

- Figure 2. SS Misdetection Rate

- Residual timing offset and carrier frequency offset

Figure 3 shows residual timing offset and Figure 4 shows residual carrier frequency offset.

- Figure 3. Residual timing offset

- Figure 4. Residual carrier frequency offset

Table 3 shows 50% and 90% tile residual timing offset and carrier frequency offset.

- Table 3. 50% and 90% tile residual timing offset and carrier frequency offset

| 50% and 90% tile | No interferer (120kHz SCS, above 6GHz) |  |  |  |
| --- | --- | --- | --- | --- |
|  | Residual timing offset (us) |  | Residual carrier frequency offset (kHz) |  |
| M Sequence | 0.065 | 5.30 | 5.20 | 12.39 |
| ZC – Concatenated | 0.065 | 5.40 | 5.22 | 12.59 |
| ZC Interleaved | 0.065 | 5.20 | 5.22 | 12.56 |
| ZC Conj-Interleaved | 0.065 | 5.13 | 4.91 | 11.93 |

- Table 4. 50% and 90% tile residual timing offset and carrier frequency offset

| 50% and 90% tile | No interferer (240kHz SCS, above 6GHz) |  |  |  |
| --- | --- | --- | --- | --- |
|  | Residual timing offset (us) |  | Residual carrier frequency offset (kHz) |  |
| M Sequence | 0.016 | 1.54 | 8.73 | 22.23 |
| ZC – Concatenated | 0.016 | 1.48 | 9.34 | 23.11 |
| ZC Interleaved | 0.016 | 1.64 | 9.80 | 23.71 |
| ZC Conj-Interleaved | 0.016 | 1.40 | 8.65 | 21.30 |

Cross-correlation performance of NR-SS sequences

- Figure 5 shows cross-correlation performance for NR-SSS using m sequence.

- Figure 5. Cross-correlation performance for m sequence