3GPP TSG RAN WG1 Meeting #89                          R1-1708453

Hangzhou, P.R. China 15th – 19th May 2017

Source:	NTT DOCOMO, INC.

Title:	Evaluation for Beam Management

Agenda Item:	7.1.2.2.3

Document for: 	Discussion

- Introduction

Beam management is a new topic in NR and the evaluation is very important to the network. In this contribution, we present our views on evaluation method for beam management and provide initial simulation results.

- Discussion on Evaluation Methodology

A realistic evaluation model shall be established to evaluate NR systems with beam management functionalities. Compared to the conventional system level evaluation of a cellular system at least the following new features shall be considered.

Antenna array structure:

The latest LTE system level evaluation assumes single panel antenna array. In NR, multi-panel antenna array shall be considered. The latest LTE system level evaluation assumes fixed TXRU to antenna element mapping throughout the simulation. In NR, with the introduction of the hybrid beamforming architecture, analog beamforming, which varies the TXRU to antenna element mapping from time to time, shall be considered. It is noticed that pure analog beamforming is only feasible in wideband. Such impact, e.g., on user scheduling algorithm, interference modelling shall be taken into consideration in system level evaluation.

UE to network association:

In the LTE system level evaluation, UE and the networks are associated with cells. Different types of cells are modeled, e.g., macro/micro-cells with hexagonal deployment and wider coverage, small cells with random location and shorter coverage. Although the parameters used in a real network may differ a lot from the evaluation assumptions, the parameters used for LTE evaluation were unified as much as possible to ensure comparable evaluation results. In NR beam management, UE shall be associated with the network via beams. The characteristics of beams, e.g., beam shape, candidate beam directions, beam number, etc., shall be defined and unified as much as possible, for the purpose of generating comparable results. Note that similar practice was done during the LTE evaluation, e.g., via specifying the antenna gain pattern and bearing/tilting angles.

Categorization of the schemes:

In the LTE evaluation, transmission schemes are classified in different categories, e.g., SU-MIMO, MU-MIMO, CoMP, etc. Candidate technologies are proposed and compared within each category, specification supported are decided to be made for the most necessary technologies within each category. For the evaluation of beam management, it is necessary to make similar approach. Candidate categories include beam acquisition procedure, beam recovery procedure, transmission procedure. For beam acquisition and beam recovery, evaluation shall be made to verify the efficiency and effectiveness of beam management procedure via checking the coverage, RS overhead, latency, robustness of the link, etc. For transmission, evaluation shall be made to verify the performance. It shall be further discussion, whether categorization shall be made to differentiate the cases of single-panel/TRP single-beam based association, single-panel/TRP multi-beam based association, multi-panel/TRP multi-beam based association etc.

Proposal 1: RAN1 discuss general categorizations of beam management technologies and define evaluation assumptions accordingly, e.g.,

- Cat. 1: Beam acquisition including at least the following aspects:

  - Beam selection methods considering different number of beams and panels

  - RS density for beam acquisition, e.g., numerology, periodicity in time domain and frequency domain

  - Beam reporting, e.g., beam grouping methods, spatial correlations for multiple beam pairs

- Cat. 2: Beam recovery including at least the following aspects:

  - Impact of blockage on multi beam based system

  - Number of monitored beam pair links to provide robustness

  - Beam recovery methods considering the RS overhead and latency.

- Preliminary Beam Management Evaluation Results

Based on the above discussion of beam management evaluation requirements, we provide some initial investigation results for beam selection and robust multi-beam monitoring.

  - Beam selection with multi-panels

For the RS design in NR, multi-panel antenna array shall be considered compared to LTE which assumes single panel antenna array. Beam selection based on one panel had been assumed in NR-MIMO calibration. In our previous contribution [1], we evaluate the performance gap between multi-panel based beam selection and single panel based beam selection with LLS platform. Here we would like to further investigate the multi-panel impact on the beam selection with both SLS and LLS platform and more realistic parameter assumptions are adopted, e.g., codebook based digital beamforming.

Following analog beamforming evaluation cases are established.

Case 1: All TXRUs use same analog beam and beam selection is based on a single TXRU, i.e., TXRU 0 of the first panel. In case 1, TRP will conduct the beam sweeping among the limited set of DFT beams, and the best analog beam pair is selected based on the criteria of maximizing receive power. For both TRP and UE side, all TXRUs will use the same analog beam (selected by TXRU0) for data transmitting and receiving.

Case 2: TXRUs can used different/same analog beams and beam selection is done for each TXRU independently. In case 2, different TXRUs can select different analog beams. All the possibilities should be traversed, and the best analog beam corresponding to each TXRU can be selected based on the criteria of maximizing the overall receive power.

For the digital beamforming part, multiple cases are evaluated based on different simulation platforms.

For UE side digital beamforming, two methods are adopted:

- Option 1: SVD based

- Option 2: MMSE receiver

For TRP side digital beamforming, two methods are adopted:

- Option 1: SVD based

- Option 2: Codebook based

We evaluated different options for case 1 and cased 2 for better understanding of the multi-panel impact on beam selection.

Fig. 1 shows the performance comparison between different case 1 and case 2 based on LLS. For UE side digital beamforming, both case 1 and case 2 adopt the MMSE receiver, while for TRP side digital beamforming, SVD and codebook based digital beamforming are adopted for comparison. Evaluation assumptions following the MIMO calibration and beam management evaluation assumptions are listed in Table A-I. From the initial simulation results, it can be observed a nearly 1 dB gap between case 1 and case 2. The possible reason is that higher multi-path diversity gain can be achieved by case 2 in comparison with case 1. Meanwhile, although codebook-based digital beamforming may bring approximately 3 dB performance degradation in comparison with ideal SVD-based beamforming, the advantage of case 2 can still be obviously reflected.

Figure 1 Beam selection based on multi/single panel(s) in LLS

Fig. 2 shows the spectrum efficiency difference between single panel beam selection and multi-panel beam selection in SLS platform. SVD based digital beamforming are adopted for TRP side and MMSE receiver are adopted in the UE side. From the initial simulation results, we can observe that single panel based beam selection have better performance that multi panel based beam selection which is different from the evaluation results based on LLS. However, factors like interference modeling, scheduling mechanism will impact the system performance, the impact of multi panel beam selection should be further investigated.

Figure 2 Spectral efficiency of beam selection based on multi/single panel(s) with different rank in SLS

  - Investigation of multi-beam monitoring

For communication at high frequency band, hybrid beamforming and multi-beam system are considered to overcome the high path loss between TRP and UE. In such system, good TRP-UE beam pair(s) shall be established based on beam management procedures. However, the performance of the selected beam pair(s) may decrease, due to multiple reasons, e.g., channel variation, UE rotation, movement, dynamic blockage, etc. So, the best TRP/UE beam pair(s) may need to be updated.

To solve these problems, UE can be configured to monitor multiple beam pair links. In our previous contribution [1], we investigate the appropriate value of monitored beam pair links to achieve a good balance between the overhead and the performance. In this contribution, we further investigate the features of the monitored beams to achieve better system performance, e.g., the capability to overcome the high path loss caused by UE rotation, blockage, etc.

Instruction of the simulation method:

Method 1(Exhaustive): UE selects N best beam pair links out of all the beam pair links in TTI0 based on RSRP, and selects the best one beam pair link for transmission. Then in every X ms (in our evaluation, X = 30), UE reselects the best one beam pair from the N beam pair links.

Method 2 (UE panel based): For each panel, UE selects N/Npanel best beam pair links out of all the beam pair links in TTI0 based on RSRP, and selects the best one beam pair link for transmission. Then in every X ms (in our evaluation, X = 30), UE reselects the best one beam pair from the N beam pair links.

Method 3-1(Rx beam based best): For each Rx beam, UE selects N/NRx beam best beam pair links out of all the beam pair links in TTI0 based on RSRP, and selects the best one beam pair link for transmission. Then in every X ms (in our evaluation, X = 30), UE reselects the best one beam pair from the N beam pair links.

Method 3-2 (Rx beam based random): For each Rx beam, UE selects N/NRx beam beam pair links out of all the beam pair links in TTI0 based on RSRP, wherein the N/NRx beam beam pair link including the beam pair link who has the highest RSRP and the others are randomly selected. Then selects the best one beam pair link for transmission. Then in every X ms (in our evaluation, X = 30), UE reselects the best one beam pair from the N beam pair links.

Evaluation assumptions following the MIMO calibration and beam management evaluation assumptions are listed in Table A-II. We choose an extreme case with UE rotation of 180° to investigate the impact of monitored beam subset. Results are summarized in Fig. 3. From the initial simulation results, we can observe that after a rotation of 180°, the UE panel based beam selection scheme has the best performance as expected, and the Rx beam based best selection scheme has the worst performance.

Figure 3 Wideband SINR of rotation angle equals to 180 degree

- Summary

In this contribution, we described our views on evaluation assumption for beam management and provide initial simulation results comparison between no beam management and different management schemes, we made the following proposal.

Proposal 1: RAN1 discuss general categorizations of beam management technologies and define evaluation assumptions accordingly, e.g.,

- Cat. 1: Beam acquisition including at least the following aspects:

  - Beam selection methods considering different number of beams and panels

  - RS density for beam acquisition, e.g., numerology, periodicity in time domain and frequency domain

  - Beam reporting, e.g., beam grouping methods, spatial correlations for multiple beam pairs

- Cat. 2: Beam recovery including at least the following aspects:

  - Impact of blockage on multi beam based system

  - Number of monitored beam pair links to provide robustness

  - Beam recovery methods considering the RS overhead and latency.

References

- R1-1705720, Evaluation for beam management, NTT DOCOMO. INC.

Appendix

Table 1 Evaluation assumptions for multi-panel beam selection in LLS

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | 30 GHz |
| Bandwidth | 80MHz |
| Subcarrier Spacing | 60kHz |
| Channel Model | CDL-B • delay spread =100ns • UE speed=3km/h. • The angles of BS, i.e., AoD, ZoD, are uniformly distributed within [-60, 60] degrees in azimuth domain and [90, 135] degrees in zenith domain, and those of UE, i.e., AoA, ZoA, are uniformly distributed within [-180, 180] degrees in azimuth domain and [45, 90] in zenith domain, via applying uniform-distribution desired mean angle in Section 7.7.5.1 in TR 38.900 accordingly. |
| TXRU mapping to antenna elements | One TXRU per panel per polarization |
| TXRU mapping weights | 2D TXRU virtualization weights for each panel is the Kronecker product between vertical and horizontal weight vectors taken from DFT, i.e., 2D sub-array partition model defined in TR36.897. |
| Criteria for beam selection for serving TRP | Select the best beam pair among the limited set of DFT beams, based on the criteria of maximizing receive power after beamforming. |
| Constraints for the range of selective beams per TRP sector | - Beam directions for TRP: -- Azimuth angle [-7*pi/16 -5*pi/16 -3*pi/16 -pi/16 pi/16 3*pi/16 5*pi/16 7*pi/16] -- Zenith angle [pi/8 3*pi/8 5*pi/8 7*pi/8] - Beam directions for UE: -- Azimuth angle [-3*pi/8 -pi/8 pi/8 3*pi/8]; -- Zenith angle [pi/4 3*pi/4]; |
| BS Antenna Configuration | (M, N, P, Mg, Ng) = (4,8,2,2,2)；(dV, dH) = (0.5, 0.5)λ (dg,H,dg,V) = (4.0, 2.0)λ |
| BS array orientation | azimuth 0 degree; mechanic downtilt: 0 degree |
| UE Antenna Configuration | (M, N, P, Mg, Ng) = (2, 4, 2, 1, 2); (dV,dH) = (0.5, 0.5)λ. (dg,V,dg,H) = (0, 0)λ. Θmg,ng=90; Ω0,1=Ω0,0+180; |
| UE array orientation | ΩUT, uniformly distributed on [0,360] degree, ΩUT,= 0 degree, ΩUT, = 0 degree |
| BS antenna pattern | HPBW=65° Max Gain = 8dBi SLAV=Am=30 polarization angles are -45° and +45° |
| UE antenna pattern | HPBW=90° Max Gain = 5dBi SLAV=Am=25 polarization angles are 0° and +90° |
| Metric | CDF of received SNR |

Table 2 Evaluation assumptions for beam acquisition in SLS

| Attributes | Values or assumptions |
| --- | --- |
| Carrier Frequency | 30 GHz |
| Mode | DL only |
| Bandwidth | 40MHz |
| Subcarrier Spacing | 60kHz |
| Channel Model | UMa in TR 38.900 |
| TXRU mapping to antenna elements | One TXRU per panel per polarization |
| TXRU mapping weights | 2D TXRU virtualization weights for each panel is the Kronecker product between vertical and horizontal weight vectors taken from DFT, i.e., 2D sub-array partition model defined in TR36.897. |
| Criteria for beam selection for serving TRP | Select the best beam pair among the limited set of DFT beams, based on the criteria of maximizing receive power after beamforming. |
| Constraints for the range of selective beams per TRP sector | - Beam directions for TRP: -- Azimuth angle [-5*pi/16 -3*pi/16 -pi/16 pi/16 3*pi/16 5*pi/16] -- Zenith angle [5*pi/8 7*pi/8] - Beam directions for UE: -- Azimuth angle [-3*pi/8 -pi/8 pi/8 3*pi/8]; -- Zenith angle [pi/4 3*pi/4]; |
| ISD | 500m |
| BS Tx power | 43dBm |
| BS Antenna Configuration | (M,N,P,Mg,Ng) = (4,8,2,2,2) (dH,dV) = (0.5, 0.5)λ (dg,H,dg,V) = (4.0, 2.0)λ |
| BS array orientation | azimuth 0 degree; mechanic downtilt: 0 degree |
| UE Configuration | (M, N, P, Mg, Ng) = (2, 4, 2, 1, 2); (dV,dH) = (0.5, 0.5)λ. (dg,V,dg,H) = (0, 0)λ. Θmg,ng=90; Ω0,1=Ω0,0+180; The polarization angles are 0 and 90 |
| UE array orientation | ΩUT, uniformly distributed on [0,360] degree, ΩUT,= 0 degree, ΩUT, = 0 degree |
| BS antenna pattern | See Table A.2.1-6 in TR 38.802 |
| UE antenna pattern | See Table A.2.1-8 in TR 38.802 |
| MIMO mode | SU-MIMO with rank=1 |
| Scheduling algorithm | Round robin scheduler |
| BS antenna height | 25m |
| UE antenna height | Same as 3D-UMa in TR36.873 |
| UE antenna gain | 5dBi |
| Noise figure for BS | 7dB |
| UE receiver noise figure | 10dB |
| UE distribution | 100% Outdoor in cars: 30km/h (The velocity only influence the Doppler shift) 10 users per TRP |
| Configurations for channel blockage | Blockage Model-A K=5 in TR38.900 |
| Configurations for UE rotation | UE is rotating with random direction only for bearing angle per drop based on UE rotation feature in TR38.900 with fixed speed of 50 rpm |
| Metric | CDF of coupling loss CDF of wideband SINR with beamforming |