3GPP TSG RAN WG1 Meeting #89							R1-1708467

Hangzhou, P.R. China 15th – 19th May 2017

Source:	NTT DOCOMO, INC.

Title:	Initial views on DCI fields

Agenda Item:	7.1.3.1.4

Document for: 	Discussion and Decision

- Introduction

In this contribution, we list up the potential DCI fields for NR.

- DCI fields

  - Potential DCI fields

Following summarizes and initial estimate of the list of all necessary DCI fields.

| DL/UL | Field description | No. of bits | Comments |
| --- | --- | --- | --- |
| Common | Carrier indicator [1] | [0 - 4] | Configurable with cross-carrier scheduling |
|  | Resource block assignment [2] | [10 - 30] | Configurable with bandwidth of a bandwidth part |
|  | Data duration and starting timing [2] | [0 - 4] | Configurable |
|  | HARQ process number [3] | [2 - 4] | Configurable |
|  | Modulation and coding scheme | [5] | Always |
|  | New data indicator | [1] | Always |
|  | Redundancy version | [2] | Always |
|  | CBG index for re-transmission [4, 5] | [0 - 4] | Configurable with CBG-based re-transmission |
|  | Layers and DMRS ports/patterns | [2 - 6] | Part of MIMO configuration |
|  | QCL indication [6] | [2] | Part of MIMO configuration |
|  | SRS request | [2] | Configurable |
| DL | HARQ-ACK timing/resource [7] | [2 - 4] | Always |
|  | Indication for HARQ-ACK multiplexing (e.g., DAI) [8] | [2 - 3] | Configurable |
|  | TPC command for PUCCH | [2] | Always |
|  | Rx beam indication | [2] | Configurable |
| UL | Aperiodic CSI-RS | [2] | Configurable |
|  | TPC command for PUSCH | [2] | Always |
|  | Precoding information and no. of layers | [2 - 4] | Configurable |

  - Notes on some DCI fields

- Carrier indicator [1]

  - RAN1 agreed to support NR carrier aggregation with CCs of up to 16.

  - In order to realize cross-carrier scheduling for up to 16 CCs, CIF size is up to 4.

  - It is FFS whether the CIF size is configurable.

- Resource block assignment [2]

  - For LTE, in case of 100 PRBs, the RA field size is at most 25 bits.

  - For NR, maximum number of PRBs is at least 275 PRBs. It is expected that RA field size is increased.

  - However, considering the payload and benefit of frequency-selective scheduling, RBG size can be increased for more than 100 PRBs. FFS how many bits are necessary for RA field , e.g., up to around 30 bits.

- Data duration and stating timing [2]

  - For NR, flexible data scheduling timing/duration will be supported.

  - Considering one slot has up to 14 symbols, up to 4 bits could be reasonable.

  - This field should also be usable for slot aggregation.

- HARQ process number [3]

  - Considering realistic NR scenarios, up to 16 HARQ processes would be necessary.

  - On the other hand, in order to realize efficient operation, the number of HARQ processes can be reduced by configuration.

- CBG index for re-transmission [4, 5]

  - In case of CBG-based re-transmission, DCI should indicate which CBG(s) is/are re-transmitted.

- QCL indication [6]

  - This field tells the UE of quasi-co-location information.

- HARQ-ACK timing/resource [7]

  - For NR, implicit resource indication of HARQ-ACK feedback should be avoided.

  - Explicit field should always be included in the DCI.

- Indication for HARQ-ACK multiplexing (e.g., DAI) [8]

  - NR will support HARQ-ACK multiplexing for multiple data over time and frequency.

  - It is important to adjust the HARQ-ACK payload depending on the actually scheduled resources.

- Conclusion

In this contribution, we provided our initial estimates on necessary DCI fields for NR.

References

- R1-1708487, ‘Scheduling and HARQ for CA and DC,’ NTT DOCOMO, INC.

- R1-1708478, ‘Resource allocation for DL data,’ NTT DOCOMO, INC.

- R1-1708482, ‘Processing time and HARQ process number for NR,’ NTT DOCOMO, INC.

- R1-1708483, ‘CBG construction,’ NTT DOCOMO, INC.

- R1-1708483, ‘CBG based (re)transmission, preemption indication and subsequent trasnmission in NR,’ NTT DOCOMO, INC.

- R1-1708451, ‘Views on beam management details,’ NTT DOCOMO, INC.

- R1-1708475, ‘PUCCH resource allocation,’ NTT DOCOMO, INC.

- R1-1708481, ‘HARQ-ACK feedback mechanisms,’ NTT DOCOMO, INC.